3GPP TSG RAN meeting #105										       RP-242403

Melbourne, Australia, September 9-12, 2024

## Status Report to TSG

Agenda item:			9.5.1

| WI / SI Name | Work Item on expanded and improved NR positioning |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | NR_pos_enh2, NR_pos_enh2-Perf |  |  |  |
| Unique ID | 981038, 981238 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241186 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2024 | Performance part: 09/2024 -> 12/2024 | Testing part: N/A |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 99% | Testing part: N/A |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Debdeep Chatterjee |
|  | Company | Intel Corporation |
|  | Email | debdeep.chatterjee@intel.com |
|  | Name | Jianxiang Li |
|  | Company | CATT |
|  | Email | lijianxiang@catt.cn |
|  | Name | Florent Munier |
|  | Company | Ericsson |
|  | Email | florent.munier@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

RAN4-core part work 100% completed at RAN 103. The following agreements include performance part conclusions.

#### 2.4.1	Agreements

##### 2.4.1.1	Decisions during RAN4#112

##### 2.4.1.1.1	General aspects

##### 2.4.1.1.2	RF

##### 2.4.1.1.3	RRM performance

General for RRM performance requirements and test cases (R4-2412683, R4-2413983):

  - RAN4#112 agrees that the WI performance part is closed.

  - The Big CR for performance part is supposed to be agreed in R4-2413983 for the item.

  - Test cases work split is updated and agreed in R4-2412683.

The following agreements were made for RedCap positioning performance RRM requirements and test cases (R4-2414032):

  - Additional BW configuration for RedCap positioning accuracy requirement with Rx FH:

    - RAN4 not to define FH accuracy requirements for additional BW configurations.

    - RAN4 not to re-use non-FH accuracy requirements for FH cases.

  - Bandwidth configuration for accuracy requirement for RedCap positioning with Rx FH:

    - Per-hop BW is defined in RB number instead of MHz.

    - RAN4#111 agreement on per-hop BW and the total BW for RedCap positioning with Rx FH is revised and the accuracy requirements for Rx FH case are defined only for the following BW groups:

      - 15 kHz: per-hop BW >= 52 RB, total BW = 268 RB

      - 30 kHz: per-hop BW = 48 RB, total BW = 272 R

      - 60 kHz (FR1): per-hop BW = 24 RB, total BW = 132 RB

      - 60 kHz (FR2): per-hop BW >= 64 RB, total BW = 264 RB

      - 120 kHz: per-hop BW = 64 RB, total BW = 264 RB

  - Group delay calibration margin for RedCap positioning:

    - RAN4 to use the following assumptions for group delay calibration margin for RedCap

      - Non-FH: existing values for non-RedCap for the applicable BW.

      - FH: existing values for non-RedCap for per-hop BW [FFS: plus an extra margin (value TBD)].

  - PRS RMC for RedCap positioning TCs:

    - [For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.] – double check with TE vendors

    - For FH TCs, introduce a new PRS RMC with large BW and repetitions.

      - A CR revision is needed to include the new PRS RMC for 15 kHz (FR1) and 120 kHz (FR2) – Qualcomm to revise a submitted CR

The following agreements were made for PRS/SRS bandwidth aggregation performance RRM requirements and test cases (R4-2414032):

  - Group delay margin for positioning measurement by aggregating PRS resources:

    - Group delay margin values defined in Tables 10.1.23.2-5/5a and 10.1.23.2-6/6a are reused to define group delay margin values for RSTD measurements performed by aggregating PRS resources on multiple PFLs.

    - Group delay margin values defined in Tables 10.1.25.2-5 and 10.1.25.2-6 are reused to define group delay margin values for UE Rx-Tx measurements performed by aggregating resources on multiple PFLs.

  - Frequency drift margin for positioning measurement by aggregating PRS resources:

    - RAN4 to define frequency drift margin for RSTD measurement on the PRS resources from the reference and target TRPs belonging to the same PFL group or different PFL groups by reusing the values of Y from the existing specification.

  - PRS configuration for positioning TCs for PRS aggregation:

    - PRS BW per PFL based on the existing PRS RMC with largest BW is considered for the accuracy TCs for positioning measurements based on the PRS aggregation.

The following agreements were made for NR sidelink positioning performance RRM requirements and test cases (R4-2414033):

  - RF calibration margin for SL RSTD/Rx-Tx:

    - Reuse the RF calibration margins defined for Uu positioning for SL RSTD and SL Rx-Tx:

      - The RF calibration margin for SL RSTD measurements in FR1:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 48 | ≥ 24 | N/A | Z1=72 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z2=36 |

      - The RF calibration margin for SL Rx-Tx measurements in FR1:

| [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)] |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 48 | ≥ 24 | N/A | 1=80 |
| ≥ 96 | ≥ 48 | ≥ 24 | 2=56 |

  - Frequency drift margin for SL Rx-Tx measurement:

    - The frequency drift margin for SL Rx-Tx is defined as Y=32 Tc, provided that the time offset between the SL PRS transmission and reception, which are used for a single SL Rx-Tx estimate, is no greater than 160 ms.

  - General aspects for measurement accuracy requirements for SL positioning:

    - On SL-PRS bandwidth 24 PRBs with 15 kHz SCS in the accuracy requirements for SL RSTD and SL Rx-Tx measurements:

      - This case was not simulated, the performance can be worse than legacy.

      - Do not include 24 PRBs in Rel-18 accuracy requirements for SL positioning.

    - The following band groups are included in the accuracy requirements for SL positioning and in the conditions for NR SL-PRS based measurements:

      - NR_TDD_FR1_B,

      - NR_TDD_FR1_C,

      - NR_FDD_FR1_G,

      - NR_TDD_FR1_J.

  - Measurement accuracy requirements for SL PRS-RSRP(P)/Rx-Tx:

    - The numbers refer to the simulation summary (R4-2410196) where the accuracies have been derived.

  - Delay and accuracy TCs for SL PRS-RSRP(P):

    - Whether to verify the accuracy of legacy measurements in SL RSRP(P) TCs

      - verify only the accuracy of SL PRS-RSRP/RSRPP

    - For SL RSRP(P) delay TCs:

      - a new TC with both measurements, but add a clarification that the UE is tested only once for RSTD/Rx-Tx

        - For UE supporting SL RSTD + SL PRS-RSRP, it is sufficient the UE passes the TC with both measurements

        - For UE supporting SL Rx-Tx + SL PRS-RSRPP, it is sufficient the UE passes the TC with both measurements

    - For SL RSRP(P) accuracy TCs:

      - a new TC with both measurements, but add a clarification that TE verifies only the accuracy of SL PRS-RSRP/RSRPP

        - For UE supporting SL RSTD + SL PRS-RSRP, it is sufficient the UE passes the TC with both measurements

        - For UE supporting SL Rx-Tx + SL PRS-RSRPP, it is sufficient the UE passes the TC with both measurements

  - SL-PRS signal level configurations:

    - Check the RAN1 specification and the Uu TCs in relation to overlapping SL-PRS from different UEs. Further discuss the options:

      - Option 1: overlapping SL-PRS can be used in TCs.

      - Option 2: overlapping SL-PRS should not be used in SL TCs.

  - SL-PRS bandwidth configurations in TCs:

      - Update the bandwidth of SL-PRS configurations as follows:

| RB numbers containing SL PRS within channel Bandwidth Note 1 | [2448] | [4896] |
| --- | --- | --- |

      - BW for accuracy TCs: 24 and 48 RBs.

      - BW for delay TCs: 48 RBs.

  - Spec structure for SL-PRS test configuration:

    - The following specification clause numbers are updated:

      - A.3.21A	NR Sidelink Measurements for Positioning

      - A.3.21A.1	Introduction

      - A.3.21A.2	NR SL-PRS configurations

      - A.3.21A.2.1	NR SL-PRS configurations for FR1

The following agreements were made for Carrier Phase Positioning (CPP) performance RRM requirements and test cases (R4-2414033):

  - Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs:

    - For the CPP test case, both RSTD/Rx-Tx measurement and CPP reporting are configured, and TE check the success rate for CPP reporting.

    - If UE supports CPP measurement, UE shall also pass the test case for RSTD/Rx-Tx measurement.

  - Additional margins for frequency drift and RF calibration:

    - RAN4 to clarify that the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols, which can be controlled at least by the window configuration.

    - Companies can bring proposals on other scenarios in WI performance maintenance phase.

The following agreements were made for LPHAP RRM performance requirements and test cases (R4-2414034):

  - Configuration of PRS measurement reporting periodicity in TCs:

    - Configure PRS measurement reporting periodicity in LPHAP test cases. The reportingInterval is smaller than the eDRX cycle in the test cases.

    - LPHAP test cases for core requirement is considered complete after the UE reports the first set of positioning measurements based on the configured reportingInterval.

    - reportingInterval in TC: 20 sec

    - How to include the above in the test case requirements subsection: use the text from the second bullet.

#### 2.4.2	Remaining Open issues

Performance part:

  - Resolve TBD values for performance part RRM requirements and test cases for RedCap positioning, positioning with bandwidth aggregation, Sidelink positioning, Carrier-phase positioning and LPHAP

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

###### RAN4 #112

- R4-2411328	(10-5, 6) Draft CR on RSTD measurement reporting delay test cases with PRS aggregation in FR1 and FR2 in RRC_IDLE state	CATT

- R4-2411329	(11-5, 6) Draft CR on RSTD measurement accuracy test cases with PRS aggregation in FR1 and FR2 in RRC_IDLE state	CATT

- R4-2411330	Draft CR on core requirements for RedCap positioning	CATT

- R4-2411331	Draft CR on interruption requirements for SRS  BW aggregation	CATT

- R4-2411332	Draft CR on LPHAP core requirements	CATT

- R4-2411333	Draft CR on core requirements for CPP	CATT

- R4-2411334	Draft CR on SL PRS-RSRP(P) measurement delay and accuracy test cases in FR1	CATT

- R4-2411335	Discussion on Core requirements of carrier phase positioning	CATT

- R4-2411336	Discussion on Core requirements of PRS BW aggregation	CATT

- R4-2411337	Discussion on Performance requirements of Sidilink positioning	CATT

- R4-2411338	Discussion on Performance requirements of RedCap UE positioning	CATT

- R4-2411339	Discussion on Performance requirements of Carrier phase positioning	CATT

- R4-2411488	Discussion on perf requirements for SL positioning	OPPO

- R4-2411489	[TC 11-1 and 11-2] CR on TC for SL measurement accuracy in FR1	OPPO

- R4-2411490	[TC 3-31 and 3-32] CR on TC for PRS-RSRPP delay with Rx FH in RRC inactive	OPPO

- R4-2411491	[TC 10-17 and 11-17] CR on TC for PRS-RSRP delay wo Rx FH in RRC IDLE	OPPO

- R4-2411492	Discussion on perf requirements for carrier phase positioning	OPPO

- R4-2411624	(8-1,8-2,8-3,8-4, 8-7,8-8) Draft CR RSCPD test case	Xiaomi

- R4-2411787	Draft CR – Performance requirements for UE Rx-Tx measurement accuracy with PRS BW aggregation (Set 2-7)	Qualcomm Incorporated

- R4-2411788	Draft CR – Test cases for UE Rx-Tx measurement accuracy with PRS BW aggregation, Sets 6-5, 6-6, 6-7, 6-8	Qualcomm Incorporated

- R4-2411789	Draft CR – Test cases for RSTD measurement delay with eDRX > 10.24s in RRC_INACTIVE, Sets 9-9, 9-10	Qualcomm Incorporated

- R4-2411790	Draft CR – Test cases for PRS-RSRPP measurement delay with eDRX > 10.24s in RRC_INACTIVE, Sets 9-15, 9-16	Qualcomm Incorporated

- R4-2411791	Draft CR – Test cases for RSTD measurement delay and accuracy with eDRX > 10.24s in RRC_IDLE, Sets 10-4, 10-10, 11-4, 11-10	Qualcomm Incorporated

- R4-2411792	Additional simulation results for 1Rx RedCap UEs without frequency hopping	Qualcomm Incorporated

- R4-2411793	On RRM performance requirements for SL positioning	Qualcomm Incorporated

- R4-2411805	Topic summary for [112][210] NR_pos_enh2_part1	Moderator (Ericsson)

- R4-2411806	Topic summary for [112][211] NR_pos_enh2_part2	Moderator (CATT)

- R4-2411807	Topic summary for [112][212] NR_pos_enh2_part3	Moderator (Huawei)

- R4-2411982	(NR_pos_enh2-Perf) Discussion on open issues for carrier phase positioning	CMCC

- R4-2411983	(NR_pos_enh2-Perf) Discussion on open issues for sidelink positioning	CMCC

- R4-2412253	(4-21, 22, 23, 24) Draft CR on UE Rx-Tx measurement accuracy TCs for RedCap positioning	vivo

- R4-2412254	(10-1 10-2) Draft CR on RSTD measurement delay TCs for RRC_IDLE mode	vivo

- R4-2412255	(11-1 11-2) Draft CR on RSTD measurement accuracy TCs for RRC_IDLE mode	vivo

- R4-2412423	(4-17~20) Test cases for RedCap CONNECTED and INACTIVE mode RSTD measurement accuracy with frequency hopping	Intel Corporation

- R4-2412643	Discussion on RRM requirements for SL positioning and CPP	Huawei, HiSilicon

- R4-2412644	draftCR on RRM requirements for SL positioning	Huawei, HiSilicon

- R4-2412645	draftCR on RRM requirements for CPP	Huawei, HiSilicon

- R4-2412646	draftCR on RRM requirements for LPHAP	Huawei, HiSilicon

- R4-2412647	Discussion on RedCap positioning and PRS/SRS BWA	Huawei, HiSilicon

- R4-2412648	draftCR on RRM requirements for RedCap positioning	Huawei, HiSilicon

- R4-2412649	draftCR on requirements for PRS BWA	Huawei, HiSilicon

- R4-2412650	draftCR on TCs for RRC_IDLE	Huawei, HiSilicon

- R4-2412651	On performance requirements for SL positioning	Huawei, HiSilicon

- R4-2412652	draftCR on performance requirements for SL positioning	Huawei, HiSilicon

- R4-2412653	On performance requirements for LPHAP	Huawei, HiSilicon

- R4-2412654	draftCR on performance requirements for LPHAP	Huawei, HiSilicon

- R4-2412655	On performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2412656	draftCR on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2412657	On performance requirements for PRS BWA	Huawei, HiSilicon

- R4-2412658	draftCR on performance requirements for PRS BWA	Huawei, HiSilicon

- R4-2412659	On performance requirements for CPP	Huawei, HiSilicon

- R4-2412679	draftCR 38.133 Core requirement for CPP	Ericsson

- R4-2412680	draftCR 38.133 Core requirement for LPHAP	Ericsson

- R4-2412681	On core requirement for bandwidth aggregation for positioning measurements	Ericsson

- R4-2412682	draftCR 38.133 Core requirement for RedCap positioning	Ericsson

- R4-2412683	Updated work split for Rel. 18 positioning	Ericsson

- R4-2412684	draftCR 38.133 Corrections to accuracy requirements for Rel18. positioning	Ericsson, Nokia

- R4-2412685	On performance requirement for LPHAP	Ericsson

- R4-2412686	draftCR 38.133 Phase II LPHAP test cases	Ericsson

- R4-2412687	On performance requirement for RedCap positioning	Ericsson

- R4-2412688	Summary of simulation results for RedCap positioning	Ericsson

- R4-2412689	draftCR 38.133 Phase II RedCap positioning test cases	Ericsson

- R4-2412690	On performance requirement for PRS/SRS aggregation	Ericsson

- R4-2412691	draftCR 38.133 Phase II test cases for bandwidth aggregation for positioning measurements	Ericsson

- R4-2412692	Summary of simulation results for PRS aggregation	Ericsson

- R4-2412693	On performance requirement for CPP	Ericsson

- R4-2412694	draftCR 38.133 Phase II test cases for CPP	Ericsson

- R4-2413044	test case for PRS-RSRPP measurement accuracy TC in RRC_CONNECTED state in FR1 without Rx FH	ZTECorporation,Sanechips

- R4-2413045	test case for PRS-RSRPP measurement accuracy TC in RRC_INACTIVE state in FR1 without Rx FH	ZTECorporation,Sanechips

- R4-2413046	test case for PRS-RSRPP measurement accuracy TC in RRC_INACTIVE state in FR2 with Rx FH	ZTECorporation,Sanechips

- R4-2413292	(NR_pos_enh2-Perf) (3-11, 3-12) PRS-RSRP measurement delay test case for RedCap positioning without Rx FH in RRC INACTIVE state in FR1 and FR2	Nokia

- R4-2413293	RRM Performance Requirements for LPHAP	Nokia

- R4-2413312	Sets (7-7) and (7-8) Measurement delay TCs for RSCP with UE Rx-Tx in RRC_INACTIVE for FR1 and FR2	Nokia

- R4-2413313	Simulation results for DL RSCPD and DL RSCP	Nokia

- R4-2413329	draftCR (3-2)(3-4) TCs for RedCap positioning without FH on RSTD measurement delay in CONNECTED and INACTIVE states	MediaTek inc.

- R4-2413330	draftCR (4-2)(4-4) TCs for RedCap positioning without FH on RSTD measurement accuracy in CONNECTED and INACTIVE states	MediaTek inc.

- R4-2413331	draftCR (10-7)(10-8) TCs for RedCap positioning without FH on RSTD measurement delay in IDLE state without eDRX	MediaTek inc.

- R4-2413332	draftCR (11-7)(11-8) TCs for RedCap positioning without FH on RSTD measurement accuracy in IDLE state without eDRX	MediaTek inc.

- R4-2413386	On remaining core issues for SL positioning and CPP	Ericsson

- R4-2413387	Draft CR 38133 on remaining core issues for SL positioning	Ericsson

- R4-2413388	On remaining performance issues for SL positioning	Ericsson

- R4-2413389	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2413390	Draft Big CR to 38133 for RRM performance part for expanded and improved NR positioning	Ericsson

- R4-2413461	On the definition of Carrier Phase and the Impact of CFO on Averaging Carrier Phase Measurements	Lenovo

- R4-2413462	RRM core maintenance for NR Carrier Phase Positioning	Nokia

- R4-2413873	Ad-hoc minutes #1 for NR_pos_enh2 WI	Ericsson

- R4-2413874	Ad-hoc minutes #2 for NR_pos_enh2 WI	Ericsson

- R4-2413875	Ad-hoc minutes #3 for NR_pos_enh2 WI	Intel

- R4-2413976	Draft CR on interruption requirements for SRS  BW aggregation	CATT

- R4-2413977	draftCR on RRM requirements for RedCap positioning	Huawei, HiSilicon

- R4-2413978	draftCR 38.133 Core requirement for RedCap positioning	Ericsson

- R4-2413979	Draft CR on core requirements for CPP	CATT

- R4-2413980	draftCR 38.133 Core requirement for CPP	Ericsson

- R4-2413981	Draft CR 38133 on remaining core issues for SL positioning	Ericsson

- R4-2413982	draftCR 38.133 Corrections to accuracy requirements for Rel18. positioning	Ericsson, Nokia

- R4-2413984	draftCR on TCs for RRC_IDLE	Huawei, HiSilicon

- R4-2413985	Draft CR on SL PRS-RSRP(P) measurement delay and accuracy test cases in FR1	CATT

- R4-2413986	[TC 11-1 and 11-2] CR on TC for SL measurement accuracy in FR1	OPPO

- R4-2413987	Draft CR 38133 on remaining performance issues for SL positioning	Ericsson

- R4-2413988	Draft CR – Test cases for RSTD measurement delay with eDRX > 10.24s in RRC_INACTIVE, Sets 9-9, 9-10	Qualcomm Incorporated

- R4-2413989	Draft CR – Test cases for PRS-RSRPP measurement delay with eDRX > 10.24s in RRC_INACTIVE, Sets 9-15, 9-16	Qualcomm Incorporated

- R4-2413990	Draft CR – Test cases for RSTD measurement delay and accuracy with eDRX > 10.24s in RRC_IDLE, Sets 10-4, 10-10, 11-4, 11-10	Qualcomm Incorporated

- R4-2413991	(10-1 10-2) Draft CR on RSTD measurement delay TCs for RRC_IDLE mode	vivo

- R4-2413992	(11-1 11-2) Draft CR on RSTD measurement accuracy TCs for RRC_IDLE mode	vivo

- R4-2413993	draftCR on performance requirements for LPHAP	Huawei, HiSilicon

- R4-2413994	draftCR 38.133 Phase II LPHAP test cases	Ericsson

- R4-2413995	[TC 3-31 and 3-32] CR on TC for PRS-RSRPP delay with Rx FH in RRC inactive	OPPO

- R4-2413996	[TC 10-17 and 11-17] CR on TC for PRS-RSRP delay wo Rx FH in RRC IDLE	OPPO

- R4-2413997	(4-21, 22, 23, 24) Draft CR on UE Rx-Tx measurement accuracy TCs for RedCap positioning	vivo

- R4-2413998	(4-17~20) Test cases for RedCap CONNECTED and INACTIVE mode RSTD measurement accuracy with frequency hopping	Intel Corporation

- R4-2413999	draftCR on performance requirements for RedCap positioning	Huawei, HiSilicon

- R4-2414000	draftCR 38.133 Phase II RedCap positioning test cases	Ericsson

- R4-2414001	test case for PRS-RSRPP measurement accuracy TC in RRC_CONNECTED state in FR1 without Rx FH	ZTECorporation,Sanechips

- R4-2414002	test case for PRS-RSRPP measurement accuracy TC in RRC_INACTIVE state in FR1 without Rx FH	ZTECorporation,Sanechips

- R4-2414003	test case for PRS-RSRPP measurement accuracy TC in RRC_INACTIVE state in FR2 with Rx FH	ZTECorporation,Sanechips

- R4-2414004	(NR_pos_enh2-Perf) (3-11, 3-12) PRS-RSRP measurement delay test case for RedCap positioning without Rx FH in RRC INACTIVE state in FR1 and FR2	Nokia

- R4-2414005	draftCR (3-2)(3-4) TCs for RedCap positioning without FH on RSTD measurement delay in CONNECTED and INACTIVE states	MediaTek inc.

- R4-2414006	draftCR (4-2)(4-4) TCs for RedCap positioning without FH on RSTD measurement accuracy in CONNECTED and INACTIVE states	MediaTek inc.

- R4-2414007	draftCR (10-7)(10-8) TCs for RedCap positioning without FH on RSTD measurement delay in IDLE state without eDRX	MediaTek inc.

- R4-2414008	draftCR (11-7)(11-8) TCs for RedCap positioning without FH on RSTD measurement accuracy in IDLE state without eDRX	MediaTek inc.

- R4-2414011	(10-5, 6) Draft CR on RSTD measurement reporting delay test cases with PRS aggregation in FR1 and FR2 in RRC_IDLE state	CATT

- R4-2414012	(11-5, 6) Draft CR on RSTD measurement accuracy test cases with PRS aggregation in FR1 and FR2 in RRC_IDLE state	CATT

- R4-2414013	Draft CR – Performance requirements for UE Rx-Tx measurement accuracy with PRS BW aggregation (Set 2-7)	Qualcomm Incorporated

- R4-2414014	Draft CR – Test cases for UE Rx-Tx measurement accuracy with PRS BW aggregation, Sets 6-5, 6-6, 6-7, 6-8	Qualcomm Incorporated

- R4-2414015	draftCR on performance requirements for PRS BWA	Huawei, HiSilicon

- R4-2414016	draftCR 38.133 Phase II test cases for bandwidth aggregation for positioning measurements	Ericsson

- R4-2414017	(8-1,8-2,8-3,8-4, 8-7,8-8) Draft CR RSCPD test case	Xiaomi

- R4-2414018	draftCR 38.133 Phase II test cases for CPP	Ericsson

- R4-2414019	Sets (7-7) and (7-8) Measurement delay TCs for RSCP with UE Rx-Tx in RRC_INACTIVE for FR1 and FR2	Nokia

- R4-2414088	draftCR on RRM requirements for CPP	Huawei, HiSilicon

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template