3GPP TSG-RAN WG3 Meeting #125	R3-244722

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for TR 38.799) Access control for NR femto

Document for:	Discussion

1	Introduction

This paper is to provide TP for Access control of NR Femto following the agreements:

With the existing CAG mechanism, the open, hybrid and closed access modes can be supported as follows:

-	To support the open access mode: The NR Femto activates a PLMN cell, which can be accessed by legacy UE without access control.

-	To support the hybrid access mode: The NR Femto cell can be shared by both PLMN and CAG, through broadcast both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.  Then, this cell is accessible as a CAG cell by UEs which have the allowed CAG list including this cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value to be set as “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the NR Femto cell.

The above is captured in R3-244722 (Huawei)

.

Annex——TP for TR 38.799 v1.0.0

Start of Change

5.3	Access control

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows:

- To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

- To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

- To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcastsing the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

End of Change