3GPP TSG-RAN WG4 Meeting #112	R4-2411037

Maastricht, Netherlands, August 19 – August 23, 2024

Source:	Nokia

Title:	NTN simulation results for DMRS bundling and PUCCH

Agenda item:	5.23.8.1

Document for:	Information

Introduction

In this contribution we present the results of our simulation campaign based on the parameters agreed during RAN4 discussions.

Discussion

PUSCH Results with DMRS bundling

Tables 2-5 show the results for PUSCH with DMRS bundling based on the simulation parameters discussed in previous RAN4 meeting and displayed in Table 1.

Table 1: Test parameters for testing PUSCH with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 1) | 0,0,0,0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4 for 15kHz SCS n8 for 30kHz SCS |
| pusch-TimeDomainWindowLength |  | 4 for 15kHz SCS 8 for 30kHz SCS |
| Frequency domain resource assignment | RB assignment | 6 RBs in the middle of the channel bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Table 2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -3.18 | -0.7 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -6.57 | -4.1 |

Table 3: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -5.52 | -3.0 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -8.54 | -6.0 |

Table 4: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -3.29 | -0.8 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-7 | pos1 | -6.63 | -4.1 |

Table 5: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -5.48 | -3.0 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70 % | G-FR1-NTN-A3-8 | pos1 | -8.78 | -6.3 |

PUCCH format 4 Results

Table 7 presents the results for PUCCH format 4 based on the simulations parameters showed in Table 6.

Table 6: PUCCH test parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | SAN type 1-O | SAN type 2-O |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Number of PRBS | 1 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index - (Number of PRBS - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of symbols | 14 |  |
| The number of UCI information bits | 22 |  |
| First symbol | 0 |  |
| Length of the orthogonal cover code | n2 |  |
| Index of the orthogonal cover code | n0 |  |

Table 7: PUCCH format 4 with 120 kHz SCS 50MHz channel bandwidth

| Number of TX antennas | Number of demodulation branches | Cyclis Prefix | Propagation conditions and correlation matrix (Annex G) | Additioan DM-RS configuration | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 4.0 | 6.5 |
|  |  |  |  | Additional DM-RS | 4.1 | 6.6 |
|  | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -2.9 | -0.4 |
|  |  |  |  | Additional DM-RS | -2.8 | -0.3 |

Conclusion

In this contribution we have presented Nokia's results from our simulation campaign for NR NTN UE demodulation for both PUSCH and PUCCH format 4.

No observations or proposals were made.