# 3GPP TSG-RAN WG4 Meeting #112	R4-2411294

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.17.4

Source: 	OPPO

Title: 	Testability and interoperability issues for CSI compression and CSI prediction

Document for:	Approval

- Introduction

In 3GPP RAN #102[1], for CSI enhancement, it is agreed to further study the testability and interoperability issues in R19. In this contribution, we will focus on the testing options for two-sided models, as well as the performance requirement for both CSI compression and CSI prediction cases.

- Discussion

  - Testing options for two-sided models

For a two-sided AI/ML model, the information transmitted through air-interface (e.g. PMI) is generated by AI/ML models (e.g. CSI encoder), rather than codebooks predefined by protocol. Regarding the testability of two-sided model, it is necessary to consider bring in a test model to cooperate with the model under test.

In the previous RAN4 meetings, 4 options on test decoder have been proposed and discussed, i.e.,

- Option 1: test decoder is provided by the vendor of the encoder

- Option 2: test decoder is provided by the vendor of the decoder

- Option 3: The test decoder(s) are fully specified and captured in RAN4 spec.

- Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

In RAN4 #110 meeting, RAN4 agree to further discuss only options 3 and 4, i.e., fully or partially specify test decoder(s).

From our understanding, to define a test decoder in specification, following aspects should be taken into account.

-  Principles to define test decoder(s)

- To meet the minimum performance requirement

The test model is used to meet the minimum performance requirement in RAN4 tests, not directly to be used for commercial utilization and deployment.

- To be a simple design

As a test model to be specified in RAN4 spec, we may have a lot of proposals from different companies. How to do the merge and compromise among different designs and views from different companies would be a big challenge and a tough work.

From the beginning, we can try to design the test model(s) as simple as possible, to facilitate calibration and promote consistency among different companies.

Proposal 1: Principles to define test decoder(s)

- to meet the minimum performance requirement in RAN4 tests

- to be a simple design

-  Steps to determine test model(s):

- Setp1 (high priority): Determine the test condition

Before design a test model, we need to determine the test conditions first, e.g.,

- test conditions for Rank1 CSI feedback and for Rank2 CSI feedback may be different, and may need 			different test model/data.

- test conditions on Tx antenna settings and CSI feedback overheads may also link to different tests and 		different needs for the test model/data.

In RAN4#110bis meeting, companies have agreed that “in order to consider model performance with more concrete details, companies are encouraged to bring parameters/values proposals, considering RAN4 existing test configuration or RAN1 baseline scenario captured in TR 38.843 in Table 6.2.1-2 for example”.

In RAN4#111 meeting, simulation parameters for verifying the feasibility of aligning model among companies are agreed as below. From our understanding, the agreed table(including configurations and scenarios) can be considered and utilized as test conditions as well.

Table 1  Parameters and candidate values for CSI test condition

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

- Step2:  Determine the test data

Including,

Step 2-1 (high priority): 	consider how to set up the test data, how to determine the test data generation method. Link level test data should be utilized. Parameters and candidate values are show in below.

Table 2  Parameters and candidate values for CSI test data

| Parameter | Value |
| --- | --- |
| Channel model | CDL-C as baseline |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. |

Note: test data should be obtained from link level channel model, training data could be obtained from system level channel model, e.g. UMa channel

Step 2-2: consider how to collect datasets from different companies

Step 2-3: consider whether/how to merge datasets from different companies to form a test model data set

To determine a test data,

- Step3:  Determine the test model structure

Including,

Step 3-1(high priority):  consider how to set up the model structure, e.g. as shown in table 3

Step 3-2: 	consider how to merge and compromise different inputs on the proposed test model structures from different companies, to form a test model structure

To determine a test model structure, as discussed in RAN4#111 meeting, following parameters and candidate values could be considered:

Table 3  Parameters and candidate values for CSI test model structure

| Model architecture parameters | Model type | CNN as baseline, e.g., Resnet based structure |
| --- | --- | --- |
|  | Model architecture | [TBD], The agreed reference model structure (CNN) can be considered as starting point |
|  | Quantization method for the encoder output | Scalar quantization as baseline, e.g. 2bit |
|  | Encoder-decoder interface | - CSI feedback overhead A: 64 bits (small payload) - CSI feedback overhead B: 112 bits (medium payload) |
|  | Fixed point representation | floating point |
|  | Format of input to encoder/output of decoder | Raw channel or CSI eigenvector |

- Step4:  Determine the test model parameter

Step 4-1 (high priority): 	determine test model parameters based on the test dataset and test model structure

Step 4-2: consider how to merge different sets of test model parameters to obtain a set of specified test model parameters

Based on our understanding, it is not necessary to spend a lot of time on discussing and aligning model hyperparameters and training methods.

- a lot of factors may lead to different parameter values eventually, e.g. hardware related, software related, random seeds and other factors

- difficult to achieve aligned model parameters among different companies, if just by the alignment of model hyperparameters and some training designs

Instead, companies can train a test model based on the test dataset and test model structure, and then provide the test model parameters while including information about the model hyperparameters and training methods involved in the training process. In the end, we can further evaluate these results and discuss how to merge different test model parameters to obtain the specified test model parameters.

Proposal 2: Steps to determine test model(s):

- Setp1: Determine the test condition(high priority)

- Step2: Determine the test data

Step 2-1 (high priority): consider how to set up the test data, how to determine the test date 				generation method

Step 2-2: consider how to collect datasets from different companies

Step 2-3:	consider whether/how to merge datasets from different companies to form a test model 			data set

- Step3: Determine the test model structure

Step 3-1(high priority): consider how to set up the model structure

Step 3-3: consider how to merge and compromise different inputs on the proposed test model 			structures from different companies, to form a test model structure

- Step4: Determine the test model parameter

Step 4-1 (high priority): determine the test model parameters based on the test dataset and test model 		structure

Step 4-2: consider how to merge different sets of test model parameters to obtain a set of specified 			test model parameters

Proposal 3: Not necessary to spend a lot of time on discussing and aligning model hyperparameters and training methods.

Proposal 4: Regarding the test condition, e.g. test configurations or baseline scenarios, following parameters and candidate values should be considered and determined:

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Proposal 5: To determine a test data, link level test data should be utilized. Following parameters and candidate values should be aligned:

| Parameter | Value |
| --- | --- |
| Channel model | CDL-C as baseline |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. |

Note: test data should be obtained from link level channel model, training data could be obtained from system level channel model, e.g. UMa channel

Proposal 6: To determine a test model structure, following parameters and candidate values should be considered:

| Model architecture parameters | Model type | CNN as baseline, e.g., Resnet based structure |
| --- | --- | --- |
|  | Model architecture | [TBD], The agreed reference model structure (CNN) can be considered as starting point |
|  | Quantization method for the encoder output | Scalar quantization as baseline, e.g. 2bit |
|  | Encoder-decoder interface | - CSI feedback overhead A: 64 bits (small payload) - CSI feedback overhead B: 112 bits (medium payload) |
|  | Fixed point representation | floating point |
|  | Format of input to encoder/output of decoder | Raw channel or CSI eigenvector |

In RAN4 #110bis and #111 meeting, some flowcharts had been proposed and discussed. In general, we agree to follow these flowcharts to conduct RAN4 test model related works. But, for the first 4 steps in the flowchart, we suggest some updates according to our proposal mentioned above.

As shown in Figure 1, (1) we add a new step1 to address the test condition issues, e.g., test configuration or baseline scenarios as discussed in the last meeting, and also listed in Table1. (2) we do not think it is not necessary to spend a lot of time on discussing and aligning model hyperparameters and training methods. We suggest to address the test data and test model structure with high priority. So, we update and rearrange the order of steps 1 to 4, and add some descriptions.

The updated step1-step4 in the flowchart is shown as below.

Figure 1  Update steps 1 to 4 in the flowchart that discussed in RAN4#110bis meeting

Proposal 7: Update the step 1-4 in the flowchart that discussed in RAN4#110bis meeting as below:

  - Performance requirement

For both CSI compression and CSI prediction, model/functionality input and output related tests should be supported.

- Regarding the input related test, performance requirement on CSI model/functionality input (e.g. CSI-RS measurement accuracy) are needed.

- Regarding the output related test, as agreed in the last meeting, throughput should be utilized to evaluate the model inference performance. Existed RAN4 test examples for “reporting of PMI” can serve as a reference, e.g. as captured in 38.101-4, “The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively.” Requirement of γ and test settings can be reused or updated(if test conditions updated).

- In terms of the relationship to legacy requirements, according to the previous conclusion, what we define is the minimum performance requirement, which is not to verify how much performance gain an AI/ML function/model can bring. The assessment of performance gain should be considered when RAN1 introduces and evaluates CSI features, rather than when RAN4 defines test cases. Furthermore, under current test conditions, e.g., by TDL channel, even if we define a higher performance testing requirement, it still has little significance for the practical application of AI/ML solutions. Therefore, we can just reuse the legacy PMI requirement (compare to random precoding) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

- Regarding the static/non-static scenarios/conditions and propagation conditions for testing, as the first RAN4 AI/ML test version, we should first consider testing cases in static scenarios and configurations. After having feasible testing cases for static configurations, then we can further consider whether to introduce non-static testing scenarios and configurations. Regarding how to define a static scenario/configuration (e.g., by defining a related testing dataset based on channel models in TR 38.901), data sets based on TR 38.901 can be considered as the starting point.

- In addition, it should be noted that if the test is limited to the legacy approach (e.g., TDL channel), it is challenging to assert that the performance test outcomes can reflect the dependability and effectiveness of the tested model. This is because for a data-driven AI/ML algorithm, the TDL channel modeling method is too simple to mimic. A CSI compression case is shown in table 4(transformer as model backbone), even if a model achieves good test results on the TDL channel, it is still hard to say that it can work on UMa and CDL channels, let alone in complex field channels. Therefore, we suggest that besides the TDL channel based tests(could be a baseline test), AI based CSI tests can be conducted under CDL channel [or other more practical channel conditions] to check a relatively generalized performance.

Table 4 CSI compression under different channel assumptions

| CSI feedback bits | Training data set | Test dataset |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 67 bits | TDL-A30 | 0.942 | 0.302 | 0.447 | 0.064 |
|  | CDL-A30 | 0.779 | 0.986 | 0.780 | 0.237 |
|  | CDL-C30 | 0.736 | 0.806 | 0.783 | 0.165 |
|  | UMa | 0.807 | 0.716 | 0.657 | 0.785 |
| 120 bits | TDL-A30 | 0.967 | 0.384 | 0.526 | 0.059 |
|  | CDL-A30 | 0.764 | 0.992 | 0.796 | 0.238 |
|  | CDL-C30 | 0.754 | 0.836 | 0.806 | 0.169 |
|  | UMa | 0.811 | 0.775 | 0.679 | 0.841 |
| 285 bits | TDL-A30 | 0.987 | 0.483 | 0.602 | 0.071 |
|  | CDL-A30 | 0.770 | 0.996 | 0.811 | 0.254 |
|  | CDL-C30 | 0.812 | 0.863 | 0.857 | 0.174 |
|  | UMa | 0.818 | 0.790 | 0.701 | 0.904 |

Proposal 8: For both CSI compression and CSI prediction, model/functionality input (CSI-RS measurement accuracy) and output (associated throughput) related tests should be supported.

Proposal 9: Existed RAN4 test examples for “reporting of PMI” can be reused or serve as a reference. Requirement of γ and test settings can be reused or updated.

Proposal 10: Reuse the legacy PMI requirement (compare to random precoding) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

Proposal 11: In R19, static test scenarios and configurations should be considered first. After having feasible testing cases for static configurations, then further consider whether to introduce non-static testing scenarios and configurations.

Proposal 12: Besides the TDL channel based tests, CSI-related tests can be conducted under CDL channel [or other more practical channel conditions] to check a relatively generalized performance.

  - AI/ML Model for Option 3 feasibility study

CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion are shown as below:

CNN based CSI encoder 								CNN based CSI decoder

Figure 2  CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion

Observation 1: FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Observation 2: Based on above CNN based CSI encoder and CNN based CSI decoder, performance evaluation results are shown as below(train by UMa channel data as agreed in the last meeting, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

In addition, based on our preliminary research, a complex encoder design is not necessary for a CSI feedback design. Especially in RAN4, if the test goal is to meet the minimum performance requirements, a simpler reference model design facilitates easier alignment of testing and evaluation results among different companies.

A simple MLP CSI encoder design and paired CNN based CSI decoder(proposed after RAN4 #111 email discussion) are shown as below:

MLP based CSI encoder 								CNN based CSI decoder

Figure 3  MLP based CSI encoder and CNN based CSI decoder

FLOPs, Number of parameters, model size for above MLP based CSI encoder and CSI decoder are evaluated as below.

In comparison to the CNN based CSI encoder, there's a substantial decrease in FLOPs in the MLP based CSI encoder, dropping from 94M to a mere 0.22M. This simplification lowers the challenges of aligning and implementing standardized CSI encoders across different companies, without compromising the CSI compression feedback performance.

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 0.22M | 1481.6M | 0.11M | 1.80M | 455KB | 7.03MB |
| 112 bits | 0.23M | 1481.7M | 0.11M | 1.82M | 467KB | 7.11MB |

Based on above MLP based CSI encoder and CSI decoder, some evaluation results are shown as below(train by UMa channel data as agreed in the last meeting, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.724 | 0.674 | 0.554 | 0.696 |
| 112 bits | UMa | 0.765 | 0.744 | 0.618 | 0.742 |

Observation 3: In comparison to the CNN-based CSI encoder, there's a substantial decrease in FLOPs in the MLP-based CSI encoder, dropping from 94M to a mere 0.22M

Proposal 13: For the reference encoder, a simplified MLP-based CSI encoder could lower the challenges of aligning and implementing standardized CSI encoders across different companies, without compromising the CSI compression feedback performance.

MLP based CSI encoder 								CNN based CSI decoder

- Conclusions

In this contribution, we have discussed the testability and interoperability issue on AI/ML based CSI enhancement and got following proposals:

Proposal 1: Principles to define test decoder(s)

- to meet the minimum performance requirement in RAN4 tests

- to be a simple design

Proposal 2: Steps to determine test model(s):

- Setp1: Determine the test condition(high priority)

- Step2: Determine the test data

Step 2-1 (high priority): consider how to set up the test data, how to determine the test date 				generation method

Step 2-2: consider how to collect datasets from different companies

Step 2-3:	consider whether/how to merge datasets from different companies to form a test model 			data set

- Step3: Determine the test model structure

Step 3-1(high priority): consider how to set up the model structure

Step 3-3: consider how to merge and compromise different inputs on the proposed test model 			structures from different companies, to form a test model structure

- Step4: Determine the test model parameter

Step 4-1 (high priority): determine the test model parameters based on the test dataset and test model 		structure

Step 4-2: consider how to merge different sets of test model parameters to obtain a set of specified 			test model parameters

Proposal 3: Not necessary to spend a lot of time on discussing and aligning model hyperparameters and training methods.

Proposal 4: Regarding the test condition, e.g. test configurations or baseline scenarios, following parameters and candidate values should be considered and determined:

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Proposal 5: To determine a test data, link level test data should be utilized. Following parameters and candidate values should be aligned:

| Parameter | Value |
| --- | --- |
| Channel model | CDL-C as baseline |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. |

Note: test data should be obtained from link level channel model, training data could be obtained from system level channel model, e.g. UMa channel

Proposal 6: To determine a test model structure, following parameters and candidate values should be considered:

| Model architecture parameters | Model type | CNN as baseline, e.g., Resnet based structure |
| --- | --- | --- |
|  | Model architecture | [TBD], The agreed reference model structure (CNN) can be considered as starting point |
|  | Quantization method for the encoder output | Scalar quantization as baseline, e.g. 2bit |
|  | Encoder-decoder interface | - CSI feedback overhead A: 64 bits (small payload) - CSI feedback overhead B: 112 bits (medium payload) |
|  | Fixed point representation | floating point |
|  | Format of input to encoder/output of decoder | Raw channel or CSI eigenvector |

Proposal 7: Update the step 1-4 in the flowchart that discussed in RAN4#110bis meeting as below:

Proposal 8: For both CSI compression and CSI prediction, model/functionality input (CSI-RS measurement accuracy) and output (associated throughput) related tests should be supported.

Proposal 9: Existed RAN4 test examples for “reporting of PMI” can be reused or serve as a reference. Requirement of γ and test settings can be reused or updated.

Proposal 10: Reuse the legacy PMI requirement (compare to random precoding) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

Proposal 11: In R19, static test scenarios and configurations should be considered first. After having feasible testing cases for static configurations, then further consider whether to introduce non-static testing scenarios and configurations.

Proposal 12: Besides the TDL channel based tests, CSI-related tests can be conducted under CDL channel [or other more practical channel conditions] to check a relatively generalized performance.

Observation 1: FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Observation 2: Based on CNN based CSI encoder and CNN based CSI decoder, performance evaluation results are shown as below(train by UMa channel data as agreed in the last meeting, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Observation 3: In comparison to the CNN-based CSI encoder, there's a substantial decrease in FLOPs in the MLP-based CSI encoder, dropping from 94M to a mere 0.22M.

Proposal 13: For the reference CSI encoder, a simplified MLP-based CSI encoder could lower the challenges of aligning and implementing standardized CSI encoders across different companies, without compromising the CSI compression feedback performance.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface