3GPP TSG-RAN WG4 Meeting # 112	R4-2411311

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung, TELUS, Bell mobility

Title: 	Views on UE RF requirements for fragmented carriers

Agenda Item:	8.6.3

Document for:	Discussion

- Introduction

In RAN#103, a SID was approved [1] with following objectives on the study of how to process the fragmented carriers with one Rx Chain.

We present our initial consideration in this paper.

- 2. Discussion

2.1 UE RF requirements

For “fully shared architecture” depicted in Figure 1, CC1 and CC2 share the same components from antenna to ADC. From our understanding, the 2 CCs cannot be picked up by band filter/duplexer (before external LNA), analog filter (before ADC) nor digital filter, because:

- The filter/duplexer is on a per band basis;

- The analog filter is a low-pass filter with adjustable cut-off frequency according to the UE CBW

- The adjustable multi-pass analog filter as shown below is not the current design and is challenging for implementation

- The digital filter is also a low-pass filter but it is easier to program.

Overall, it is anticipated that the performance for “fully shared architecture” would be degraded compared with the other two architectures.

CC1

CC2

× Such kind of analog filter is challenging

Observation 1: It is anticipated that the performance for “fully shared architecture” would be degraded compared with the existing requirements.

We further discuss which requirements may need to be re-visited for re-evaluation in the following sections.

- 2.4.1 ΔRIBNC

ΔRIBNC is explicitly included in the SID as one of the potential Rx requirements may require adjustment. This requirement is a per-band/CA requirement with specific test configuration (i.e., SCS/aggregated channel bandwidth/Wgap/RB allocation) which accounts for additional factors such as LO phase noise and other transmitter noise terms that cannot be mitigated by reducing the size or adjusting the location of the PCC uplink allocation, so first we think the test configuration defined in Table 7.3A.2.2-1(PC3, single CC UL) and Table 7.3A.2.2-2(PC3, intra-band CA UL) of TS 38.101-1 could be reused for evaluation. However, it is observed that the requirements are defined for some CA in “largest Wgap” case which represents the smallest Tx-Rx separation equal to the size of duplex gap such as n26(2A), while for some bands are in both “largest Wgap” case and “midpoint Wgap” case where the Tx-Rx separation can be much larger or sufficiently large to afford no relaxation such as n3(2A). The guidance to determine the Wgap values/RB allocation and how to calculateΔRIBNC can be found in [5][6].

Proposal 1: For selected example combos, the configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be reused for performance evaluation, forΔRIBNC.

Take n26(2A)ΔRIBNC as example, the requirements for which are defined in worst case, in which the Pcell UL is at the right edge of UL spectrum while the victim DL Scell is at the left edge of DL spectrum, the distance in between is the duplex gap. In our view the interference attenuation from Pcell UL to Scell DL is mainly provided by the duplexer gap. Therefore, the REFSENS of “fully shared architecture” is not expected to be degraded a lot compared to “fully separated architecture” or “partially shared architecture”. But we are also happy to see valid evaluation/simulation of this requirement, and if the performance/requirement is quite similar, it would be not preferred to define a new set of requirements.

Figure 4: Test configuration for n26(2A) forΔRIBNC

Observation 2: ForΔRIBNC, the interference attenuation from Pcell UL to Scell DL is mainly provided by duplexer gap, thus it is expected that the performance of “fully shared architecture” would be similar to the existing requirements.

- 2.4.2 ACS/IBB/NBB

Although narrow-band blocking is not explicitly mentioned in the SID, we think it also requires some evaluation, as all these three requirements include two parts, i.e., in-gap requirements and out-of-gap requirements.

First, similarly withΔRIBNC, the UL configuration should be in accordance with Table 7.3A.2.2-1 of TS 38.101-1 for evaluation purpose.

Proposal 2: For the evaluation of ACS/In-band blocking/Narrow-band blocking, the UL configuration are in accordance with Table 7.3A.2.2-1 of TS 38.101-1.

Further, as depicted in Figure 1, the “fully shared architecture” is not able to well filter out the interference between the 2 wanted CCs of operator A. Therefore in case there is another CC from operator B as interference, it cannot be cleared up with the “fully shared architecture” which in turn would impact all the in-gap relevant requirements includes in-gap ACS, in-gap IBB and in-gap NBB. Further evaluation would be needed on the amount of the degradation taken the PSD difference between operator A and operator B into account.

Figure 5. The interferer of ACS and in-ban blocking

Proposal 3: In-gap ACS, in-gap IBB and in-gap NBB requirements for “fully shared architecture” need to be re-evaluated because of the poor interference rejection capability of analog filter (before ADC) and digital filter.

For out-of-gap ACS/IBB/NBB, current spec is per CC requirements apply with all DL CC being active, while for “fully shared architecture” which is not able to filter out CC1 and CC2 may make the characteristics more like an intra-band contiguous CA. Moreover, the performance of analog filter may start to deteriorate as the bandwidth increases, so not sure if the out-of-gap relevant requirements may also need to be relaxed, further study could be considered.

Proposal 4：FFS whether out-of-gap ACS/IBB/NBB requirements need to be relaxed.

- 2.4.3 Wide band intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal. The intermodulation product of the two interfering signals can overlap with the wanted signal. Considering the fact that intermodulation products are mainly generated by non-linear components and overlaps with the wanted signal, it is anticipated the performance would not be degraded with “fully shared architecture”.

Observation 3：It is anticipated the performance would not be degraded with “fully shared architecture”, for wide band intermodulation.

- 2.4.4 OOB blocking/Spurious response

Proposal 5: Out-of-band blocking and spurious response do not need requirement adjustment as they are tested with the interferer falling more than 15 MHz above or below the band and the attenuation is mainly provided by per band duplexer/filter.

2.2 Signalling design

Proposal 6: Put off the signalling related discussions until the UE performance aspects are clear.

- 3. Conclusion

Observation 1: It is anticipated that the performance for “fully shared architecture” would be degraded compared with the existing requirements.

Proposal 1: For selected example combos, the configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be reused for performance evaluation, forΔRIBNC.

Observation 2: ForΔRIBNC, the interference attenuation from Pcell UL to Scell DL is mainly provided by duplexer gap, thus it is expected that the performance of “fully shared architecture” would be similar to the existing requirements.

Proposal 2: For the evaluation of ACS/In-band blocking/Narrow-band blocking, the UL configuration are in accordance with Table 7.3A.2.2-1 of TS 38.101-1.

Proposal 3: In-gap ACS, in-gap IBB and in-gap NBB requirements for “fully shared architecture” need to be re-evaluated because of the poor interference rejection capability of analog filter (before ADC) and digital filter.

Proposal 4：FFS whether out-of-gap ACS/IBB/NBB requirements need to be relaxed.

Observation 3：It is anticipated the performance would not be degraded with “fully shared architecture”, for wide band intermodulation.

Proposal 5: Out-of-band blocking and spurious response do not need requirement adjustment as they are tested with the interferer falling more than 15 MHz above or below the band and the attenuation is mainly provided by per band duplexer/filter.

Proposal 6: Put off the signalling related discussions until the UE performance aspects are clear.

- 4. Reference

| [1] | RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MTK, RAN#104 |
| --- | --- |