3GPP RAN WG4 Meeting #112	R4-2411339

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on Performance requirements of Carrier phase positioning

Source: 	CATT

Agenda item:	6.1.2.6

Document for:	Discussion

Introduction

In RAN4#111 meeting, fruitful progress regarding the performance requirements was achieved which was captured in the WF [1]. In this paper, we will further discuss a few open issues related to performance requirements.

Discussion

Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs

An open issue is to further discuss whether both the accuracies of legacy measurements and CPP measurements are verified in one TC. Proposals are copied below for reference:

Proposals: - Option 1: - Verify both the accuracies of legacy measurements and CPP measurements in one TC with a 90% success rate to reflect UE’s real positioning performance in the deployment. - Option 2: - Not verify the accuracy requirements for legacy RSTD/Rx-Tx measurement in the RSCPD/RSCP TC.

Since the legacy measurements and CPP are performed on the same PRS resources and reported together in the real deployment, therefore we think both the measurement results shall satisfy the corresponding accuracy requirements, which should be tested by one TC to reflect the UE’s real positioning performance. Though some companies concern that it may tight the accuracy requirements implicitly, UE should do their best to fulfil both the accuracy requirements of RSTD and CPP in the tests with a 90% success rate. Otherwise, passing the tests that only verify CPP measurement accuracy does not guarantee UE will have a good positioning performance in the field.

Proposal 1: Verify both the accuracies of legacy measurements and CPP measurements in one TC with a 90% success rate to reflect UE’s real positioning performance in the deployment.

Conclusions

This paper discussed the accuracy requirements for carrier phase positioning, and the following proposals are provided:

Proposal 1: Verify both the accuracies of legacy measurements and CPP measurements in one TC with a 90% success rate to reflect UE’s real positioning performance in the deployment.

References

[1] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, RAN4#111.