# 3GPP TSG-RAN WG4 Meeting #112				                       R4-2411616

# Maastricht, Netherlands, 19th - 23rd August, 2024

Source: 	Xiaomi

Title: 	Initial discussion on simulation assumptions and results for LP-WUS/WUR

Agenda Item:	8.22.4.1

Document for:	Discussion

# 1Introduction

In RAN4#111 meeting, the basic link level simulation assumptions for LP-SS evaluation were agree in [1].

In this contribution, our initial link level simulation results for OOK based LP-SS are provided.

- Discussion

- Simulation assumptions

The simulation assumptions are given in Table 1:

Table 1: General parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| LP-SS block BW | 132 subcarriers for SCS=30kHz |
| LP-SS | OOK-1 OOK-4 with M = 2,4 |
| LP-SS overhead | 2 OFDM symbols |
| LP-SS sequences | M sequence |
| BS transmit antennas for LP-SS blocks | 1 tx |
| UE receive antennas | 1 rx |
| Subcarrier spacing | 30 kHz |
| Number of LP-SS blocks per SS burst set, K | 1 |
| SNR | -3dB |
| Propagation conditions | TDL-C 300ns |

- Simulation results

Table 2: simulation results for OOK-1

|  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- |
| TDL-C | 1 sample | -4.23 | 3.56 |
|  | 3 sample | -2.71 | 2.05 |
|  | 5 sample | -2.29 | 1.90 |

Table 3: simulation results for OOK-4, M=2

|  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- |
| TDL-C | 1 sample | -4.27 | 3.36 |
|  | 3 sample | -2.91 | 2.20 |
|  | 5 sample | -2.37 | 1.85 |

Table 4: simulation results for OOK-4, M=4

|  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- |
| TDL-C | 1 sample | -4.34 | 3.60 |
|  | 3 sample | -2.97 | 1.88 |
|  | 5 sample | -2.72 | 1.63 |

Based on evaluation results shown in previous section, we could make following observations:

- For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-4 with M=2 and OOK-4 with M=4;

- The above simulation results does not model the interference from neighbor cell. Considering the potential interference as well as UE implementation(RF) margin, we think measurement with 3 samples can achieve the CA/DC Idle Mode Measurements accuracy requirements (e.g., +/- 6 dB) in all evaluated cases.

Observation 1: For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-1, OOK-4 with M=2 and OOK-4 with M=4.

Observation 2: Assuming CA/DC Idle Mode Measurements accuracy requirements (e.g., +/- 6 dB), measurement with 3 LP-SS samples is sufficient at SNR = -3dB in TDL-C channel.

- Conclusion

Observation 1: For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-1, OOK-4 with M=2 and OOK-4 with M=4.

Observation 2: Assuming CA/DC Idle Mode Measurements accuracy requirements (e.g., +/- 6 dB), measurement with 3 LP-SS samples is sufficient at SNR = -3dB in TDL-C channel.

- Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Vivo