3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411639

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	On the modification of existing RX requirements for SBFD-capable BS

Agenda item:			8.19.2.3

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. And in the RAN4#111, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6]. In this contribution, accordingly, we would like to provide our analysis and viewpoints on the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Way Forward in RAN4#111

On the topic of modification of existing requirements, the group discussed the necessity for additional co-existence study for In-band blocking and Scenario/Case for additional co-existence study, in RAN4#111, thus resulting the following way forward [6]:

Issue 3-4-1 & Issue 3-4-2: Necessity for additional co-existence study for In-band blocking and Scenario/Case for additional co-existence study (if agreed) - Way forward: - Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting: - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858 - The case to be studied is Case 3 (TDD DL to SBFD UL subband). - FFS the power level which should be collected from the simulation results to derive in-band blocking levels: - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement. - Other options are not precluded. - FFS the reference point for the power level which should be before or after RX beamforming. - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB. - FFS different grid-shift values should be considered. - Option-1: 10%. - Option-2: Other percentage values are not precluded. - FFS how to use the power level probability distributions to derive the in-band blocking levels - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios. - FFS the value of X - FFS the impact of CLI handling scheme introduced in RAN1.

# 3. Modification on Existing RX requirement for SBFD

### 3.1  OTA sensitivity

As the outcome from RAN4 study item, regarding reference sensitivity requirement for SBFD-capable BS, due to the self-interference caused internally to receiver side, RAN4 reached the following consensus [2]:

- For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed. - Otherwise, OTA sensitivity requirement could be derived based on the following equation: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase. - The following aspects need more discussion during a WI phase - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

Based on the discussion in previous two RAN4 meetings, two major issues on OTA sensitivity requirement have been raised and focused by companies:

Issue-1: Whether the performance degradation is a fixed value or declaration-based value?

Issue-2: Whether the interference considered in OTA sensitivity degradation is only self-interference or also includes other co-channel interference sources (such as inter-site interference, and inter-sector interference)?

The above two issues are correlated. Especially if the performance degradation is not declaration-based but a fixed value specified in the specification, the value itself shall be decided by Issue-2.

For the assumption for the interference considered in the OTA sensitivity degradation, we still think the principle of single BS’s UL performance shall be tested since the test is not intended to test the system, and other factors not belong to this BS shall be precluded: e.g., for traditional UL reception, interference could come from other cell’s UE uplink transmission, which is also not considered in existing OTA sensitivity test. From that perspective, self-interference is considered because it comes from the same BS implementation, while other interference including inter-site interference and inter-sector interference are hard to be modelled, especially because of the gNB-to-gNB CLI handling to be introduced in RAN1.

Proposal 1: For the assumption for the interference considered in the OTA sensitivity degradation, only self-interference shall be considered.

### 3.2  Dynamic range

Dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth. In this condition a throughput requirement shall be met for a specified reference measurement channel. For the requirement of dynamic range, the following agreements is achieved in the study phase [2]:

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase.

As proposed by other companies, both uplink signal and gNB-to-gNB CLI signal should be considered in the IoT level. The IoT level in the existing requirement is derived by assuming that: While measuring the receiver dynamic range requirement, uncertainty due to the receiver’s own thermal noise floor should be minimized. So the interference level should be increased by a certain amount of margin to mask the receiver’s own noise floor. From a scenario of view, it is reasonable to adopt the similar amount of interference signal margin for NR as for E-UTRA. The mean power of interfering signal is defined in the following method:

PIntf = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20

Based on our understanding, since the interference is already defined by 20dB over thermal noise, we see no strong reason to further increase the level of IoT to consider the impact of gNB-to-gNB CLI to be introduced in RAN1.

Proposal 2: In the existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations, by considering gNB-to-gNB CLI handling mechanism to be introduced in RAN1.

### 3.3  In-band selectivity and blocking

For the requirement of in-band selectivity and blocking, the following agreements is achieved in the study phase [2]:

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

For conducted ACS requirement, the group already agreed to “take the existing wanted signal of ACS requirement by using the existing reference sensitivity level”, for which we are not intended to reopen the discussion.

Observation 1: For conducted ACS requirement, RAN4 already agreed to “take the existing wanted signal of ACS requirement by using the existing reference sensitivity level”, which shall not be revisited.

As provided as the way forward in RAN4#111, companies are encouraged to provide the evaluation results for in-band blocking in this meeting. By following Scenario 1, 3, 5, 6, and 9, and Case 3 (TDD DL to SBFD UL subband) defined in TR 38.858, it is intended to evaluate the power level from the co-existence study to derive in-band blocking levels.

To facilitate this discussion, we firstly have the assumptions to be used in this preliminary simulation as follows:

| Simulation parameters | Value to be assumed in preliminary simulation |
| --- | --- |
| Reference point for power level | After RX subarray beamforming gain and before array gain, i.e, at TAB |
| Grid-shift | 10% |
| Probability to derive the power level | 99% |

Accordingly, we have plotted the distributions of received power levels, for (1) adjacent channel gNB DL power level, (2) wanted signal level, (3) co-channel UE UL power level and (4) co-channel gNB DL power level, for Scenario 1, 3, 5, 6, and 9 as follows:

| Scenario 1: FR1 UMa-UMa |  |
| --- | --- |
| Scenario 3: FR1 Indoor-Indoor |  |
| Scenario 5: FR1 UMi-UMi |  |
| Scenario 6: FR2 UMa-UMa |  |
| Scenario 9: FR2 Indoor-Indoor |  |

Observation 2: Based on the distributions of received power levels plotted for Scenario 1, 3, 5, 6, and 9, the adjacent channel gNB DL power level could be the major factor contributing to the total co-channel received power.

Another observation is the percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received powers, since the increased percentage grid shifting leads to higher pathloss between the interfering gNB and victim gNB directly. By adopting different percentages of grid shifting to derive the in-band blocking levels, the resultant requirements could imply certain level of deployment restriction.

Observation 3: The percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received power, by translating the distance between interfering gNB and victim gNB into pathloss.

Proposal 3: RAN4 shall align the co-existence simulation results by comparing the distributions of adjacent-channel and co-channel gNB DL received powers, by comparing the simulation assumptions and other details firstly.

### 2.4 Receiver intermodulation

For the requirement of receiver intermodulation, the following agreements is achieved in the study phase [2]:

Regarding the receiver intermodulation requirement, in general, RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement RAN4 reached the following consensus: - Conducted RX intermodulation: Adopt the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level. - OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

From previous meeting discussion, it was proposed by some company to consider “an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness”, and “whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.” Here we would like to trigger RAN4 to discuss more on this new requirement and till now, we can’t identify the necessity that the new requirement can match a certain intermodulation scenario.

Proposal 4: If RAN4 want to introduce the additional RX intermodulation requirement (a single input signal placed to cause IM with the RX sub-band), the new intermodulation scenario shall be confirmed firstly with evidence showing the proposed scenario exists in practice.

# 4. Conclusion

In this contribution, we provided the viewpoints on the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Proposal 1: For the assumption for the interference considered in the OTA sensitivity degradation, only self-interference shall be considered.

Proposal 2: In the existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations, by considering gNB-to-gNB CLI handling mechanism to be introduced in RAN1.

Observation 1: For conducted ACS requirement, RAN4 already agreed to “take the existing wanted signal of ACS requirement by using the existing reference sensitivity level”, which shall not be revisited.

Observation 2: Based on the distributions of received power levels plotted for Scenario 1, 3, 5, 6, and 9, the adjacent channel gNB DL power level could be the major factor contributing to the total co-channel received power.

Observation 3: The percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received power, by translating the distance between interfering gNB and victim gNB into pathloss.

Proposal 3: RAN4 shall align the co-existence simulation results by comparing the distributions of adjacent-channel and co-channel gNB DL received powers, by comparing the simulation assumptions and other details firstly.

Proposal 4: If RAN4 want to introduce the additional RX intermodulation requirement (a single input signal placed to cause IM with the RX sub-band), the new intermodulation scenario shall be confirmed firstly with evidence showing the proposed scenario exists in practice.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.