3GPP TSG-RAN WG4 Meeting #112	R4-2412060
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.12.3

Source:	vivo

Title:	Discussions on Rel-19 FR1 OTA requirement work

Document for:	Approval

1	Introduction

In the agreed workplan [1], RAN4 need to start the performance part discussion in this meeting:

- RAN4 #112 (Aug 2024)

- General

- Update Workplan based on RAN plenary decision on FR2 MIMO OTA requirements scope

- Core part

- XR test methodology test methodology

  - Make progress on testing time reduction solutions for XR

  - Make progress on other XR test method related aspects

- NTN test methodology

  - Make progress on usage scenarios and test methodologies for NR NTN and IoT NTN devices

  - Make progress on performance metric for NR NTN and IoT NTN devices

- FR1 MIMO OTA dynamic channel model test methodology

  - Make progress on channel modelling, environmental condition and validation approach for dynamic channel model

  - Make progress on UE performance metric under dynamic channel model

- Performance part started

- TRP TRS requirements

  - Initial views on framework and prioritized bands for 1Tx FR1 TRP TRS requirement

  - Initial views on framework and 2Tx lab alignment activity for 2Tx non-coherent UE

- MIMO OTA requirements

  - Initial views on framework and prioritized bands for FR1 MIMO OTA requirement

  - FR2 TBD

This contribution provides our initial views on Rel-19 FR1 OTA requirement work.

2	Discussion

2.1 TRP TRS requirements work

The working scope for performance part has been updated in RAN#104 meeting [2].

- Specify TRP and TRS requirements:

  - Specify FR1 TRP and TRS requirements for NR bands for UE based on operators demand

    - Handheld UEs with 1Tx at new bands n3, n5, n7, n8, n20, n38, n77, n79 are first priority

      - RAN4 may further down-select the number of bands. Check in RAN#107

    - Handheld UEs with 2Tx (non-coherent UL MIMO) at n41 and n78 depending on measurement data availability are first priority.  Additionally, n77 and n79 are second priority.

    - Headworn XR OTA requirements will not be considered until development of suitable head phantom and sufficient number of commercial devices become available

    - Coherent UL MIMO UE OTA requirements will not be considered until sufficient number of commercial devices become available

    - Size 2 Handheld UEs with 1Tx at legacy bands n1, n28, n41, n78 are first priority

Note: Handheld UEs includes Size 1 (wide, width >72mm and ≤92mm) and Size 2 (narrow, width ≥56mm and ≤72mm);

For 1Tx requirements, the AC lab alignment activities in Rel-17 [3] and Rel-18 [4] have fully demonstrated the performance of each volunteer test labs, there is no need to perform further alignment activity at other bands with UE 1Tx configuration.

Another issue is the band down-selection, the supported bands is highly related to the UE market and operators’ deployment, feedback from vendors and operators is needed on which band should be focused for collecting measurement results.

Observation 1: For 1Tx requirements work, there is no need to perform further AC lab alignment testing in Rel-19.

Proposal 1: Feedback from interested companies is needed to check the number of available devices for different bands, then RAN4 can make reasonable decision on a smaller scope of 1st priority bands in Rel-19.

For non-coherent UL MIMO radiated performance, this is new work in the OTA industry, no SDO has defined the non-coherent UL MIMO radiated performance, the test labs performance under UE 2Tx condition is not ensured. Therefore, corresponding lab alignment activity for AC system to specify non-coherent UL MIMO TRP requirements is needed.

Proposal 2: AC lab alignment for non-coherent UL MIMO at n41/n78 should be done in Rel-19, the framework can be similar to Rel-18 [5], encourage test labs to join this activity.

For XR and coherent UL MIMO, the performance work should be postponed until the phantom and commercial devices available.

It has been agreed that RAN4 should conclude the Rel-18 requirements for bands 1/28/41/78 for Size 1 handheld UE in RAN4#112 meeting. Then RAN4 can further discuss whether the requirements for Size 2 Handheld UEs with 1Tx could be derived based on size 1 values of same band.

Otherwise, new measurement is needed, but considering the workload to specify both 1Tx requirements and 2Tx lab alignment activity, it is suggested that RAN4 do not consider a measurement-based approach to specify Size 2 Handheld UE requirements at band 1/28/41/78.

Proposal 3: The Size 2 requirement can be specified based on an offset-based approach, rather than new measurement campaign.

Proposal 4: If the offset-based approach is agreed, RAN4 can also further discuss whether small number of measurements is needed for performance checking to confirm the offset value.

2.2 FR1 MIMO OTA requirements work

The working scope for MIMO OTA performance part has also been updated in RAN#104 meeting [2].

- Specify MIMO OTA requirements:

  - Specify FR1 MIMO OTA requirements for new bands for handheld UE based on operators demand (2 layers or 4 layers)

    - Band n3, n8, n77, n79 are first priority

    - The requirements are specified based on Rel-17 static MPAC test method

Similar situation as TRP TRS 1Tx requirements, no further lab alignment activity is needed for FR1 MIMO OTA, and the band down-selection can also be discussed.

Proposal 5: For TRP TRS 1Tx requirements work and FR1 MIMO OTA requirements work, there is no need to perform further lab alignment testing in Rel-19.

The measurement campaign can be started early after the volunteer test lab, framework and targeted bands are confirmed.

3 Conclusion

In this paper, we provide our further views on FR1 OTA requirement work.

Observation 1: For 1Tx requirements work, there is no need to perform further AC lab alignment testing in Rel-19.

Proposal 1: Feedback from interested companies is needed to check the number of available devices for different bands, then RAN4 can make reasonable decision on a smaller scope of 1st priority bands in Rel-19.

Proposal 2: AC lab alignment for non-coherent UL MIMO at n41/n78 should be done in Rel-19, the framework can be similar to Rel-18 [5], encourage test labs to join this activity.

Proposal 3: The Size 2 requirement can be specified based on an offset-based approach, rather than new measurement campaign.

Proposal 4: If the offset-based approach is agreed, RAN4 can also further discuss whether small number of measurements is needed for performance checking to confirm the offset value.

Proposal 5: For TRP TRS 1Tx requirements work and FR1 MIMO OTA requirements work, there is no need to perform further lab alignment testing in Rel-19.

4	References

- R4-2406084, Workplan for Rel-19 OTA WI, vivo, CAICT, RAN4#110bis, Apr 2024.

- RP‑241623, Revised WID: FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN#104, June 17-20, 2024

- R4-2212817, Outcome of 3GPP TRP TRS lab alignment activity, vivo, RAN4#104-e, 2022

- R4-2408096, Final Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results, vivo, RAN4#111, 2024

- R4-2312568, Further updated working procedure for Rel-18 TRP TRS lab alignment and harmonization campaign, vivo, RAN4#108