3GPP TSG-RAN WG4 Meeting #112	R4-2412670

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	8.22.4.2

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#111, and the outcomes are captured in WF [1]. Based on [1] the following issues need to be further discussed.

- General aspects

- LP-WUR measurement

- MR relaxation

- Entry/exit criteria

- LP-WUR at CONNECTED mode

In this paper, we will provide our views on RRM requirements related to LP-WUR. Besides, we will also provide updates to the simulation assumption based on latest RAN1 progress.

Discussion

General aspects

RRM requirements to be defined

In RAN2#126, the following is agreed regarding entry/exit of LP-WUS monitoring.

- Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Our understanding of the agreement is that

- When the entry condition for LP-WUS monitoring is met, UE may monitor LP-WUS with LP-WUR and if it does so, it may turn off MR for paging monitoring, and UE should wake up MR for paging reception when it receives WUS targeting at it.

- When the exit condition for LP-WUS monitoring is met, UE should turn on MR for paging monitoring.

For the procedure to work properly, RAN4 needs to define requirement for serving cell evaluation against the entry/exit condition for LP-WUS monitoring. The requirements should include at least 3 aspects – measurement delay, measurement accuracy and measurement filtering.

Measurement delay

RAN4#110-bis already agreed to define requirements for LR measurement which is used for evaluating the exit condition, and it at least includes the measurement delay in terms of number of samples. What RAN4 has missed so far is the requirements for MR measurement which is used for evaluating the entry condition, but most likely the existing serving cell evaluation requirements in clause 4.2.2.2 (for evaluation against the S-criterion) can be re-used also for evaluation against the entry condition for LP-WUS monitoring.

Measurement accuracy

RAN4#110-bis also discussed the measurement accuracy, but it was left open whether and how to define the accuracy requirement.

We support to define the accuracy requirements as it is important for correct triggering of MR for paging monitoring based on the exit condition. Among the options for how to specify the requirements, we support option 2, which is same as cell reselection requirements, where accuracy is implicitly defined as margin (highlighted in yellow in the copied requirements).

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Measurement filtering

The issue was raised up in RAN4#111 by P1-1 of Issue 1-1-4.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation - Proposals - P1-1: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (Qualcomm) - The number of measurement samples needed for filtering. - The number of consecutive times the UE needs to meet the entry/exit criteria - P1-2: Define requirements on entry/exit criteria for LP-WUR measurement/ MR RRM measurement relaxation based on RAN1 and RAN2 design (CT) - P1-3: RAN4 to discuss the requirement about exit the LR by serving cell evaluation based on the LP-RSRP/LP-RSRQ simulation result and the requirement about entry the LR by serving cell evaluation based on the SSB RSRP/RSRQ. (Ericsson) Recommendations:

In general, we support to define filtering as part of the requirements for serving cell evaluation against the entry/exit condition, which is also defined in existing serving cell evaluation requirements in clause 4.2.2.2 (for evaluation against the S-criterion).

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

In our view, the filtering requirement is different from delay requirement. The latter defines the upper limit on the time that UE can use to achieve certain accuracy, while the former defines the lower limit on the time the UE has to spend on the evaluation to eliminate the impact of fast fading. The filtering requirement also defines the minimum separation between samples.

Summary

Based on above discussions, we have the following proposal on the RRM requirements to be defined.

Proposal 1: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including

- Measurement delay (number of samples)

- Measurement accuracy (margin for evaluation)

- Measurement filtering (number of samples and separation between samples)

Cases/States to be considered for MR and LR measurement

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

RAN4#111 discussed the Cases/States to be considered for MR and LR measurement. Case #1 fully offloading case was agreed, while other cases with MR measurement are FFS.

Case #2

We do not support to consider Case #2.

- There is no clear necessity to require MR to measure the serving cell since LR is already measuring the serving cell.

  - In particular, the MR measurement will be with relaxation, so it is much more infrequent compared to the LR measurement.

  - Some companies commented that LR measurement may be less accurate than that of MR. In our view, however, it is both necessary and feasible to keep similar accuracy between LR and MR.

  - Some companies commented that MR measurement may be used by UE to assist LR measurement. We believe this can be left to UE implementation rather than mandated by requirements.

- There is additional power consumption due to MR measurement even with relaxation.

- It will complicate the spec since another threshold (coverage level) is needed to switch between Case #1 and Case #2.

Case #3

We support to consider Case #3.

- Neighbor cell measurement should be managed separately than RRM offloading, and it would limit NW flexibility and use of LR if they cannot be enabled at same time.

- When MR is already used for neighbor cell measurement, it is better to require UE to also use MR for serving cell measurement.

  - The additional power consumption from serving cell measurement is marginal.

  - It is difficult to compare the serving cell and neighbor cell measurements if the former is measured by LR and the latter is measured by MR.

- Some companies asked how to switch from Case #1 to Case #3. Technically, we do not see an issue to use LR serving cell measurement to trigger neighbor cell measurement, except that the existing threshold may not be applicable if the LR measurement is based on LP-SS because LP-RSRP/Q may not be directly comparable with the existing threshold for SS-RSRP/Q. How to handle the threshold can be left to RAN2.

Case #4

We support to consider Case #4. In our understanding, Case #4 is a specific case when neighbour cell measurement is triggered by higher priority carriers. Since evaluation for cell reselection to a higher priority carrier does not require a comparison between serving cell and neighbour cell measurement, there is no need to require UE to use MR to measure serving cell.

Summary

Based on above discussions, we have the following proposal on the RRM requirements to be defined.

Proposal 2: RAN4 to support the following cases for MR and LR measurement in addition to fully offloading case.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement | Remarks |
| --- | --- | --- | --- | --- |
| #3 Relaxed case b | ON with relaxation measurement | ON with relaxation measurement | ON | Neighboring cell measurement on lower or equal priority carrier is triggered based on serving cell quality measured by LR |
| #4 Relaxed case c | OFF | ON with relaxation measurement | ON | Neighboring cell measurement on higher priority carrier is triggered |

LP-WUR measurement

Requirements for different LR architectures

Issue 1-1-1: Core requirements to be specified for LP-WUR measurement - Proposals - P1: RAN4 shall focus on following cases (Samsung CMCC vivo ZTE): - Case 1: LP-SS for OOK based LP-WUR; - Case 2: Existing PSS/SSS for OFDM based on LP-WUR; - P1-1: Suspend the discussion of OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state, until RAN1 achieve the consensus about whether it can be target for sync and RRM measurement. (Samsung CMCC) - P1-2: It is not necessary for RAN4 to define measurement requirements for OFDM-based LP-SS RRM measurement at least in this stage. (ZTE) - P2: New requirements may need be specified for the interruption time between LP-WUS received paging indication to MR ready for Paging monitoring (Samsung xiaomi Docomo) - P2-1: RAN4 to define requirements on UE paging monitoring to make sure that UE does not miss paging when entry/exit condition for LP-WUS monitoring is met (HW) - P3: RAN4 to discuss what is the impact of different architecture options on LP-WUR requirement and study the need to define requirements separately for supported architecture types (Nokia) - P4: The feasibility of OFDM-based LP-SS measurement shall be discussed, a LS could be sent to RAN1 on RAN’4 conclusion. (ZTE) Background: RAN4 110bis agreement - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS Recommendations: - Update the first bullet of RAN4 110bis’s agreement on issue 2-2-1 as - Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Suspend the discussion of OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state until more RAN1 progress is available

We support to define following requirements for different LR architectures.

- OOK based LR measurement on LP-SS

- OFDM based LR measurement on LP-SS

- OFDM based LR measurement on SSB

1) is straightforward since OOK based LR can only measure LP-SS. OFDM based LR can measure both LP-SS and SSB, so we support to define requirements for both 2) and 3). If only of them is defined, it implies that OFDM based LR is mandated to measure a certain signal, which unnecessarily limits the flexibility of OFDM based LR.

RAN4#110-bis already agreed to define requirements for 1) and 3). For 2) some companies proposed to suspend the discussion until RAN1 makes conclusion on whether overlaid sequence on LP-SS is supported. In our view, RAN4 should define requirements for 2) no matter of RAN1 supports overlaid sequence on LP-SS or not. Supporting overlaid sequence on LP-SS will allow better sync and measurement performance for OFDM based LR, but OFDM based LR can use LP-SS for sync and measurement without overlaid sequence.

One may ask if OFDM based LR can measure SSB, why does it need to measure LP-SS. In our view, LP-SS can be assumed at the same center frequency as LP-WUS, and it is very likely to be outside the bandwidth of initial DL BWP containing SSB, in order to avoid reducing the scheduling resources for common channels. Using LP-SS allows OFDM based LR to perform sync and RRM measurement without RF re-tuning. Also, the measurement from LP-SS can better reflect the channel quality for LP-WUS reception compared to measurement from SSB which may be far away from LP-WUS in frequency domain.

Based on above discussions, we suggest RAN4 to define requirements for 2). The exact requirements and applicability can be FFS until RAN1 conclusion on whether to support overlaid sequence on LP-SS, as well as RAN4 initial evaluation on the performance of 1), 2) and 3), are available.

Proposal 3: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. Exact requirements and applicability are FFS.

SINR target

Issue 1-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode - Proposals - P1: RAN4 to decide the Es/Iot side condition for LP-WUR based RRM requirement when we have RAN1 conclusion on target SINR for coverage and RAN4 RF conclusion on noise figure (Apple CMCC) - P2: Update the wording of previous agreement as “The outcome of RAN1’s study in Rel-19 WI on SINR target is used as the starting point for RAN4 LP-WUR requirement study”. (vivo) - P3:RAN4 to wait for the RAN1 conclusions on the target SNR (Nokia ZTE) - P3-1: RAN4 shall study the target SNR as baseline since RAN1 has agreed the target SNR shall be reported. (ZTE) - P4: Given any outcome from RAN1 discussion on the target SNR, RAN4 still need to consider other RF impairments to determine the final value for the target SNR. RAN4 to evaluate the RRM performance for LP-SS using the existing side condition of the Idle mode performance requirements (i.e., -4dB for Idle mode in FR1). (MTK) Recommendations: Issue 1-2-2: Noise figure and others impact(RF impairment) on SNR target X - Proposals - P1: Not consider adjustment on the target SNR from RAN1 due to the noise figure of LP-WUR since it has already been considered in RAN1 assumption. (oppo) Recommendations:

RAN#117 made the following agreement regarding the SNR values to achieve coverage of PUSCH for message3. It is noted that the NF difference between LR and MR is already considered when RAN1 derives the values, so RAN4 does not need to further discuss the impact of NF.

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

Based on RAN1 agreements, we suggest to use -3dB SINR as the starting point for defining requirements for LR measurement. It corresponds to the worst-case assumption of NF which should be covered by the minimum requirements.

Proposal 4: Use -3dB SINR as starting point for defining requirements for LR measurements.

Accuracy

Issue 1-2-3: Accuracy for LP-WUR measurement - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (Apple xiaomi LG Ericsson Huawei MTK ZTE) - P2: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline. FFS on whether or how to define dedicated accuracy requirements. (CATT CMCC) - P3: RAN4 to consider a common target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR. (xiaomi) - P4: First determine the measurement accuracy based on the legacy RAN4 requirement and target SNR/SINR based on RAN1 Rel-19 study. Determine required measurement samples based on RAN4 simulations. Determine measurement delay based on sampling number and reference signal periodicity. RAN4 to decide whether the legacy EMR Idle mode accuracy requirement and Connected mode accuracy requirement can be reused for LP-WUR measurements later based on the simulation results (CT) - P5: RAN4 to define the dedicated accuracy requirement in the performance section (CT) - P6: The accuracy requirement defined for Redcap with 1Rx can be used as the base when defining requirements for LP-WUR serving cell measurement. (vivo) - P6-1: Further relaxation on the accuracy target based on reference accuracy maybe needed. The amount of relaxation could be based on RAN4’s simulation outcome. (vivo) Recommendations:

We do not see a clear need to define a baseline for the accuracy requirements. We understand the accuracy from LR measurement should be on similar level as existing MR measurement, but RAN4 does not need to agree on which of the existing requirements for MR should be used as baseline. Instead, RAN4 can simply focus on the simulation assumption, agree on the SINR conditions and number of samples for evaluation, and check the simulation results. If the resulted accuracy is too bad, RAN4 can further adjust the SINR condition or the sample numbers. The exact accuracy requirements would then be derived from simulation results for the selected assumptions on the SINR condition and sample number.

Proposal 5: RAN4 not to define baseline for accuracy requirements for LR measurement.

Time and frequency error

Issue 1-2-5: Time/frequency sync - Proposals - P1: RAN4 to wait conclusions from RAN1 on timing error and frequency error. (Apple vivo) - P2: RAN4 to discuss time and frequency error when agreeing on the simulation assumption for evaluating LP-WUR measurement performance. (Huawei) Recommendations:

RAN1#117 agreed on the following regarding to residual frequency error that LR may experience.

Agreement (further amended as shown in red on Friday) The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR. - FFS X which is no larger than 20ppm - FFS: Initial frequency error assumption: 200ppm - Initial frequency error assumption: up to company report

Based on our understanding, the residual frequency error after frequency error correction can be better than 20ppm, and we suggest to use 5ppm for the simulation. We do not need to consider the impact of drift, as the frequency drift is quite slow compared to the LR measurement period.

On timing error, it should compose a residual frequency error after timing error correction and an impact of drift. Based on 5ppm frequency error and 320ms measurement interval, the timing drift would be 1.6us. The residual error depends on the sampling rate and how accurate the time sync can be done. The latter depends on the receiver (OOK or OFDM based) and whether there is overlaid sequence on LP-SS. For the simulation assumption, we suggest to leave this to companies to report, and RAN4 can further calibrate.

It is noted that the LR need to do time sync for each of the LP-SS or SSB occasion before measurement, so the time error that measurement experience is the residual timing error.

Proposal 6: RAN4 to use the following value for evaluating LR measurement performance.

- Frequency error: 5ppm

- Time error: up to companies to report

Measurement interval

In order to define the measurement period requirements for LR, RAN4 needs to discuss not only the number of samples based on simulation, but also the measurement interval. In previous meetings, some proposals were raised up, e.g. LP-SS periodicity or DRX/eDRX cycle.

The LR measurement are at least for two purposes. One is to get sync for LP-WUS monitoring, for which it may be sufficient to measure based on DRX/eDRX cycle, i.e. before LP-WUS monitoring. The other is to evaluate against the exit threshold so that UE would wake up MR timely when LR is not supposed to work. For the second purpose, the LR measurement should follow a smaller periodicity e.g. 320ms for LP-SS, as otherwise the delay for exiting LR can be too long and may cause missing paging.

Considering that the LR measurement is not power hungry, we suggest to define the LR measurement requirements based on [320]ms for both LP-SS and SSB.

Proposal 7: Define the LR measurement requirements based on measurement interval of [320]ms for both LP-SS and SSB based measurement.

MR relaxation

Requirements for serving and neighbour cell

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation - Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple xiaomi oppo LG CT ZTE) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. If both LP-WUR and MR are ON, RAN4 to discuss whether UE uses LP-WUR measurement to decide MR relaxation or UE uses both LP-WUR and MR measurement to decide MR relaxation (Apple) - P1-1: RAN4 to specify MR RRM relaxation requirements for serving cell measurements in Idle/Inactive mode when (MTK) - LP-SS based LP-WUR RRM measurement is activated, or - PSS/SSS based LP-WUR RRM measurement is activated Not to consider RRM relaxation based on the hybrid measurements of both (LP-SS and SSS). Recommendations:

Based on Proposal 2 in section 2.1.2 for Cases/States to be considered for MR and LR measurement, our view is that RAN4 should define relaxed MR measurement requirements for both serving and neighbor cells.

It is noted that the discussed cases in section 2.1.2 are when serving cell RRM is offloaded to LR, so the condition in P1 (UE supporting LP-WUR) is not accurate.

Proposal 8: RAN4 to specify relaxed MR measurement requirements for both serving and neighbor cells when serving cell RRM is offloaded to LR.

Relaxation factor

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell - Proposals - P1: Regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, RAN4 to discuss (Apple) - whether serving and neighbor cell measurements can share the same criteria for triggering relaxation or not and, - whether serving and neighbor cell measurements can share the same relaxation requirement (e.g., scaling factor) or not. - P2: RAN4 needs to further study serving cell measurement relaxation, and further neighboring cell measurement relaxation by MR (Samsung CATT Nokia) - P2-1 (CATT): What is the difference between MR RRM measurement relaxation and legacy RRM measurement relaxation (e.g., not-at-cell edge or low mobility in R16/R17)? Whether the legacy criteria for RRM relaxation can be reused for relaxing serving and neighbor cell measurements for UE MR ? Whether RRM measurement of MR is used for neighbor cell measurement when the UE is operating with LP-WUR can be discussed after further input from RAN2. After determining the meaning and the criteria (entry/exit conditions) for MR RRM measurement relaxation in RAN2/RAN4, the method and requirements for MR RRM relaxation will be further discussed in RAN4. - P3: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation (CT vivo ZTE) - P2-1: Equal or larger than 8 (CMCC) - P2-2: >=16 (Huawei) - P4: RAN4 to wait other WGs’ progress to clarify the MR RRM relaxation scenario before discussing the relaxation requirement. (Ericsson) - P5: The legacy SINR condition and accuracy are re-used(HW) Recommendations:

On the delay requirements for relaxed MR measurement, as the relaxation is aimed at power saving, we believe the power saving gain should be the main factor to determine the relaxation factor. Figure 1 shows our evaluation on the power saving gain from SI for MR relaxed measurement, with different relaxation factors.

Figure 1: Power saving gain for MR relaxed measurement with different relaxation factors

It can be seen that the power saving gain reduced obviously when relaxation factor becomes smaller. With 10 times relaxation, the gain is 20% in ideal case for paging, and to achieve meaningful gain in realistic case, the relaxation factor needs to be > 20. We suggest RAN4 to consider 16 as the smallest relaxation factor.

P1 raised up an issue whether serving and neighbor cell measurements can share the same scaling factor. Our view is ‘yes’ because the power consumption of the MR measurement is determined by both serving and neighbor cell measurements. A meaningful power saving gain is obtained only when both measurements are relaxed with relaxation factor >= 16.

Proposal 9: For MR measurement with relaxation the relaxation factor should be >= 16, and same relaxation factor applies to serving and neighbor cell measurements.

Entry/exit criteria

LP-WUS monitoring

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring - Proposals - P1: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary (Apple) - P2: For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ (Samsung) - P3-1: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring (xiaomi) - P3-2: The entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement should be considered jointly.(CMCC) - P3-3: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and LP-WUR RRM measurement. (vivo) - P3-4: RAN4 to confirm that same entry/exit condition for LP-WUS monitoring applies for serving cell RRM measurement offloading (from MR to LP-WUR). (HW) Notes: MR measurements results shall be larger than the threshold and satisfy the related MR relaxation conditions. (ZTE) - P4: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2 (LG) - P5: For the entry/exit conditions for LP-WUS monitoring, the threshold for the serving cell measurement performed by MR or LR shall be decided by RAN4 (ZTE) Recommendations: Suggest to check whether the following top level description: LP-WUR monitoring is triggered no later than one of cases identified in issue 1-1-2

RAN1/2 has agreed on the baseline for the entry/exit condition for LP-WUS monitoring, and it is based on serving cell quality, i.e. RSRP and RSRQ. In our view, serving cell quality mainly reflects the coverage level, and another factor that needs to considered for determining whether UE should enter or exit LP-WUS monitoring is the inter-carrier interference level.

As the dynamic range of LR is smaller compared to MR, e.g. the bitmap for ADC is assumed to be 4 or 8 bits, it is more likely to get blocked by the inter-carrier interference. The issue was discussed in RF session, where the conclusion was that “RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR”.

In our view, interference level that LR can tolerate is smaller than that for MR. On the other hand, the actual interference level does not change depending on whether UE is using LR or MR in another carrier which may be used by another operator. Therefore, even UE is close to the serving gNB, i.e. the serving cell RSRP and RSRQ are high, the LR may not be usable because of high inter-carrier interference. In this case, the UE should not use LR as the LP-WUS detection performance cannot be guaranteed.

We suggest RAN4 to consider additional entry/exit condition for LP-WUS monitoring on inter-carrier interference level besides the serving cell quality, and we are open to further discuss the details, e.g. how to quantify the interference level.

Proposal 10: For LP-WUS monitoring, RAN4 to consider additional entry/exit condition on inter-carrier interference level besides the serving cell quality.

RRM offloading to LR

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (Apple oppo vivo) - P2: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition - P3: RAN4 to firstly discuss the differences between entry/exit conditions for LP-WUS monitoring defined in RAN1/RAN2 and conditions for LP-WUR measurement. RAN4 should at least consider LP-SS and PSS/SSS when discussing the entry/exit conditions for LP-WUR measurement. RAN4 to wait for RAN1/RAN2, if the entry/exit conditions for LP-WUS monitoring are introduced by RAN1/RAN2, the thresholds on serving cell measurement result can be further discussed in RAN4. (CATT) - P4: Both serving cell quality and UE mobility should be considered for the criteria(entry/exit conditions) for MR RRM measurement relaxation &LP-WUR measurement (State2) and only LP-WUR measurement (State 3). Whether network configure the high priority frequency carrier measurement or not should also be considered when defining the criteria (entry/exit conditions). (CMCC) - P5: When UE enters the State 3 (Measurement only based on LP-WUR), The serving cell quality should at least be Srxlev > SIntraSearchP and Squal > SIntraSearchQ (CMCC) - P6: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective (LG) - P7: RAN4 to discuss serving cell evaluation for entry/exit conditions for PSS/SSS and LP-SS based LR. PSS/SSS based entry/exit evaluation can be prioritised. (Nokia) Recommendations:

In RAN2#126, the following agreement was reached regarding serving cell RRM measurement offloading. It can be seen that RAN2 will focus on specifying the offloading criterion. To avoid duplicated discussion, we suggest RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR, unless triggered by other WGs.

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

Proposal 11: RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR unless triggered by other WGs.

MR relax measurement

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (oppo vivo Apple) - RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2. (Apple) - P2: RAN4 should firstly discuss the meaning of MR RRM measurement relaxation on which phases and what purposes are included for RRM relaxation in the entire procedure, i.e. for cell selection/reselection or handover, for serving cell measurement or neighbor cell measurement, etc. (CATT) - P3: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation (LG) - P3a: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition Recommendations:

In RAN2#126, the following agreement was reached regarding MR measurement relaxation. It can be seen that RAN2 will further discuss the neighbor cell measurement relaxation criteria. The MR measurement of serving cell can be discussed after RAN4 concludes on the Cases/States to be considered for MR and LR measurement as discussed in section 2.1.2. To avoid duplicated discussion, we suggest RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

- RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Proposal 12: RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

LP-WUR at CONNECTED mode

Issue 1-5-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is for connected mode in this WI. (Apple Docomo oppo LG CT vivo ZTE) - P2: Postponed until more progress (Ericsson) - P3: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress (Samsung) Recommendations:

We support P2. It is true that the objective of LP-WUR at CONNECTED mode does not involve RAN4, but it may be too earlier to preclude any RRM impacts as the RAN1/2 discussions are ongoing, and there may be RRM impacts e.g. as proposed in P3. RAN4 can first focus on the LP-WUR requirements in IDLE mode, and postpone the discussion for CONNECTED mode until more RAN1/2 conclusions are available.

Proposal 13: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Conclusions

In this paper we provided our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including

- Measurement delay (number of samples)

- Measurement accuracy (margin for evaluation)

- Measurement filtering (number of samples and separation between samples)

Proposal 2: RAN4 to support the following cases for MR and LR measurement in addition to fully offloading case.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement | Remarks |
| --- | --- | --- | --- | --- |
| #3 Relaxed case b | ON with relaxation measurement | ON with relaxation measurement | ON | Neighboring cell measurement on lower or equal priority carrier is triggered based on serving cell quality measured by LR |
| #4 Relaxed case c | OFF | ON with relaxation measurement | ON | Neighboring cell measurement on higher priority carrier is triggered |

Proposal 3: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. Exact requirements and applicability are FFS.

Proposal 4: Use -3dB SINR as starting point for defining requirements for LR measurements.

Proposal 5: RAN4 not to define baseline for accuracy requirements for LR measurement.

Proposal 6: RAN4 to use the following value for evaluating LR measurement performance.

- Frequency error: 5ppm

- Time error: up to companies to report

Proposal 7: Define the LR measurement requirements based on measurement interval of [320]ms for both LP-SS and SSB based measurement.

Proposal 8: RAN4 to specify relaxed MR measurement requirements for both serving and neighbor cells when serving cell RRM is offloaded to LR.

Proposal 9: For MR measurement with relaxation the relaxation factor should be >= 16, and same relaxation factor applies to serving and neighbor cell measurements.

Proposal 10: For LP-WUS monitoring, RAN4 to consider additional entry/exit condition on inter-carrier interference level besides the serving cell quality.

Proposal 11: RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR unless triggered by other WGs.

Proposal 12: RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

Proposal 13: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo