3GPP TSG-RAN WG4 Meeting #112							R4-2412730

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.5.2.1

Source: 	OPPO

Title: 	on intra-band non-collocated UE RF requirement

Document for:	Approval

- Introduction

In RAN4#110bis meeting, the intra-band non-collocated UE RF requirements has been discussed and the WF [1] has been agreed. In this paper, we give further analysis on the UE RF requirement.

- Discussion

During the last RAN4#110bis meeting, the agreement for power imbalance requirement for intra-band non-collocated UE RF requirements has been captured as below from the WF[1].

< Issue 2-1-1:  Power imbalance requirements and REFSENS >

Agreement:

- Use 25dB power imbalance for 4Rx REFSENS + 1dB for Type 4

< Issue 2-1-2:  How to capture the power imbalance and REFSENS requirements >

Agreement:

- Define the Type 4 receiver imbalance requirements in the same table as for Type 2.

  - FFS on the details of modification

< Issue 2-1-3:  Minimum DL frequency separation >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

  - Option 2. Max (5/2* BWanother, [50] MHz).

< Issue 2-1-4:  Maximum DL frequency separation >

Agreement:

- No limitation for maximum DL frequency separation

Firstly, on the power imbalance requirement, the 25dB power imbalance for 4RX REFSENS+1dB for type 4 has been agreed. How to capture the requirement needs further discussion. Based on the experience of type 2 requirement, the UE behaviour and capability signalling as well as the NW signalling all need to be considered. Before going to detail wording as well as the signalling, some initial high level agreement can be reached.

7.10A	Power imbalance for CA

7.10A.1	General

Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal.

Power imbalance requirement in this subclause is only applicable for a UE capable of intraBandNR-CA-non-collocated-r18 and is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

7.10A.2	Minimum requirement

For the test parameters in table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. Four antenna port reference sensitivity allowance ΔRIB,4R in table 7.3.2-2 needs to be considered for UE equipped with more than 4 antenna port. NOTE 5: Void. |  |  |  |  |

For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

For a UE capable of [Rel-19 capability] for the following CA band combinations in table 7.10A.2-2, the RX requirements for two RX ports are applicable for the component carrier if it is provided with [Rel-19 NW signalling type 4a] and RX requirements for four RX ports are applicable for the component carrier if it is provided with [Rel-19 NW signalling type 4b].

Table 7.10A.2-2: NR CA combinations

CA combination

CA_n77(2A)1

CA_n78(2A)1

NOTE 1: The applicability is specified in clause 4.2 d)

As marked as yellow in the above note 3 and note 4 which are captured from the latest TS 38.101-1, we deleted type 2 since type 2 is not a definition used in the TS 38.101-1 as well as other specifications. One solution might use the capability signalling to indicate the UE type and this can be further discussed.

Proposal 1: To capture the type 4 UE RF requirement, current note3 in table 7.10A.2-1 needs to update with both type 2 and type 4 but not directly use the terminology of “type 2” and “type 4”.

Secondly, for the Note 4 in table 7.10A.2-1, since only type 2 is introduced and hence the 2RX REFSENS is used. However, with the introduction of type 4a/4b, 4RX REFSENS will be used. In such case, the ΔRIB,4R in table 7.3.2-2 needs to be considered and hence to update the note 4 correspondingly.

Proposal 2: To capture the type 4 UE RF requirement, current note 4 in table 7.10A.2-1 needs to update with the ΔRIB,4R in table 7.3.2-2 to consider the 4RX REFSENS.

Thirdly, the paragraph describing the RX requirement applicability will also needs update. Currently the 2RX requirement apply for each component carrier if UE is working at type 2. In such case, for type 4a UE, the component carrier configured with 2 layers should fulfil 2RX requirement while the component carrier configured with 4 layers should fulfil 4RX requirement. Same to type 4b that 4RX requirement apply for each component carrier.

Proposal 3: for type 4a UE, the component carrier configured with 2 layers should fulfil 2RX requirement while the component carrier configured with 4 layers should fulfil 4RX requirement.

Proposal 4: For type 4b UE. 4RX requirement apply for each component carrier.

With the above high level agreement, then further wording refinement can be discussed.

For the minimum DL frequency separation, two options are provided as

  - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

  - Option 2. Max (5/2* BWanother, [50] MHz).

For option 1, it is from the exact deployment frequency range while for option 2 it is from the testing configuration perspective. As discussed also above, the note 3 in table 7.10A.2-1 that the power imbalance will be verified in one of the test configurations. This means the UE should in any case fulfil the power imbalance requirement in the channel bandwidth configuration as separation with Max (5/2* BWanother, [50] MHz). Hence to use option 2 as the DL separation is feasible. However, as this has already defined in the RF imbalance requirement, we believe there is no need to re-capture such limitation.

Proposal 5: The min DL separation is proposed as Max (5/2* BWanother, [50] MHz). There is no need to further capture this in the specification.

3	Conclusions

In this paper, we give further discussion on Type 4 UE RF requirements.

Proposal 1: To capture the type 4 UE RF requirement, current note3 in table 7.10A.2-1 needs to update with both type 2 and type 4 but not directly use the terminology of “type 2” and “type 4”.

Proposal 2: To capture the type 4 UE RF requirement, current note 4 in table 7.10A.2-1 needs to update with the ΔRIB,4R in table 7.3.2-2 to consider the 4RX REFSENS.

Proposal 3: for type 4a UE, the component carrier configured with 2 layers should fulfil 2RX requirement while the component carrier configured with 4 layers should fulfil 4RX requirement.

Proposal 4: For type 4b UE. 4RX requirement apply for each component carrier.

Proposal 5: The min DL separation is proposed as Max (5/2* BWanother, [50] MHz). There is no need to further capture this in the specification.

4 References

[1] R4-2410686 	WF on Rel-19 non-collocated UE RF requirements, KDDI