3GPP TSG-RAN WG4 Meeting #112	R4-2413318

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	DL interruptions for mid band combinations

Source:	Qualcomm Incorporated

Agenda item:	7.10.2

Work Item: 	DL_intrpt_combos_TxSW_R19-Core

Document for:	Approval

# 1.	Introduction

New WI [1] was introduced in RAN#104 that discusses and possibly mandates UE to support TX switching without DL interruption and switching event and force not to use the capability uplinkTxSwitching-DL-Interruption-r16 to indicate interruption for a band pair. In this paper we discuss one band combination specifics for the interruption.

# 2. 	Discussion

In the WID, the table 1 indicates the band combination candidates for refusing DL interruptions.

Table 1: Band combinations requested for mandating no DL interruption

| Configuration | Uplink band | contact name, company | Contact email | other supporting companies (min. 3) |
| --- | --- | --- | --- | --- |
| CA_n1-n5-n78 | n1, n5, n78 | Bo Liu, China Telecom | liubo1@chinatelecom.cn | China Unicom, Huawei, HiSilicon, CATT |
| CA_n3-n5-n78 | n3, n5, n78 | Bo Liu, China Telecom | liubo1@chinatelecom.cn | China Unicom, Huawei, HiSilicon, CATT |
| CA_n1-n3-n5-n78 | n1, n3, n5, n78 | Bo Liu, China Telecom | liubo1@chinatelecom.cn | China Unicom, Huawei, HiSilicon, CATT |

We focus on architectures for and root cause on how the interruption happens. Our focus is especially the band combination 1+3 which are in the same frequency range. If one assumes that UE has five antennas in this frequency range and out of the five, two are TX capable antennas, this can be assumed as typical implementation currently and antennas cover from 1.7 GHz to 2.7 GHz. Requirement for bands n1 and n3 is to have 2 receivers and in some cases n1 is known implement also 4 receivers but in the context of this paper, we focus on five antennas and two receivers each band.

We do not need to look at the architecture any more detail to notice that for 2Tx-2Tx, there will be an interruption

Figure 1. Antenna assignments for 1Tx-1Tx and 2Tx-2Tx switch cases.

In Figure 1, 2Tx-2Tx switch requires reassignment of RX antennas and therefore performing a switch creates an interruption but 1Tx-1Tx may not need it.

The current “no DL interruption” is agnostic to switch case so only possibility to enable this design is to allow interruption.

Proposal:  For bands withing same frequency range using same antenna groups, DL interruption is always allowed for TX switching.

# Conclusion

We discussed especially the proposed 1+3 band combination need for DL interruption and made on proposal:

Proposal:  For bands withing same frequency range using same antenna groups, DL interruption is always allowed for TX switching.

# Reference

[1]	RP-241672, “New WID on Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink”, RAN vice-chair (China Telecom), 3GPP TSG RAN Meeting #104, Shanghai, China, 17th - 20th June, 2024