# 3GPP TSG-RAN WG4 Meeting #112	R4-2413516

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda item:			8.8.4

Source:	Samsung

Title:	Way Forward for [112][329] NTN_testing_NGSO_channel_model

Document for:	Approval

Introduction

This t-doc provides WF for [112][329] NTN_testing_NGSO_channel_model.

WF

Sub-topic 1-1 General

Issue 1-1-1: Work plan on channel model and requirements

- Agreement

  - Focus on TE-emulated channel model in initial stage for NGSO testing

    - Once sufficient progress made for channel model introduction, RAN4 can start the discussion on corresponding requirements and tests according to WI objective.

Issue 1-1-2: Relation-ship between TE-emulated channel model for satellite motion and fading channel model

- Agreement:

  - Decouple TE-emulated time-varying Doppler (frequency) and delay shifts due to satellite mobility from traditional fading channel

Sub-topic 1-2 TE emulated channel model for statellite mobility

Issue 1-2-1 Methodology for Time varying Doppler and Delay shifts modelling

- Agreement

- RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options:

  - Option 1: Eckstein-Hechler model

  - Option 2: Keplerian model

  - Option 3: Existing model from TR 38.811 5.3.4.3

  - Option 4: Proposal 2 from R4-2412552

dht=modvNGSO⋅t, 2dh,max-dh,max Tt=dtc=dh(t)2+hNGSO2c Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2 where - Tt: Propagation delay of the signal from the satellite at time t, - F(t): Doppler (frequency) shift of the signal from the satellite at time t. - dh(t): the horizontal distance between the satellite and UE at time t, - d(t): the distance between the satellite and UE at time t, - dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E), - c: the speed of light, - fd: Maximum Doppler shift (Hz)

  - Option 5: Proposal 2 from R4-2412783

- The instant satellite speed v is given as: v=μR+h - The initial angle β of vector geocentric to UE relative to the x axis is given as: β=π-α-arcsin⁡R∙cosαR+h - The satellite position s is given as: s=j∙R+h - The UE position u is given as: u=R∙ej∙β+v∙tR+h - The Doppler fd is given as: fd=vc∙fc∙realu-su-s - The delay T is given as: T=¦u-s¦c where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.

  - Other options not precluded

Note: Companies are encouraged to bring more information for option 1 and option2.

- Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model

  - FFS on the initial ephemeris information and parameters

Issue 1-2-2 Parameters for TE-emulated channel model

- Agreement

- RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose:

  - Satellite altitude:

    - Option 1: LEO -600km only

  - Option 2: Both LEO-600km and LEO-1200km Elevation angle range

  - UE position

  - Whether to consider a non-zero time-varying feeder link delay

- Other parameters not precluded

Issue 1-2-3 Test procedure/configuration

- Agreement

  - During test, ephemeris information shall be updated periodically and aligned with TE-emulated channel model which match with satellite motion trajectory

Sub-topic 1-3 RAN4 requirements and tests

Issue 1-3-2 demodulation requirements

- Agreement

  - RAN4 further discuss how to apply the varying Doppler and delay shifts to the UE performance requirements and necessary simulation assumption on UE side pending on channel model discussion progress.

    - UE assumption on propagator model for satellite motion for RAN4 simulation purpose

      - Option 1: Keplerian model

      - other options not precluded

  - RAN4 further discuss test applicability if new PDSCH demodulation requirements introduced pending on channel model discussion progress.

- Reference

[1] R4-2413429, “Topic summary for [112][329] NTN_testing_NGSO_channel_model “, Moderator (Samsung)