# 3GPP TSG-RAN WG4 Meeting #112                                                                R4-2414426

# Maastricht , Netherlands, 19th – 23th August, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1282 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (DC_R18_2BLTE_1BNR_3DL2UL) CR to TS 38.101-3 Rel18 Removal of Unnecessary NE-DC Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_xBLTE_yBNR_zDL2UL-Core |  |  |  |  |  |  | Date: |  | 2024-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Despite specification in 4.2 which indicates that NE-DC requirements are similar to EN-DC (unless specified otherwise) some unnecessary requirements have been specified. With this CR we plan to remove them and replace some of them with general statements. Some editorial corrections are performed, too. NOTE: In the last meeting the CR R4-2410647 was agreed in RAN4, however it was rejected in RAN-P because of a typo in the cover page |  |  |  |  |  |  |  |  |
| Summary of change: |  | clean up of the unnecessary NE-DC requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | unnecessary NE-DC requirements will remain specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6,7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | sections 6.1 and 7.1 were removed to avoid repetition since 4.2 included the necessary requirement |
| --- | --- |

## << Start changes >>

#### 6.2B.1.4a	(Void)

## << Next changes >>

##### 6.2B.2.1a	(Void)

## << Next changes >>

#### 6.2B.2.4a	(Void)

## << Next changes >>

#### 6.2B.3.3A	Inter-band NE-DC within FR1

For inter-band NE-DC between E-UTRA and FR1 NR, UE additional maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

Unless specified in Table 6.2B.3.3A-1, for inter-band carrier aggregation with uplink assigned to LTE and NR bands, the requirements in [2] clause 6.2.3 apply for NR uplink component carrier and the requirements in [4] clause 6.2.4 apply for LTE uplink component carrier.

Unless otherwise stated, for inter-band NE-DC with uplink assigned to NR and LTE bands and specified in Table 6.2B.3.3A-1, the combined requirements and allowed A-MPR are applicable on both LTE and NR bands when LTE and NR component carriers are active. The requirements in Table 6.2B.3.3A-1 are specified in terms of an additional spectrum emission requirement. The emission requirements specified in Table 6.2B.3.3A-1 also apply for the frequency ranges that are less than FOOB (MHz) from the edge of the channel bandwidth specified in TS 36.101 [4] and TS 38.101-1 [2], respectively.

Table 6.2B.3.3A-1: Additional Requirements for inter-band NE-DC (two-bands)

| NR CA combination | Band | Applied NS | Requirements (clause) (TS 36.101 [4]) | Requirements (clause) (TS 38.101-1 [2]) | A-MPR (table/clause) (TS 36.101 [4]) | A-MPR (table/clause) (TS 38.101-1 [2]) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n1_28 | n1 | 05 | N/A | 6.5.3.3.4 | N/A | Clause 6.2.3.4 | 1, 2 |
|  |  | 05U | N/A | 6.5.3.3.4, 6.5.2.4.2 | N/A | Clause 6.2.3.4 |  |
|  | 28 | 17 | 6.6.3.3.10 | N/A | Table 6.2.4-1 (NS_17) | N/A |  |
| DC_n78_1 | 1 | 05 | 6.6.3.3.1 | N/A | Table 6.2.4-1 (NS_05) | N/A |  |
| NOTE 1: NS_05U can be signalled for NR bands that have UTRA services deployed and protected range is specified in clause 6.5.2.4.2 of TS38.101-1[2] and the requirements in clause 6.5.2.4.2 are only appliable to the signalling band. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |  |  |  |  |

## << Next changes >>

#### 6.2B.3.4A	(Void)

## << Next changes >>

##### 6.2B.4.1.4a	(Void)

## << Next changes >>

##### 6.2B.4.2.1a	(Void)

## << Next changes >>

##### 6.2B.4.2.4a	(Void)

## << Next changes >>

### 6.3B.1a	(Void)

## << Next changes >>

### 6.3B.2a	(Void)

## << Next changes >>

### 6.3B.3a	(Void)

## << Next changes >>

#### 6.4B.1.4a	(Void)

## << Next changes >>

##### 6.4B.2.1a.3	In-band emissions

For the MCG the in-band emission requirements in Table 6.4.2.3-1 in TS 38.101-1 [2] are applied within the aggregated transmission bandwidth configuration of the NE-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the MCG at the edge of the aggregated transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] are applied, within the aggregated transmission bandwidth configuration of the NE-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the SCG at the edge of the said aggregated transmission bandwidth configuration.

## << Next changes >>

#### 6.4B.2.4a	(Void)

(Void)

## << Next changes >>

#### 6.5B.1.3a	(Void)

## << Next changes >>

#### 6.5B.1.4a	(Void)

## << Next changes >>

#### 6.5B.2.3a	(Void)

.

## << Next changes >>

##### 6.5B.3.3a.1	General spurious emissions

##### (Void)

## << Next changes >>

#### 6.5B.3.4a	(Void)

##### 6.5B.3.4a.1	(Void)

## << Next changes >>

#### 6.5B.4.4a	(Void)

## << Next changes >>

#### 6.5B.5.1a	(Void)

## << Next changes >>

#### 6.5B.5.3a	(Void)

## << Next changes >>

#### 6.5B.5.4a	(Void)

## << Next changes >>

### 6.6B.4a	(Void)

## << Next changes >>

#### 7.3B.2.1a	(Void)

## << Next changes >>

##### 7.3B.2.3a.4	MSD for intermodulation interference due to dual uplink operation for NE-DC in NR FR1

## << Next changes >>

#### 7.3B.3.4a	(Void)

## << Next changes >>

### 7.4B.1a	(Void)

## << Next changes >>

### 7.4B.3a	(Void)

## << Next changes >>

### 7.4B.4a	(Void)

## << Next changes >>

### 7.5B.1a	(Void)

## << Next changes >>

### 7.5B.3a	(Void)

## << Next changes >>

### 7.5B.4a	(Void)

## << Next changes >>

#### 7.6B.2.1a	(Void)

## << Next changes >>

#### 7.6B.2.3a	(Void)

## << Next changes >>

#### 7.6B.2.4a	(Void)

## << Next changes >>

#### 7.6B.3.1a	(Void)

## << Next changes >>

#### 7.6B.3.3a	(Void)

## << Next changes >>

#### 7.6B.3.4a	(Void)

## << Next changes >>

#### 7.6B.4.1a	(Void)

## << Next changes >>

#### 7.6B.4.3a	(Void)

## << Next changes >>

#### 7.6B.4.4a	(Void)

## << Next changes >>

### 7.7B.1a	(Void)

## << Next changes >>

### 7.7B.3a	(Void)

## << Next changes >>

### 7.7B.4a	(Void)

## << Next changes >>

#### 7.8B.2.1a	(Void)

## << Next changes >>

#### 7.8B.2.3a	(Void)

## << Next changes >>

#### 7.8B.2.4a	(Void)

## << Next changes >>

### 7.9B.1a	(Void)

## << Next changes >>

### 7.9B.3a	(Void)

## << Next changes >>

### 7.9B.4a	(Void)

## << End changes >>