Source:	Beijing Xiaomi Mobile Software Co., Ltd

Title:	Proposed updates for chapter 5 and 7 in TR 26.933

Document for:	Discussion &Agreement

Agenda Item:	7.8

# Introduction

This proposal is aimed to update the remaining parts in clauses 5 and 7 of TR 26.933[1].

# Content

## update of 5.1.1.6 Traditional approaches used in immersive audio

First change

#### 5.1.1.6.2.3 Binaural acoustic stimulation

According to the principle of binaural signal, the typical solution involves placing microphones on each ear of the user or on a model of a human head or ears to capture the binaural cues. This model could be a head with a torso to simulate all influences, including those from the ear, head and reflections from the torso. Alternatively, it could be a single head or simply a model with a pair of ears.

#### 5.1.1.6.3  The challenge of immersive audio on UE

The previous technology primarily relied on hardware solutions, which resulted in larger sizes and higher costs, making it less appealing in today's economic portable UE market. Therefore, it may be more feasible to consider software solutions, utilizing specific algorithms to achieve immersive audio.

End of First change

## remove 5.1.1.6, 5.2, 5.3 and 5.4

MEMS microphones, particularly digital MEMS microphones, are commonly used in current UE. These microphones can directly output digital signals and integrate various components such as preamps, ADC, and clock with the transducer in a single package. So it’s reasonable to remove 5.2 preamps, 5.3 DC and 5.4 clock. Since no other microphones are commonly used in the current UE, it is also proposed to remove the 5.1.1.6 other microphone.

Second change

End of Second change

## update of 7.1 AEC

Third change

### 7.1.4 AEC for different UE

Theoretically, increasing the distance between the microphone and loudspeakers can enhance AEC performance by reducing the echo level coupling from the loudspeakers to the microphones. However, the demand for additional microphones and loudspeakers due to immersive audio brings the distance significantly closer.

This reduction is especially evident in smaller UE, such as mobile phones and earbuds, where the compact size restricts the distance, leading to a higher echo level and shorter echo delay. In some devices, the microphone and loudspeaker may even be placed in the same sound port.

Conversely, larger UE like cars may experience a longer echo delay, but they offer a more stable acoustic environment since all the microphones and speakers for communication are installed within the cabin.

End of Third change

# Conclusion

Above three changes are proposed to update the corresponding parts in TR26.933.

## References

- S4-240764_[FS_DaCED] TR 26.933 v0.6.0