Source:	Deutsche Telekom, Fraunhofer HHI, InterDigital, Nokia Corporation, Philips, Sony

Title:	[FS_Beyond2D] Scenario UE-to-UE Beyond 2D Video Communication

Agenda Item:		9.9

Document for:		Discussion and Agreement

# Introduction

A new study item FS_Beyond2D (SP-240479) was approved at SA#103. One of the objectives of the study is:

2.	Establish and document a set of beyond 2D video end-to-end reference scenarios, including real-time communication, streaming services, split rendering, and messaging and corresponding workflows (capturing, encoding, packaging, delivery, decoding, rendering, including general constraints on latency, as well as complexity) to support 3GPP network related delivery and devices leveraging the generation or display technologies. This includes identifying and defining relevant beyond 2D formats in the context of above workflows, and representation technologies to support delivery of these formats within 3GPP networks.

In this contribution, a draft scenario on UE-to-UE Beyond 2D Video communication is proposed for incorporation into TR 26.956 as basis for future work. The scenario is structured according to the template provided in TR 26.065 v0.0.1 (S4-240825).

========================= CHANGE 1 (all new) ==========================

## 6.x	Scenario #x: Scenario UE-to-UE Beyond 2D Video Communication

- Scenario name

Scenario UE-to-UE Beyond 2D Video Communication

- Motivation for the scenario

What is the market relevance of the proposed scenario within the next few years? Are there any commercially available or pre-released products or prototypes?‘

The proposed scenario handles the UE-to-UE Beyond 2D Video communication that provides experiences beyond what is achievable with 2D content.

“Beyond 2D” content may be in the form of volumetric video, which is a frame-based immersive experience whereby each frame represents a volumetric region in 3D space in which any point is either non-occupied or having a color that may depend on the viewing direction. Volumetric video has the potential to provide a more immersive and interactive experience for use cases in diverse domains such as e.g. education, entertainment, and industrial monitoring.

This scenario considers Beyond 2D Video communication which has many applications. Having lifelike communication between UE's will reduce the need for travel and contribute to sustainability goals.

- Description of the scenario

This provides a description of beyond 2D video end-to-end workflows, which includes identifying and defining beyond 2D formats being used in the context and representation technologies to delivery these formats. The following aspects may be considered for each workflow:

This scenario considers UE-to-UE format exchange (conversational). Typically, the exchange will be symmetrical and both UEs will capture and render. Storage and delayed playback on the same UE will also be possible.

In this scenario (Figure 1), a first UE captures beyond 2D video and transmits to a second UE that renders one or two viewports of the transmitted scene.

Figure 1: Direct UE->UE communication

[Ed.(BK): The figure will be redrawn if needed for copyright reasons.]

- Capturing and processing

Capture capabilities of UEs will vary and are typically constrained by form factor. A mobile phone has multiple closely packed cameras and may also have a depth sensor. Other devices like computer monitors or camera bars are larger and can have more cameras with more separation. Mobile phones capable of acquiring two views with or without depth are the focus of this scenario. Study of other form factors and other view counts are optional.

The beyond 2D video is captured and processed using one or more cameras. Zero or more of those cameras may be range-sensing cameras, and one or more of these cameras have colour sensors. In the case of two or more cameras that are not rigidly connected, camera extrinsics are online calibrated. For mobile phones cameras are rigid, and both offline and online calibration may be considered.

Captured content is converted in real-time to a representation format that is suitable for encoding. The parameters of the source cameras may or may not be part of the representation format. At a minimum, inter-view consistent depth information is estimated, but more processes like reprojection, pruning, refinement, meshing and texturing steps may be needed depending on the representation format and application-specific constraints.

- Encoding

One codec that may be considered is MV-HEVC, which is the multiview extension of HEVC.

Another codec that can be used to realize this scenario is MPEG Immersive Video (MIV) specified in ISO/IEC 23090-12.

In addition to MV-HEVC and MIV, other codecs may be studied as part of this scenario.

The multiple camera views and depth maps if available are encoded to create a unified representation. An example could be MIV constrained to one atlas and packed video data. The single video sub-bitstream would be encoded with HEVC Main10 profile. The bitstream would contain all camera parameters that are necessary for 6DoF rendering.

- Packaging and delivery

The encoded bitstream is packed to RTP payload format according to the used codec.

For example, if MIV codec is used, MIV bitstream may be packaged as a sequence of V3C units including V3C units for the packed video data, or the MIV bitstream can be packaged as three separate tracks, one for common atlas data, atlas data, and packed video data.

IETF proposal avtcore-rtp-v3c enables carriage of MIV as a RTP stream. In this proposal at least the VPS and possibly also atlas data is carried over SDP. Integration in WebRTC is possible.

- Decoding

The decoder(s) will make use of hardware video decoder capabilities for pixel data, and a  metadata describing information needed for rendering is decoded/parsed by a CPU.

- *Post-processing

- Rendering

Rendering is typically performed on a GPU without dedicated hardware.

Rendering can be on:

- a device for 2D presentation (fixed viewpoint) such as a phone,

- a device for 3D presentation (multiple viewpoints) such as an autostereoscopic display,

- a device for 6DoF presentation (dynamic viewports) such as an HMD or an autosteoscopic display with eye tracking.

When a viewing space is used, then:

- What is rendered is one or two viewports with perspective projection and with 6 degrees of freedom (3-D position and 3-D orientation).

- The pose of the viewport is within a viewing space that can be signalled or implicitly determined from a decoded frame. A viewing space can limit both position, orientation or both in combination. For instance, it is generally not intended for a viewport to intersect with scene elements.

- When a viewport would be is rendered that is outside of the viewing space, then the renderer has to perform a mitigation to avoid a viewing experience that is not intended by the content provider.

- General constraints on latency, bandwidth, reliability and complexity

The codec should support low-delay reference frame structure.

For example, the MIV access units and the video sub-bitstream can be organized using a low-delay reference frame structure. All sub-bitstreams could have the same GOP structure, but atlas data and common atlas data frames may be skipped.

The common atlas data with camera parameters will only change infrequently (once per second or less), for instance each time an online camera calibration is updated. While it is possible to transmit common atlas data at non-IRAP frames, this is not expected in this scenario.

The atlas data with patch information may be static when transmitting only full views or dynamic when an encoder selects regions of the source views for transmission based on e.g. occlusion detection or depth segmentation. In MIV, patch information is always intra-coded (unlike V3C in general).

All encoder, decoder and renderer processes are real-time and may have a latency in the order of at most a few frames.

- Supporting companies and 3GPP members

- This documents the 3GPP members that support this scenario in terms of providing the information, test material, test requirements and the characterization for the tests. For each of the identified necessities, a tick box is created in the template.

- Preferably several 3GPP members are included in the support, and in addition a video service provider may be included (not necessarily a 3GPP member).

- Cross-verification is preferably done by the supporters of the scenario

Deutsche Telekom, Fraunhofer HHI, InterDigital, Nokia Corporation, Philips, Sony

- Source format properties

This defines a clear range of the considered and relevant source formats, including the signal properties, but also the characteristics of the content. As an example, the texture and depth format properties of the source may be used which include:

The source format has 2 views with perspective projection (PSP). Study of 1, 3 or 4 views is optional.

Each view has the following components:

- Required: Texture (color)

- Optional: Depth coded as normalized disparity

- Optional: Object ID map with ordinal values, 0 indicating "invalid pixel" (to support depth cameras), 1 indicating "background" and all other values indicating "foreground object i".

Views may be undistorted, otherwise distortion parameters have to be provided.

- Spatial resolutions

Each component of each view is 1920 × 1080, 1080 × 1080, 1440 × 1440, or 2048 × 2048.

- Chroma Format

The texture components are YCbCr.

All other components can be luma only, or YCbCr with chroma planes set to neutral gray.

- Chroma Subsampling

The texture components are 4:2:0.

All other components can be 4:0:0, or 4:2:0 with chroma planes set to neutral gray.

- Aspect ratios

The pixel aspect ratio of all video components is 1:1.

- Frame rates

The source frame rate is 30, 50 or 60 fps.

Note that the video frame rate may be different from the rendering frame rate, especially when the viewport pose is dynamic.

- Colour space formats

All texture components will use the ITU-R BT.709 colour space. The colour space format of other components is undefined because the chroma planes are not used.

- Transfer Characteristics

All texture components will use the ITU-R BT.709 transfer characteristics with limited range. The transfer characteristic of other components is linear with full rnage.

- Bit depth

The source texture components will be 8 or 10 bit.

The source depth components (if any) will be in between 8 and 16 bit.

The source object ID maps (if any) will be 16 bit.

- Viewpoints

The viewpoints are within a viewing space that can be provided as metadata or implicitly derived from the parameters of the set of source views.

Other signal properties

- Encoding and decoding constraints and settings

Typical encoding constraints and settings such as

- Relevant Codec and Codec Profile/Levels according to TS26.119

- Random access frequency

- Error resiliency requirements

- Bitrates and quality requirements

- Bitrate parameters (CBR, VBR, CAE, HRD parameters)

- ABR encoding requirements (switching frequency, etc.)

- Latency requirements and specific encoding settings

- Encoding context: real-time encoding, on device encoding, cloud-based encoding, offline encoding, etc.

- Required decoding capabilities

- Synchronization requirements

TBD

- Performance Metrics and Requirements

- A clear definition on how the performance needs to be evaluated including metrics, etc addressing the main KPIs of the scenario.

[Ed.(BK): To be aligned with the agreed evaluation framework.]

The tests are run for a chosen level as described in 5 a. Bitstreams are provided. Camera calibration, depth estimation, and encoding are not evaluated.

The test will have four rate points and QP values are selected for each sequence to approximately match the 5 to 50 Mbps range. When saturation occurs before 50 Mbps a lower value may be chosen in consultation. When there are multiple video components or packed regions then the other QP values need to be directly derived from the texture QP using an equation or look-up table. (They cannot depend on the sequence.)

- Objective measures such as PSNR, VMAF, etc, may be used.

[Ed.(BK): To be aligned with the agreed evaluation framework.]

The IV-PSNR tool, available at https://gitlab.com/mpeg-i-visual/ivpsnr, is available to compute full-reference objective metrics:

- Weighted sphere PSNR (WS-PSNR)

- Immersive video PSNR (IV-PSNR)

All source views that were used for encoding are provided. Each source view is reconstructed by decoding and rendering (view synthesis). The IV-PSNR tool is then run on all source views and the score is averaged over all views.

Depending on bit rate, quality of depth maps and rendering, either the video codec or view synthesis is the limiting factor. BD-PSNR is calculated for both metrics because the metric behaves more predictably than BD-rate.

- Justification on whether objective metrics are sufficient and representative of the subjective performance.

[Ed.(BK): To be aligned with the agreed evaluation framework. The discussion here on correlation of objective and subjective metrics may need to be moved to that framework after more deliberation.]

There is experience in testing of immersive video in MPEG context. The test conditions as described are a simplification and evolution of:

Dziembowski, B. Kroon, J. Jung (Eds.), Common test conditions for MPEG immersive video, ISO/IEC JTC 1/SC 29/WG 04 N 0372, July 2023, Geneva.

The main challenge with testing of Beyond 2D video is that codecs are asymmetric. The input is a number of source views (with depth maps), and the output of the decoder + renderer can be any viewport within a spatial region around those source views. In the mentioned CTC two tests are used:

- Objective evaluation at source view positions

- Subjective evaluation of pose trace videos (dynamic viewports)

This has resulted in a lack of correlation between objective and subjective results, but despite that it is the best-known approach. Alternatives that have been tried and dismissed (for now):

- Objective evaluation at dynamic viewports: It includes view synthesis in the reference condition and this skews the results towards a specific renderer. It prevents an A/B comparison of different renderers.

- Subjective evaluation at source view positions: This is not how the end-user will interact with the content, and it does not evaluate artifacts due to viewport dynamics.

For this test, because the aim is to prove feasibility of a scenario, objective evaluation may be sufficient, especially when supplemented with (informal) real-time demonstration of the same bitstreams that were used for objective evaluation.

- Interoperability Considerations for the application

- Streaming with DASH/HLS/CMAF/QUIC

- RTP based delivery

The Beyond 2D Video bitstream needs to be carried over RTP with SDP for this use case of this scenario. WebRTC may be considered. It is not necessary to prove this as part of the feasibility test, if written evidence can be provided.

In the example of using MIV as a codec, there are implementations for DASH (InterDigital/Philips) and RTP + SDP (Nokia).

- Test Sequences

A set of selected test sequences that are provided by the proponents in order to do the evaluation. 	They should cover a set of source format properties

Test sequences that were used during the development of a codec are discouraged because they may create a bias towards that specific codec. Sequences that were used in a verification test are permissible.

Preferably test sequences match with the intended use case both in terms of technical requirements and content semantics.

- Detailed test conditions

Provides a proposal for detailed test conditions, for example based on a reference software 	together with the sequences and configuration parameters.

[Ed.(BK): To be aligned with the agreed evaluation framework.]

For each candidate codec, a suitable decoder + renderer needs to be made available for testing purposes.

A reporting template or script will be provided to compute BD-PSNR based on IV-PNSR log files of all rates and sequences.

For MIV the following test conditions were followed:

- Dziembowski, B. Kroon, J. Jung (Eds.), Common test conditions for MPEG immersive video, ISO/IEC JTC 1/SC 29/WG 04 N 0372, July 2023, Geneva.

- External Performance data

References to external performance data that can be added, for example other SDOs, public 	documents and so on.

For MIV the following performance data is available:

- D. Mieloch (Ed.), Verification test report of MPEG immersive video, ISO/IEC JTC 1/SC 29/WG 04 N 0341, April 2023, Antalya.

NOTE: This performance data was based on different source view properties and the results may not translate to this study.

- Additional Information

Images were copied from S4-240831

https://datatracker.ietf.org/doc/draft-ietf-avtcore-rtp-v3c/