- 3GPP TSG SA WG5 Meeting #156	S5-244615d3

- Maastricht, The Netherlands 19 - 23 August 2024

- Source:	China mobile, ZTE Corporation, NEC

- Title:	Rel-19 pCR TR 28.858 Add Potential Solution for Fine-tuning

- Document for:	Approval

Agenda Item:	6.19.1

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.858: " Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2"

# 3	Rationale

This is to add the solution for ML fine-tuning.

# 4	Detailed proposal

1st Change

#### 5.1.X	ML Fine-tuning

#### 5.1.x.4	Possible solutions

##### 5.1.x.4.1	Possible solution

The exsting MLModel IOC and MLTrainingRequest IOC can be enhanced to support fine-tuning.

Enhancement Aspects on MLTrainingRequest IOC:  Since the exsting MLTrainingRequest IOC cannot support consumer requested ML fine-tuning, both following enhancements are possible.

- FineTuningIndication, indicates the consumer requested ML fine-tuning. The support qualifier should be CM, which means this attribute should be present when the MLTrainingRequest MOI represents the request for ML Model Fine-tuning. Besides, change the support qualifier of “aIMLInferenceName” to CM, which means that this attribute should be present when the MLTrainingRequest MOI does not represent the request for ML Model Fine-tuning.

- Pre-TrainingModelRef, indicates the DN of the pre-training model that will be used to fine-tuning

Enhancement Aspects on MLModel IOC:

Since fine-tuning is aiming at a specific inference type for a pre-trained model, the AIML capabilities would be narrowed  which has the best performance. It is proposed to introduce following attributes:

- FinetuningModelIndication, indicates that the ML Model is a Finetuning Model. Besides, change the support qualifier of “aIMLInferenceName” to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

- PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

End of changes