3GPP TSG-SA5 Meeting #156 	S5-244633

Maastricht, Netherlands, 19 – 23 August 2024

- Source:	Nokia

- Title:	Rel-19 pCR Evaluation and conclusion of enhancement of failure prediction analytics

- Document for:	Approval

Agenda Item:	6.19.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

# 3	Rationale

This pCR proposes the evaluation of the existing solution for enhancement of failure prediction use case.

# 4	Detailed proposal

Start of first change

### 5.7.2	Use case 2: Enhancing failure prediction

#### 5.7.2.1	Description

This use case is to enhance the existing MDA capability for failure prediction. The enhancements to the existing use case on failure prediction is targeted towards effective correlation of the predicted alarms and the actual alarms which has resulted as predicted.

The existing use case of failure prediction provides a prediction of failure with some additional information, see TS 28.104 [x]. This includes information such as predicted time of failure occurrence, perceived severity, and the recommended actions. These additional information helps the consumer to understand the effect of the failure in the network to a basic extent. However, the current solution does not provide additional information (e.g. predicted failure recovery time or the trend of the problem causing the failure) which may enable the consumer to anticipate the impacts to the network more effectively.

The predicted failure recovery time indicates the time by which the target entity recovers from the predicted failure. For instance, MDA can predict the time to recover for a given failure given the predicted root cause of the predicted failure and the history information.

The attribute “trend” indicates the trend of the failure, whether the failure will intensify or weaken in the future. In case of periodic analytics reports, the trend information can provide information on how the identified issue might transform in the future. This is similar to the “trend” attribute in the alarm solution in 28.532 [3].

Also, for the consumers to effectively use the output of MDA assisted failure prediction, it is required by the consumer to seamlessly correlate the predicted failure and the actual failure when it occurs in the system. The alarm stage 2 solution is defined in 28.532 [3]. To utilize the failure prediction output effectively for this purpose, it is required to enhance the existing failure prediction to include additional information that may help in effective correlation. To achieve this, it is required to enhance the existing solution towards the alarm solution in 28.532 [3].

#### 5.7.2.2	Potential Requirements

REQ-FAILURE_PRED_ENHANCE_MDA-01: MDA capability for enhanced failure prediction should be able to provide the following information as part of the failure prediction analytics.

- a predictedRootCauseIndicator indicating the possible root cause of the predicted failure.

- Trend indication about the predicted failure in case of periodic reporting.

- Predicted end time of the failure

#### 5.7.2.3	Potential solutions

The analytics output for MDA assisted fault management may be enhanced with the following attributes.

-	predictedRootCauseIndicator. This attribute may indicate the predicted root cause of the potential failure that has been predicted. This attribute may be of type string.

-	predictedTrendIndication – this attribute may indicate the predicted trend of the failure that has been predicted. In case of periodic MDA outputs, this attribute may indicate the trend of the predicted failure is worsening or getting better or not changing. This attribute can be an enumerated value with possible allowed values of “less severe”, “no change” and “more severe”. The detailed definition of these enumerated values can be found in clause 8.1.2.6 of ITU-T Recommendation X.733 [11].

-	predictedRecoveryTime – this attribute may indicate the predicted recovery time of the predicted failure.

#### 5.7.2.4	Evaluation of solutions

The solution proposed in clause 5.7.2.3 satisfies the requirements and this solution is feasible for normative work.

Start of next change

# 6	Conclusions and Recommendations

## 6.x	Fault management related analytics and alarm prediction

The use case, requirements and solution for the capability of enhancing failure prediction is described in clause 5.7.2.3. It is recommended to extend the output of failure prediction as described in clause 5.7.2.3 in TS 28.104 [2].

End of changes