3GPP TSG-SA5 Meeting #156	S5-244702 (rev of S5-244168)

- Maastricht, Netherlands, 19 - 23 August 2024

- Source:	NTT DOCOMO, China Mobile

- Title:	pCR 28.869 VNF Config Manager solutions and evaluation

- Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

The group is asked to discuss and approve.

# 2	References

[1]	3GPP TR 28.869 v0.3.0 Study on cloud aspects of management and orchestration

# 3	Rationale

This contribution adds solutions and evaluation for Use case #1: Cloud-native VNF configuration management in clause 5.1.1.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                        ETSI GS NFV-IFA 049: “Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions specification.

…

[ref-eve022]        ETSI GR NFV-EVE 022: “Network Functions Virtualisation (NFV) Release 5; Architectural Framework; Report on VNF configuration”

2nd Change

### 5.1.1	Use case #1: Cloud-native VNF configuration management

#### 5.1.1.1	Description

Besides allocation and configuration of the necessary resources, to bring up a cloud native VNF into operation, i.e., full instance provisioning and configuration of the corresponding VNF instances needs to take place. Configuration actions can also be performed during the cloud native VNF instance operation.

An operator needs to be able to manage and orchestrate through the 3GPP management system the configuration of the cloud native VNF instances. The operator needs to be able to perform through the 3GPP management system, operations like set and query configuration information.

#### 5.1.1.2 	Potential requirements

REQ-CVNF_CM-1: The 3GPP management system should have the capability to configure cloud-native VNF instances.

REQ-CVNF_CM-x: The reference point between 3GPP management system and the platform providing VNF generic OAM and other PaaS Services should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of configuring cloud-native VNF instances.

REQ-CVNF_CM-y: The 3GPP management system should have the capability to provide configuration data to configuration files used for Day-0 configuration of a cloud-native VNF.

REQ-CVNF_CM-z: The 3GPP management system should have the capability to orchestrate the management of configuration data for cloud-native VNF instances according to the configuration scope and functionality offered by the platform providing VNF generic OAM and other PaaS Services.

#### 5.2.1.3 Potential solutions

##### 5.2.1.3.x	VNF Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of the VNF Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. Some key functionalities supported by the VNF Configuration Manager function are the capability to convey configuration information to one or more VNF/VNFC instances, the capability to perform pre-configuration actions (e.g. create configuration backup) and post-configuration actions (e.g. rollback running configuration) and the capability to query configuration information of VNF/VNFC instances.

Configuration information includes virtualization-dependent configurations and virtualization-independent configurations.

The VNF Configuration Manager does not understand the semantics of the configuration information that is conveyed to the VNF/VNFC instances.

Figure 5.2.1.3.x-1 depicts the interaction and reference point between 3GPP management system and the VNF Configuration Manager.

Figure 5.2.1.3.x-1: Interaction and reference point between 3GPP management system and VNF Configuration Manager

The solution enables the 3GPP management system having the capability to manage configuration for the cloud-native VNFs by interacting with the VNF Configuration Manager function.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-x.

##### 5.2.1.3.y	Network Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution considers the use of the Network Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. The Network Configuration Manager function can be used to set network configuration information related to one or more VNF/VNFC instances. For example, the Network Configuration Manager can be used to configure the VNF Connection Points (CPs), VNF networking for VNF instances in a service mesh, etc.

Figure 5.2.1.3.y-1 depicts the interaction and reference point between 3GPP management system and the Network Configuration Manager.

Figure 5.2.1.3.y-1: Interaction and reference point between 3GPP management system and Network Configuration Manager

The solution enables the 3GPP management system having the capability to manage configuration for the cloud-native VNFs by interacting with the Network Configuration Manager function.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-x.

##### 5.2.1.3.z	Configuration Server

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of a Configuration Server introduced in ETSI GR NFV-EVE 022 [ref-eve022]. The Configuration Server can be understood as a logically centralized storage for configuration. In addition, Configuration Server services can be used to store and fetch configuration data, convert configuration data formats, validate configuration data schema and apply version control for configuration data. The interface exposed and operations supported by the Configuration Server are defined in ETSI GS NFV-IFA 049 [2].

Figure 5.2.1.3.z-1 depicts the interaction and reference point between 3GPP management system and the Configuration Server.

Figure 5.2.1.3.z-1: Interaction and reference point between 3GPP management system and Configuration Server

As depicted in Fig 5.2.1.3.z-1, a consumer of the Configuration Server interfaces could be other VNF generic OAM function such as the VNF Configuration Manager defined in ETSI GS NFV-IFA 049 [2] or the 3GPP management system (an interacting MnF residing inside SBMA).

The solution enables the 3GPP management system having the capability to handle configuration data, like distributing, fetching and transforming configuration data, for one or multiple cloud-native VNFs.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-x.

##### 5.2.1.3.m	Day-0 Configuration for cloud-native VNFs

This solution is about Day-0 configuration for cloud-native VNFs. Typically, Day-0 configuration refers to configuration that needs to be applied before completing the instantiation process of the VNF (either VM-based or container-based). An example is the address of the VNF management entity (e.g., VNFM) since this is not typically provided in the VNFD and needs to be configured by other means. Typically, such configurable properties are declared in a configuration file inside the VNF Package.

For example, in the case of ETSI NFV for containerized cloud-native VNFs, configurable properties can be included in a configuration file in the Managed Container Infrastructure Object Package (MCIOP), whose actual values can be provided by another artifact file, while for VM-based solutions configuration data can be specified in the boot data attribute of the VNFD pointing to an initialization file contained in the VNF package.

Regarding the entity providing the actual data, different options regarding Day-0 configuration for cloud-native VNFs have been investigated in ETSI GR NFV-EVE 022 [ref-eve022]. For example, the VNFM performs the mapping between the configurable properties (in the VNFD) and the corresponding items in the configuration files. VNFM can receive the actual values from NFVO or the 3GPP management system during or before the instantiation request of the cloud-native VNF.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-y.

##### 5.2.1.4	Evaluation of solutions

Orchestration and management systems like NFV-MANO are used to hide the complexity through abstractions of the underlying infrastructure, regarding resource management and VNF LCM operations from OSS. Nevertheless, in cloud-native environments, operations related to cloud-native VNF configuration can be highly complex due to the different dimensions and scopes of configuration to be considered, such as virtualization-dependent configuration, virtualization-independent configuration, network connectivity configuration, etc.

In this regard the solutions presented (i.e., VNF Configuration Manager function, Network Configuration Manager, function, Configuration Server, Day-0 configuration) are not alternative to each other and can work in conjunction to support cloud-native VNF configuration. This implies that consumers of such functionalities, like the 3GPP management system, need to orchestrate the process of configuration for cloud-native VNF instances when leveraging such solutions.

EXAMPLE 1:	The VNF Configuration Manager function can store and retrieve configuration data to/from the Configuration Server.

EXAMPLE 2:	A Day-0 configuration mechanism is used to connect VNF to VNFM. After VNF instantiation, the Network Configuration Manager configures the VNF connection points (CPs) with the appropriate IP addresses.

End of Changes