3GPP TSG-SA5 Meeting #156 	S5-244705

Maastricht, Netherland, 19 - 23 August 2024

- Source:	Ericsson

- Title:	pCR TR 28.875 new solution mobile IAB node configuration update based on location

- Document for:	Approval

Agenda Item:	6.19.16

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.875 v0.2.0 Study on management of IAB nodes

# 3	Rationale

This contribution adds new solution to TR 28.875 [1] about mobile IAB node configuration update based on location.

# 4	Detailed proposal

1st Change

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[3]	3GPP TS 38.401: “NG-RAN; Architecture description”.

[4]	3GPP TS 28.315: “Management and orchestration; Plug and Connect; Procedure flows”.

[y]	3GPP TS 28.532: “Management and orchestration; Generic management services”

…

2nd Change

# 6	Potential solutions

### 6.x	Potential solutions #x Mobile IAB node configuration update based on location

Mobile IAB node may need new configuration while it moves, for example, new configuration of NR cells or IP configuration for OAM connectivity. This solution enables IAB node to update the configuration based on its location:

- 3GPP management system supports the mapping of configuration with locations of the IAB node. The location information can be one or more of location information such as TAC, TAI, PLMN info and gNBId of IAB-donor-CU depends on mobility scenarios.

- When IAB node powers up and connects to OAM3GPP management system for the 1st time, the IAB node can provide location information with IAB node indication via notifications to OAM3GPP management system as specified in TS 28.532 [y] (e.g., notifyMOIChanges) to indicate its current location. Upon the reception of notifications, 3GPP management system can provision the initial configuration (e.g.via createMOI operation) as specified TS 28.532 [y] based on its current location.

- When IAB node moves to a new location which may need new configuration, the IAB node can provide new location information via “notifyMOIChanges” or “notifyMOIAttributeValueChanges” or other notifications as specified in TS 28.532 [y] to 3GPP management system. Upon the reception of notifications of new location, 3GPP management system can provision the new configuration via “modifyMOIAttributes” or other operations as specified in TS 28.532 [y] to the IAB node based on its new location.

End of change