3GPP TSG-SA5 Meeting #156	S5-244809

Maastricht, Netherlands, 19 - 23 August 2024

- Source:	Nokia

- Title:	Rel-19 pCR 28.872 Improve description of Conditional activation of planned configurations

- Document for:	Approval

Agenda Item:	6.19.9 - Study on Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TR 28.872: "Study on Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TR 28.872[1].

Begin of modifications

## 5.6	Use case #6: Conditional activation of planned configurations

### 5.6.1	Description

A planned configuration may have to be activated automatically when certain conditions are met. For example, performance metric collection jobs or assurance functions shall be created when metrics that are always collected cross some thresholds.

### 5.6.2	Potential requirements

Req-1: The 3GPP management system should allow MnS consumers to create conditions on MnS producers that specify when planned configurations are activated.

### 5.6.3	Potential solutions

#### 5.6.3.1	Potential solution #1: Dedicated job

MnS consumers can specify conditions on MnS producers. Planned configurations are automatically activated when the conditions evaluate to true.

Conditions shall be specified using the exact same mechanism as the "ConditionMonitor", i.e. Jex for the HTTP/JSON solution and XPath for the NETCONF/YANG solution. Note though that the "ConditionMonitor" is designed to switch on and off a process: When the condition evaluates to false the process is (switched) off, when the condition evaluates to true, the process is (switched) on. For the present use case the condition needs to trigger an action. The triggering condition is therefore the transition of the condition evaluation result from false to true or true to false. The triggering condition needs to be specified together with the condition expression.

Furthermore, the case where the condition expression toggles between true and false needs consideration. For example, a condition may become inactive when it triggered once the activation of a plan. This work is left for the normative phase.

The information model for conditional plan activation is as follows:

conditional-plan-activation-jobs

<job-id>

request

plan-ids

here are the activation controls

condition-expression

conflict-mode

results

<plan-id>

state

time

error-details

The following table specifies the attributes of the validation job.

| Attribute name | Description |
| --- | --- |
| plan-ids | The identifiers of the plans to be activated. |
| condition-expression | The condition expression. |
| trigger-condition | The trigger condition, either "FALSE_TO_TRUE" or "TRUE_TO_FALSE". |

HTTP/JSON

The "configuration-conditions" collection resource is introduced. Conditions are specified using Jex. When the evaluation result of the condition transits from false to true the MnS producer shall automatically create a configuration activation job for the planned configurations specified by "plan-ids".

Example:

POST 3gpp/ProvMnS/1900/conditional-plan-activation-jobs HTTP/1.1 Host: example.org Content-Type: application/json { "plan-ids": ["p1", "p7"], "condition-expression": "some Jex expression", "trigger-condition": "FALSE_TO_TRUE" }

After applying the POST request, the resource structure on the MnS producer looks as follows.

{ "conditional-plan-activation-jobs": { "cpaj1": { "plan-ids": ["p1", "p7"], "condition-expression": "some Jex expression", "trigger-condition": "FALSE_TO_TRUE" } } }

### 5.5.4	Evaluation of potential solutions

It is recommended to select potential solution #1.

End of modifications