- 3GPP TSG-SA5 Meeting #156	S5-244247

- Maastricht, The Netherlands, 19-23 August 2024

- Source:	Nokia, Verizon

- Title:	Rel-19 pCR TR 28.858 Add solution for ML model confidence threshold

- Document for:	Approval

- Agenda Item:	6.19.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 				3GPP TR 28.858 “Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2”.

# 3	Rationale

This pCR is to add a solution for ML model confidence threshold use case that was agreed in SA5#155 meeting.

# 4	Detailed proposal

Start of modification

- 5	AI/ML management use cases and requirements

## 5.1	ML model training

### 5.1.5	ML model confidence

#### 5.1.5.1	Description

Confidence is a measure of the probability that a prediction made by the ML model is correct. It is often expressed as a percentage, with 100% being absolute certainty and 0% being absolute uncertainty. Confidence score can be useful in prediction-making processes, where a certain level of confidence might be required before taking action based on the ML model’s prediction. However, it is important to note that a high confidence score does not guarantee the the ML model’s prediction is correct, but it only indicates the ML model belief in its prediction. In simpler terms, confidence is a measure of the ML model’s certainty about its prediction. It is not a measure of the ML model’s accuracy or correctness. An ML model can be very confident about a prediction and still be wrong. Conversely, an ML model can be less confident about a prediction and still be correct.

#### 5.1.5.2		Use Cases

##### 5.1.5.2.1		Model Confidence Threshold in ML Training

In the ML training phase, the ML training function may support the capability to evaluate the confidence value of the trained ML model. The ML model confidence value indicates the average confidence value (in unit of percentage) that the ML model would perform for data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. Therefore, the MnS consumer may prefer to indicate the confidence threshold value that the trained ML model should satisfy before the ML model is deployed for inference.

#### 5.1.5.3			Potential requirements

REQ-MODEL_CONF-1: The AI/ML training MnS producer should have a capability allowing an authorized MnS consumer to indicate the confidence threshold that the AI/ML MnS producer should meet on the trained ML model.

### 5.1.5.4					Possible Solutions

Introduce a new attribute, e.g., modelConfidenceIndication, in the MLTrainingRequest IoC requested by an authorized MnS consumer to the ML Training MnS Producer. This attribute indicates to the ML Training MnS Producer the minimum average confidence value (in unit of percentage) that the ML model should meet on the data with the same distribution as training data. If the minimum average confidence value cannot be satisfied, then the ML Training MnS Producer should retrain the ML model. The attribute is of type real with allowed values {0…100}.

End of modifications