3GPP TSG-SA5 Meeting #156	S5-244898d1

Maastricht, Netherlands, 19 - 23 August 2024

- Source:	Huawei

- Title:	Potential solution of EE KPIs evaluated from network quality performance dimension

- Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 v0.3.0: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

This contribution proposes to add potential solution of EE KPIs evaluated from network quality performance dimension in TR 28.880 [1].

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

### 5.6.3	Potential solutions

#### 5.6.3.j	Potential solution #<j>: <EE KPIs evaluated from network quality performance dimension>

##### 5.6.3.j.1	Introduction

In this potential solution, EE KPIs are considered from network quality performance dimension.

##### 5.6.3.j.2	Description

Network quality, measured from the following performance measurements point of view

- DL RAN UE throughput

- UL RAN UE throughput

The UE perceived throughput in NG-RAN is an important performance parameter for operating 5G network. If the UE throughput of the NR cell cannot meet the performance requirement, some actions need to be performed to the network, e.g. reconfiguration or capacity increase. So it is necessary to define UE throughput KPI to evaluate whether the end-users are satisfied. See clause A.7 of TS 28.554 [2].

The proposed performance dimension of EE KPIs, which are evaluated from network quality performance dimension as reflected by some existing KPIs in TS 28.554 [2], are captured in Table 5.6.3.j.2-1.

Table 5.6.3.j.2-1: EE KPIs evaluated from network quality performance dimension

| Performance dimension to evaluate EE KPIs | Performance part of EE KPIs | Reference in TS 28.554 [2] | Note |
| --- | --- | --- | --- |
| User throughput | DL RAN UE throughput for a NRCellDU | 6.3.6.3.1 | This KPI describes the average DL RAN UE throughput for a NRCellDU. |
|  | DL RAN UE throughput for a sub-network | 6.3.6.3.2 | This KPI describes the average DL RAN UE throughput for a sub-network. |
|  | DL RAN UE throughput for a network slice subnet | 6.3.6.3.3 | This KPI describes the average DL RAN UE throughput for a network slice subnet. |
|  | UL RAN UE throughput for a NRCellDU | 6.3.6.4.1 | This KPI describes the average UL RAN UE throughput for a NRCellDU. |
|  | UL RAN UE throughput for a sub-network | 6.3.6.4.2 | This KPI describes the average UL RAN UE throughput for a sub-network. |
|  | UL RAN UE throughput for a network slice subnet | 6.3.6.4.3 | This KPI describes the average UL RAN UE throughput for a network slice subnet. |

The EE KPIs evaluated from network quality and data traffic performance dimensions are captured in Table 5.6.3.j.2-2.

Table 5.6.3.j.2-2: EE KPIs evaluated from network quality and data traffic performance dimensions

| Performance dimensions to evaluate EE KPIs | EE KPIs | Reference in TS 28.554 [2] | Note |
| --- | --- | --- | --- |
| Latency as the network quality performance indicator and Data Volume (DV) as the network data traffic performance indicator in combined dimensions mode | Energy efficiency of URLLC network slice (Based on both latency and Data Volume (DV) of the network slice) | 6.7.2.3.3 | A KPI that shows the energy efficiency of network slices of type URLLC. |

End of change