3GPP TSG-SA WG6 Meeting #62	S6-243141

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on update to solution #10

Spec:	3GPP TR 23.700-72 V1.1.1

Agenda item:	8.1

Document for:	Approval

Contact:	Sapan Shah, sapan.shah@samsung.com

1. Introduction

This pCR updates solution #10.

2. Reason for Change

Solution #10 (before enhancements done in previous meeting):

- When UE1 and UE2 shares the location, both UEs need to send registration (or registration update) message to indicate LMS that both UEs allowed to share the same location report and they are currently at same location.

  - Similarly, registration update is required when both UEs goes away from each other

- The solution is not clear enough and not covering all scenarios

  - LMS is dependent on both users to update the registration.

  - If both UEs came near by and only one UE update the registration then how SEAL LMS will behave is not clear.

  - If both UEs has updated registration to use same location, later both UEs goes apart from each other. What if only one UE updates the registration?

  - Most importantly, the solution is limited to single user only.

Solution #10 (after enhancements done in previous meeting):

- Registration (or registration update) message is send by both UEs to indicate LMS that both UEs are allowed to share same location report.

- Now, when both UEs come within proximity of each other, any one UE can enable sharing same location report. Avoding dependency on other UE also to enable the location sharing.

  - Similarly, when both UEs goes away any one UE can disable the location sharing.

- It also covers case when UEs belong to different users too.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72 V1.1.1.

* * * First Change * * * *

#### 6.10.1.2	Procedure of LMS identifying multiple UEs that sharing the same location data

The procedure is applicable to the UEs belonging to same user or different users.

Pre-condition:

1)	SEAL LMC-1 (in UE-1) has offline configured SEAL LMC-2 (in UE-2) to receive location reports.

NOTE 1:	Both UE-1 and UE-2 are connected to the access network and able to send individual location report to LMS. The offline configuration is used here to determine whether two UEs are within proximity of each other or not.

Figure 6.10.1.2-1: Procedure of LMS identifying multiple UEs that sharing the same location data

1.	Similar with the procedure of clause 9.3.15 of 3GPP TS 23.434 [9], the LMC1 sends a location service registration request to the LMS including the identifier of the UE, the UE-based location capabilities, the associated ID with other UEs (e.g. group ID) , the type of UE (e.g. IoT), the sharing location indicator,etc. The sharing location indicator indicates the associated UEs may share the same location.

NOTE 1:	The value of sharing location indicator may be decided by the user of VAL UEs or via VAL UE's reporting based on some positioning capabilities (e.g., Prose). It's up to the UE's implementation.

2.	The LMS checks authorization for the VAL UE's location service registration request and sends location service registration response to the LMC1 if the authorization is approved.

3.	Same with Step 1, the LMC2 sends a location service registration request to the LMS with the UE ID, the associated ID with other UEs (e.g. group ID), the type of UE, the sharing location indicator, etc.

NOTE 2:	The associated ID for LMC1 and LMC2 is the same.

4.	The LMS sends location service registration response to the LMC2.

5.	The SEAL LMC-1 (in UE-1) receives off-network location report from the SEAL LMC-2 (in UE-2). Based on location report, if UE-2 is within certain range of UE-1, the SEAL LMC-1 determines that the UE-2 (where SEAL LMC-2 resides and shared the off-network location report) is within allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are close enough and so UE-1's location can be used by SEAL LMS instead of UE-2's location).

NOTE 3:	The SEAL LMS can initiate verification to confirm whether both UE-1 and UE-2 are at same location or not. And it can be considered during normative work.

6.	The SEAL LMC-1 (in UE-1) receives off-network location report from the SEAL LMC-2 (in UE-2). Based on location report, if UE-2 is within certain range of UE-1, the SEAL LMC-1 determines that the UE-2 (where SEAL LMC-2 resides and shared the off-network location report) is within allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are close enough and so UE-1's location can be used by SEAL LMS instead of UE-2's location).

7.	The SEAL LMS authenticates and authorizes the SEAL LMC-1. If authorized, the LMS identifies that LMC1 and LMC2 are associated (e.g. same group ID) based on the received registration information of LMC1 and LMC2, and if during the registration process, the sharing location indicator is set to true, the LMS further determines that the two UEs share the same location data. The LMS enables reuse of location of UE-1 for UE-2.

8.	The SEAL LMS sends enable location reuse response to SEAL LMC-1.

9.	Similar with the procedure of 9.3.4 of 3GPP TS 23.434 [9], the VAL server 1 sends a location information request to the LMS to request the location data for the UE1 including the UE ID and the optional associated ID with others (e.g. group ID).

10.	The LMS sends a location information request to the LMC1.

11.	The LMC1 responds to the LMS with a location report (e.g. report 1).

12.	The LMS stores or updates the received location report for the LMC1.

13.	The LMS sends the VAL UE1's location report (i.e. report 1) to the VAL Server 1.

14.	Same with Step 9, the VAL server 2 sends a location information request to the LMS to request the location data for the UE2 including the UE ID and the optional associated ID with others (e.g. group ID).

NOTE 4:	The VAL server 1 and the VAL server 2 may be the same.

15.	As the LMS has identified that LMC1 and LMC2 are associated and share the same location data in the Step 5, the LMS determines that the reuse of location of UE-1 for UE-2 is enabled and so the LMS determines to send the stored location report of UE1 (i.e. report 1) to the VAL server 2 instead of obtaining the VAL UE 2's location data. But if the LMS didn’t store the VAL UE1's location data or the stored location data is invalid or the value of sharing location indicator is not true, the LMS will ask the VAL UE2 to report the UE location data directly.

16.	The SEAL LMC-1 (in UE-1) receives off-network location report from the SEAL LMC-2 (in UE-2). Based on location report, if UE-2 is out of certain range of UE-1, the SEAL LMC-1 determines that the UE-2 (where SEAL LMC-2 resides and shared the off-network location report) is outside allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are not close enough and so UE-1's location can not be used by SEAL LMS instead of UE-2's location).

17.	The SEAL LMC-1 sends request message to SEAL LMS to disable reuse of location of UE-1 for UE-2. The request message includes user's identity, security parameters, UE-1 identity, UE-2 identity and latest location report of UE-1.

18.	The SEAL LMS authenticates and authorizes the SEAL LMC-1. If authorized, the LMS disables reuse of location of UE-1 for UE-2.

19.	The SEAL LMS sends disable location reuse response to SEAL LMC-1.

* * * End of Change * * * *