3GPP TSG-SA WG6 Meeting #62	S6-243155

# Maastricht, NL, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0304 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for adaptive location configuration and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Solution#11 of 3GPP TR 23.700-72 was concluded to be considered for the normative work. This CR proposes the implement the Solution#11 in 3GPP TS 23.434. For more details about the solution, please, see 3GPP TR 23.700-72. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adaptive location reporting configuration is specified in SS_LocationReporting API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The non-user-friendly functionality of the SS_LocationReporting API. The application may configure too frequent periodic notification that will overflow the network with the reports or configure big periodicy for the location that leads for low accuracy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.4, 9.3.2.47(new), 9.3.2.48(new), 9.3.5, 9.3.19(new), 9.4.1, 9.4.2.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - . |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested. This information element may be represented as VAL service area ID(s) when this information flow from VAL server to the location management server. |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE as specified in table 9.3.2.4-2. |
| Minimum time between consecutive reports | O see (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. If the Adaptive reporting IE within the provided Triggering criteria is set to "SUGGESTIVE UPDATE", the endpoint information shall include an additional endpoint for the suggested adaptive configurations. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

Table 9.3.2.4-2: Location reporting Triggering criteria

| Information element | Status | Description |
| --- | --- | --- |
| Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered. |
| Adaptive reporting | O | It indicates the request for an adaptive location reporting by dynamically adjusting the configuration. Following are possible values: - "DIRECT UPDATE", indicates the SEAL LMS to directly update the SEAL LM client once configuration is adjusted; or - "SUGGESTIVE UPDATE", indicates the SEAL LMS to provide suggested configuration to VAL server before updating to the SEAL LM client. |
| Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| Location change condition | O (see NOTE) | It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates, SAI, ECGI, RAT, VAL service area. If the Frequency of reporting is event-triggered, this IE shall be provided. |
| Inside or outside indication | O (see NOTE) | It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations represented by the VAL service area ID(s). |
| Reporting schedule | O | It indicates the requested reporting schedule, e.g., days of the week and/or time period for the location reporting. |
| NOTE: Triggering criteria based on the VAL service area represented by the VAL service area ID(s) may be provided when this information flow is from the VAL server to the location management server. |  |  |

* * * Next change * * * *

#### 9.3.2.47	Adaptive location reporting configuration provisioning notification request

Table 9.3.2.47-1 describes the information flow from the location management server to the VAL server (or authorized SEAL LM client) for adaptive location reporting configuration provisioning.

Table 9.3.2.47-1: Adaptive configuration provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Indicates the requestor identity (i.e., SEAL LMS). |
| UE identity | M | Indicates the VAL UE or VAL User identity |
| Proposed adaptive configuration | M | Indicates the proposed adaptive location configuration as described in table 9.3.2.4-2. The |

* * * Next change * * * *

#### 9.3.2.48	Adaptive location reporting configuration provisioning notification response

Table 9.3.2.48-1 describes the information flow from the VAL server (or authorized SEAL LM client) to the location management server for adaptive location reporting configuration provisioning response.

Table 9.3.2.48-1: Adaptive configuration provisioning notification response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the VAL server or authorized SEAL LM client decision, e.g., accept or reject. |

* * * Next change * * * *

### 9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers as specified in table 9.3.2.4-1, e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger corresponding to location management client 1.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger.

a.	If the adaptive reporting is requested in step 1, the location management server interacts with NWDAF (e.g., UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [16]) to obtain the location management client 1 moving statistic and prediction

i.	If "DIRECT UPDATE" is requested, the location management server determines the reporting configuration based on VAL service ID, VAL server ID, and the retrieved location management client 1 moving statistic and prediction. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or to increase or decrease the location accuracy (location change condition), or to change the required positioning method as supported by the UE.

ii.	If "SUGGESTIVE UPDATE" is requested, the location management server initially uses the configuration provided by VAL server and later dynamically adjusts the reporting configuration to determine adaptive location configuration based on VAL service ID, VAL server ID, and by analysing the retrieved location management client 1 moving statistic and prediction and received location information. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or to increase or decrease the location accuracy (location change condition), or to change the required positioning method as supported by the UE.

b.	Depending on the information specified by the location reporting trigger or determined reporting configuration (if "DIRECT UPDATE" is requested), location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server.

Further, if adaptive reporting is enabled by the VAL server with "SUGGESTIVE UPDATE", the location management server dynamically adjusts the reporting configuration as specified above (in step 2.a.ii).

Once location configuration is adjusted and if it is required to update the triggers to the SEAL client, the SEAL LMS may suggest the adaptive location configuration to the VAL server (or authorized SEAL LM client) as specified in clause 9.3.19. If VAL server accepts the suggested adaptive location configuration, then the location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. If VAL server rejects the suggested adaptive location configuration, then the location management server will discard the updated configuration.

* * * Next change * * * *

### 9.3.19	SEAL location management server provides adaptive configuration

Figure 9.3.19-1 illustrates procedure for SEAL LMS to provide adaptive location configuration suggestion to the VAL server for the VAL UE for which VAL server has requested for adaptive reporting (as specified in clause 9.3.5).

NOTE 1:	The procedure is applicable to SEAL LM client-2 (as specified in above clause 9.3.5) if SEAL LM client-2 has requested adaptive reporting.

Pre-conditions:

1.	The VAL server has requested to SEAL LMS to suggest adaptive configuration for VAL UE;

2.	The VAL server has provided endpoint information in the Location reporting trigger to the SEAL LMS.

3.	The SEAL LMS has received location reports from SEAL client and based on the different information the SEAL LMS as determined the new/adaptive location configuration.

Figure 9.3.19-1: Adaptive location reporting configuration provisioning

1.	The SEAL LMS sends a message to the VAL server (or authorized SEAL LM client). The message includes UE identity and suggested updates for location reporting trigger configuration.

2.	Upon receiving the request from the SEAL LMS, the VAL server (or authorized SEAL LM client) sends a response message. The response message includes whether the VAL server accepts or rejects the updated location reporting configuration suggestion or not. If the VAL server accepts the location reporting configuration as suggested by the SEAL LMS, then the SEAL LMS updates the location reporting configuration to the SEAL LM client of the target UE.

* * * Next change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting Notify_Adaptive_Configuration | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |

* * * Next change * * * *

#### 9.4.2.6	Provide_Adaptive_Configuration_Notification operation

API operation name: Provide_Adaptive_Configuration

Description: Provisioning of the adaptive location configuration.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.47

Outputs: Refer subclause 9.3.2.48

See subclause 9.3.19 for the details of usage of this API operation.

* * * End of changes * * * *