3GPP TSG-SA WG6 Meeting #62	S6-243275

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on key issue of differentiating API invokers

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.2

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to introduce a new KI of differentiating API invokers, i.e., API invoker being application run on the UE or the AS,  and application management function being the API invoker.

2. Reason for Change

There are several kinds of CCF APIs and service APIs can be invoked by the external parties as indicated in Figure below.

For those CCF APIs and service APIs, the comsumer of those APIs are different as shown below. (For the CCF APIs, only those APIs related to the API invoker is shown)

| NO | API | Consumer | Remark |
| --- | --- | --- | --- |
| 1 | CAPIF_Discover_Service_API | Management application from the Application developer, or aggregator | Application developer use such API to search the APIs and its instructions. |
| 3 | CAPIF_Events_API | Management application from the Application developer, or aggregator | Application developer subscribes events: SERVICE_API_AVAILABLE, SERVICE_API_UNAVAILABLE or SERVICE_API_UPDATE; API_INVOKER_ONBOARDED or API_INVOKER_OFFBOARDED or API_INVOKER_UPDATED; application subscribes: ACCESS_CONTROL_POLICY_UPDATE; SERVICE_API_INVOCATION_SUCCESS or SERVICE_API_INVOCATION_FAILURE The above events are defined in TS 29.222. |
| 4 | CAPIF_API_invoker_management API | Management application from the Application developer, or aggregator | Onboard/offboard API Invoker |
| 5 | CAPIF_Security API | Management application from the Application developer, or aggregator; 2B/2C application | Application utlizes this CCF API to negotiate the security of CAPIF-2/2e. |

Currently, the definition of API invoker, functional model and the procedure indicate the single API invoker has both the CAPIF-1/1e and CAPIF-2/2e, the procedure may cause misunderstandings that the above CCF APIs and service APIs are always invoked by the same function entity, It is necessary to differentiate API invoker per type as their intention to invoke the CCF APIs or service APIs are totally different.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-22 0.4.0.

* * * First Change * * * *

5.x	Key issue #X: differentiating API invokers

5.x.1	Description

There are several kinds of CCF APIs and service APIs may be invoked by the external parties as below.

For those CCF APIs and service APIs, the comsumer of those APIs are different as shown below. (For the CCF APIs, only those APIs related to the API invoker is shown)

| NO | API | Consumer | Remark |
| --- | --- | --- | --- |
| 1 | CAPIF_Discover_Service_API | Management application from the Application developer, or aggregator | Application developer (or aggregator) uses it to search the CAPIF/servcie APIs description and its usage instructions. |
| 3 | CAPIF_Events_API | Management application from the Application developer, or aggregator | Application developer subscribes events: SERVICE_API_AVAILABLE, SERVICE_API_UNAVAILABLE or SERVICE_API_UPDATE; API_INVOKER_ONBOARDED or API_INVOKER_OFFBOARDED or API_INVOKER_UPDATED; Application subscribes: ACCESS_CONTROL_POLICY_UPDATE; SERVICE_API_INVOCATION_SUCCESS or SERVICE_API_INVOCATION_FAILURE The above events are defined in TS 29.222. |
| 4 | CAPIF_API_invoker_management API | Management application from the Application developer, or aggregator | Application developer (or aggregator) uses it to onboard/offboard API Invoker |
| 5 | CAPIF_Security API | Management application from the Application developer, or aggregator; 2B/2C application | 2B/2C application utlizes this CCF APIs to negotiate the security of CAPIF-2/2e. |

Currently, the definition of API invoker, functional model and the procedure indicate the single API invoker has both the CAPIF-1/1e and CAPIF-2/2e, the procedure may cause misunderstandings that the above CCF APIs and service APIs are always invoked by the same function entity, It is necessary to differentiate API invoker per type as their purpose/intention to invoke the CCF APIs or service APIs are totally different.

5.x.2	Open issues

This key issue includes the following aspects:

-	how to differentiate various types of API invoker.

-	How to enhance the definition, functional model and procedures to reflect different API invokers and the relationship among different API invokers if required.