3GPP TSG-SA WG6 Meeting #62	S6-243497

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243151)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0213 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to store mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MSGin5G service supports storing mechanism. However, in some cases, the user may want to update the store and forward parameters (like message expiry time) or simply wants to discard the stored message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution proposes to introduce new procedure to update store and forward parameters and also to discard stored message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The end user will not have capability to update the message expiry time (or other parameters) for the stored messages. Further, the end user will not have capabiity to discard stored messages. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6.1 (NEW), 8.3.6.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.3.6.1	Procedure for MSGin5G Client to update the stored message

Figure 8.3.6.1-1 shows the procedure for MSGin5G Client to update the stored message. This procedure enables user to update store and forward parameters of the stored message or discard to stored message. It is up to the end user or application client to decide if update the stored message is required or not.

Pre-conditions:

1.	The MSGin5G Client has originated a message to a recipient UE and it is stored to the MSGin5G Server as the recipient UE is not available (as specified in clause 8.3.6).

Figure 8.3.6.1-1: MSGin5G update stored message parameters

0.	The user or the Application Client decides to update the stored message. In order to update the stored message, the user or the Application Client decides to either update store and forward parameters or discard the stored message. The Application Client informs MSGin5G Client to update the stored message.

1.	The MSGin5G Client sends a request message to update the store and forward parameters to the MSGin5G Server. The message includes the Message ID for which store and forward parameters needs to be updated, and new/updated store and forward parameters (e.g. Message expiration time) as specified in Table 8.3.2-2.

Table 8.3.6.1-1: MSGin5G update stored message parameter request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client or the sending Application Server. |
| Message ID | M | Unique identifier of the stored message which requires update (i.e. for which parameters need to be updated or needs to be discarded) |
| Store and forward parameters (NOTE) | M | Updated parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2. |
| NOTE: This IE is only present if the request is to update stored and forward parameters. |  |  |

2.	Upon receiving the request, the MSGin5G Server checks whether the message is stored based on received Message ID. If message is stored, and if "Store and forward parameters" IE is present, then the MSGin5G Server updates the store and forward parameters. if "Store and forward parameters" IE is not present, the MSGin5G server discards the stored message. The MSGin5G Server sends a response back to the MSGin5G Client. If the MSGin5G Client is not authorized to update the stored message (e.g. to upate store and forward parameters or to discard the mssage) or the message does not exists, then the response message includes a failure cause.

Table 8.3.6.1-2: MSGin5G update stored message parameter response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the result is Failure. |  |  |

* * * End of Change * * * *