3GPP TSG-SA WG6 Meeting #62	S6-243624

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243242)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0082 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU set handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SEALDD (as XR app enabler) can facilitate PDU set handling for XR app, as concluded in TR 23.700-23 (KI#8, sol#4) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure description in clause 9.2.2.2 and 9.2.2.3. The corresponding IE table impacts are also added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not supporting new feature in Rel19 XR work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 9.x.y, 9.x.y.1, 9.x.y.2, 9.x.y.3, 9.x.y.4 9.x.y.3.1, 9.x.y.3.2, 9.x.y.3.3, 9.x.y.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.x.y	SEALDD enabled XR data transmission service for XR application

#### 9.x.y.1	General

The SEALDD enabled XR data transmission service facilitates the XR application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

#### 9.x.y.2	Procedure

Figure 9.x.y.2-1 illustrate the procedure for establishing XR data transmission connection.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.x.y.2-1: SEALDD enabled XR data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for XR traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_XRTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements.

2.	Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes XR transmission connection with the SEALDD server. The SEALDD server may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the XR transmission connection response.

7-8.	Same as step 8-9 of clause 9.2.2.2.

The XR application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

#### 9.x.y.3	Information flows

#### 9.x.y.3.1	SEALDD enabled XR transmission request

Table 9.x.y.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting XR transmission service.

Table 9.x.y.3.1-1: SEALDD enabled XR transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |

#### 9.x.y.3.2	SEALDD enabled XR transmission request

Table 9.x.y.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the XR transmission request.

Table 9.x.y.3.2-1: SEALDD enabled XR transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S information Data transmission connection information for multi-modal XR | O | Address(s)/port(s) and/or URL(s) of the SEALDD server to receive the multi-flow packets from the VAL server for application traffic transfer (e.g. audio flow, tactile flow) |
| Cause | O See NOTE | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

#### 9.x.y.3.3	SEALDD XR transmission connection establishment request

Table 9.x.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the XR transmission connection establishment.

Table 9.x.y.3.3-1: SEALDD XR transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flow IDs | M | Identity of the SEALDD-UU flows. |
| VAL server ID | O | Identity of the VAL server. |
| VAL service ID | O | Identity of the VAL service. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD client side used to establish SEALDD multi-modal XR connection. |

#### 9.x.y.3.4	SEALDD XR transmission connection establishment response

Table 9.x.y.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the XR transmission connection establishment.

Table 9.x.y.3.4-1: SEALDD XR transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection. |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

#### 9.x.y.4	APIs

##### 9.x.y.4.1	General

Table 9.x.y.4.1-1 illustrates the APIs exposed by SEALDD server for XR transmission.

Table 9.x.y.4.1-1: List of SEALDD server APIs for XR transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_XRTransmission | Request | Request/Response | VAL server |
| Sdd_XRTransmissionConnnection | Establish | Request/Response | SEALDD client |

##### 9.x.y.4.2	Sdd_XRTransmission Request operation

API operation name: Sdd_XRTransmission Request

Description: The consumer requests for one time for multi-modal XR transmission service.

Inputs: See clause 9.x.y.3.1.

Outputs: See clause 9.x.y.3.2

See clause 9.x.y.2 for details of usage of this operation.

##### 9.x.y.4.3	Sdd_XRTransmissionConnection_Establish operation

API operation name: Sdd_XRTransmissionConnection_Establish

Description: The consumer requests for multi-modal transmission connection establishment.

Inputs: See clause 9.x.y.3.3.

Outputs: See clause 9.x.y.3.4.

See clause 9.x.y.2 for details of usage of this operation.

* * * END of Change * * * *