3GPP TSG-SA WG6 Meeting #62	S6-243669

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243046,S6-243616)

Source:	InterDigital Inc., Samsung

Title:	Pseudo-CR on Spatial anchor management and discovery

Spec:	3GPP TS 23.437 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes normative text on spatial anchor creation and discovery.

2. Reason for Change

The following has been concluded in TR 23.700-21 regarding spatial anchor management and discovery.

i.	for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

3. Conclusions

This pCR provides normative text for KI#1 from Solution #1 and Solution #2.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r23434]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[r23502]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

* * * Next Change * * * *

## 8.3	Spatial anchors management procedures

### 8.3.a	Spatial anchor creation

#### 8.3.a.1	General

Spatial anchor creation enables a SAn client or VAL server to create spatial anchor(s) with the SAn server.

#### 8.3.a.2	Procedure

Figure 8.3.a.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.a.2-1: Spatial anchor creation

- The requestor (e.g., SAn client or VAL server) sends a spatial anchor create request to the SAn server. The request includes a requestor identifier, security credentials, and list of spatial anchors to be created as described in clause 8.3.a.3.1. The request includes access control rules defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). If the request includes spatial anchor specific information, that information remains transparent to the SAn server and can be stored with the spatial anchor. The request can indicate to create multiple spatial anchors.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a name, a URL, a media data, a type, a manufacturing company, a date, an expiry date, a ratings, and customer premise information (e.g. a residence, office or shop), etc. Such information may be stored in key-value pair and is transparent to the SAn server.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the spatial anchor(s), assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The SAn server sends a spatial anchor create response to the requestor. If the SAn server created spatial anchors, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and can include a reason for failure.

#### 8.3.a.3	Information flows

##### 8.3.a.3.1	Spatial anchor create request

Table 8.3.a.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.a.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position | M | The 3D position of the spatial anchor in space (x, y and z coordinates). |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor. |
| > Spatial anchor specific information | O | A container to contain application specific information. |

##### 8.3.a.3.2	Spatial anchor create response

Table 8.3.a.3.2-1 describes information elements for the spatial anchor create response from the SAn service.

Table 8.3.a.3.2-1: Spatial anchor create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor create request was successful. |
| > List of spatial anchor ID | O | The list of created spatial anchor identifiers. |
| Failure response | O | Indicates that the create spatial anchor request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.3.b	Spatial anchor update

#### 8.3.b.1	General

Spatial anchor update enables a SAn client or VAL server to update spatial anchor(s) with the SAn server.

#### 8.3.b.2	Procedure

Figure 8.3.b.2-1 illustrates the spatial anchor update procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.b.2-1: Spatial anchor update

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor update request to the SAn server. The request includes a requestor identifier, security credentials, the identifier of spatial anchors, and spatial anchor information to update as described in clause 8.3.b.3.1. The request may include information to update multiple spatial anchors.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized and the spatial anchor identifier provided in the request is found, the SAn server updates the spatial anchor information based on information provided in the request.

3.	The SAn server sends a spatial anchor update response to the requestor. If the SAn server updated the spatial anchor, the response includes an indication of success and the identifier of updated spatial anchor(s). Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.3.b.3	Information flows

##### 8.3.b.3.1	Spatial anchor update request

Table 8.3.b.3.1-1 describes information elements for the spatial anchor update request from the SAn client or VAL server to the SAn server.

Table 8.3.b.3.1-1: Spatial anchor update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| > VAL service information | O | Information about the application service associated with the spatial anchor. |
| >> Service ID | O | The identifier of the application service. |
| >> Service endpoint | O | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be updated. |
| > Spatial anchor ID | M | Identifier of the spatial anchor to be updated. |
| > Position | O | The 3D position of the spatial anchor in space (x, y and z coordinates). |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor. |
| > Spatial anchor specific information | O | A container to contain application specific information. |

##### 8.3.b.3.2	Spatial anchor update response

Table 8.3.b.3.2-1 describes information elements for the spatial anchor update response from the SAn  server.

Table 8.3.b.3.2-1: Spatial anchor update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the update spatial anchor request was successful. |
| > List of spatial anchor ID | O | List of identifiers of the updated spatial anchors. |
| Failure response | O | Indicates that the update spatial anchor request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.3.c	Spatial anchor deletion

#### 8.3.c.1	General

Spatial anchor deletion enables a SAn client or VAL server to delete spatial anchor(s) with the SAn server.

#### 8.3.c.2	Procedure

Figure 8.3.c.2-1 illustrates the spatial anchor deletion procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.c.2-1: Spatial anchor deletion

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor delete request to the SAn server. The request includes a requestor identifier, security credentials, the identifier of spatial anchors to delete as described in clause 8.3.c.3.1. The request may include information to delete multiple spatial anchors.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized and the spatial anchor identifier provided in the request is found, the SAn server deletes the spatial anchor information.

3.	The SAn server sends a spatial anchor delete response to the requestor. If the SAn server deleted the spatial anchor(s), the response includes an indication of success and the identifier of deleted spatial anchor(s). Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.3.c.3	Information flows

##### 8.3.c.3.1	Spatial anchor delete request

Table 8.3.c.3.1-1 describes information elements for the spatial anchor delete request from the SAn client or VAL server to the SAn server.

Table 8.3.c.3.1-1: Spatial anchor delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of spatial anchors ID | M | List of identifiers of the spatial anchors to be deleted. |

##### 8.3.c.3.2	Spatial anchor delete response

Table 8.3.c.3.2-1 describes information elements for the delete spatial anchor response from the SAn server to the SAn client or VAL server.

Table 8.3.c.3.2-1: Delete spatial anchor response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the delete spatial anchor request was successful. |
| > List of spatial anchor ID | O | List of identifiers of the deleted spatial anchors. |
| Failure response | O | Indicates that the delete spatial anchor request has failed. |
| > Cause | O | The cause for the request failure. |

### 8.3.d	Spatial anchor discovery

#### 8.3.d.1	General

Spatial anchor discovery enables a SAn client or VAL server to discover spatial anchor(s) with the SAn server. The discovery of spatial anchor(s) is based on matching spatial anchor discovery filters provided in the request.

#### 8.3.d.2	Procedure

Figure 8.3.d.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.d.2-1: Spatial anchor discovery

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor discovery request message to the SAn server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and can include spatial anchor discovery filters as described in clause 8.3.d.3.1. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request can additionally include a VAL client identifier.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server identifies spatial anchor(s) that satisfies the spatial anchor discovery filters.

NOTE 1:	User preferences in the spatial anchor discovery filters can be compared with spatial anchor specific information.

For each matching spatial anchor, the SAn server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the access control rules. To determine the location of interest of the requestor, the SAn server may use location information provided in the request. If location information is not provided in the request, the SAn server may obtain location information by invoking the 3GPP Core Network Location Services as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [r23502] directly or via the NEF, or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [r23434].

3.	The SAn server sends a spatial anchor discovery response to the requestor. If the SAn server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors and may includes the recommended list of spatial anchors. Otherwise, the response includes an indication of failure and may include a reason for failure.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the SAn client, the SAn client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

#### 8.3.d.3	Information flows

##### 8.3.d.3.1	Spatial anchor discovery request

Table 8.3.d.3.1-1 describes information elements for the spatial anchor discovery request from the SAn client or VAL server to the SAn server.

Table 8.3.d.3.1-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Discovery filters | O | The filters used for determining the spatial anchors. |
| > List of spatial anchor ID | O | The list of spatial anchor identifiers to discover. |
| > Area of interest | O | The area of interest (e.g., topological area and/or geographical area). |
| > Service ID | O | The identifier of a VAL service. |

##### 8.3.d.3.2	Spatial anchor discovery response

Table 8.3.d.3.2-1 describes information elements for the spatial anchor discovery response from the SAn server.

Table 8.3.d.3.2-1: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor discovery request was successful. |
| List of spatial anchors | O | List of discovered spatial anchors. |
| > Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| > Position | M | The 3D position of the spatial anchor in space (x, y and z coordinates). |
| > VAL service information | M | Information about the application service associated with the spatial anchor. |
| >> Service ID | M | The identifier of the application service. |
| >> Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| > Spatial anchor specific information | O | A container to contain application specific information. |
| Failure response | O | Indicates that the spatial anchor discovery request has failed. |
| > Cause | O | The cause for the request failure. |

## 8.4	SEAL APIs for spatial anchor management

### 8.4.1	General

Table 8.4.1-1 illustrates the SEAL APIs for spatial anchor management.

Table 8.4.1-1: List of SEAL APIs for spatial anchor management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SAnManagement | Create | VAL server, SAn client | Request/Response |
|  | Update | VAL server, SAn client |  |
|  | Delete | VAL server, SAn client |  |
|  | Discover | VAL server, SAn client |  |

### 8.4.1	SS_SAnManagement API

#### 8.4.1.1	General

Service description: This API enables a VAL server (or SAn client) to manage spatial anchors on SAn server.

#### 8.4.1.2	Create operation

API operation name: SS_SAnManagement_Create

Description: The consumer requests to create spatial anchors.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.a.3.1-1.

Outputs: See Table 8.3.a.3.2-1.

See clause 8.3.a for details of usage of this operation.

#### 8.4.1.3	Update operation

API operation name: SS_SAnanagement_Update

Description: The consumer requests to update spatial anchors.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.b.2.1-1.

Outputs: See Table 8.3.b.3.1-1.

See clause 8.3.b for details of usage of this operation.

#### 8.4.1.4	Delete operation

API operation name: SS_SAnManagement_Delete

Description: The consumer requests to delete spatial anchors.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.c.3.1-1.

Outputs: See Table 8.3.c.3.2-1.

See clause 8.3.c for details of usage of this operation.

#### 8.4.1.5	Discover operation

API operation name: SS_SAnManagement_Discover

Description: The consumer requests to discover spatial anchors.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.d.3.1-1.

Outputs: See Table 8.3.d.3.2-1.

See clause 8.3.d for details of usage of this operation.

* * * End of Changes * * * *