3GPP TSG-SA WG6 Meeting #62	S6-243713

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243361, S6-243321)

Source:	Samsung, ZTE

Title:	Pseudo-CR on user consent for group

Spec:	3GPP TR 23.700-22 V0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This contribution proposes on how resource owner’s consent required within the group is managed.

2. Reason for Change

3GPP TS 23.222, clause 8.31, specifies mechanism for API invoker obtaining authorization from resource owner. However, it does not specify how to handle the case of obtaining authorization information with resource owner consent in group communication.

This pCR proposes to add key issue for the above gap.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 V0.4.0.

* * * First Change * * * *

## 5.1	Key issue #1: Managing resource owner consent

### 5.1.1	Description

With the introduction of support for Subscriber-aware Northbound API access in 3GPP Rel-18 a number of requirements were added at stage 1, one of which was motivated by the desire for a UE to be able to control whether or not to provide information considered private to a 3rd party entity. Specifically, the requirement is that the 5G system shall be able to allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party. At stage 2 the CAPIF-8 reference point was introduced to CAPIF where the aspect of consent was highlighted through the statement that the resource owner communicates with the authorization function in the CAPIF core function to manage resource owner consent, with such communication being expected to be performed over CAPIF-8. However, the mechanism for managing such consent was not specified with the functionalities over CAPIF-8 being is FFS and out of scope of the Rel-18 of the specification.

Further, user consent may also be required during group communication to access the information related to the members of the group. For example, the VAL server hosting dynamic platooning group requires access to the location of the members e.g. for performing operations of a group, joining the group.

### 5.1.2	Open issues

The CAPIF does not address management of resource owner consent in the context of supporting RNAA. The open issues are:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored.

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

4.	How to enable CAPIF resource owner’s consent in the context of group.

NOTE:	Aspects pertaining to the definition of resource owner consent/authorization over CAPIF-8 are in the scope of SA3, noting that the R18 security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [3].

* * * Next Change * * * *

<Proposed change in revision marks>