3GPP TSG-SA WG6 Meeting #62	S6-243730

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0309 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure of querying for history location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-72 v1.1.1, the KI#2 for history location querying has been concluded as follows: Solution #3 addressing the KI#2 can be considered in the normative work. Whether the procedure of location history tracing service can reuse the current SEAL-LM procedure will be determined in normative phase. And the detailed APIs and information flows can also be discussed in the normative phase. So it’s needed to specify the functions, procedures and information flows for Sol#3 in TS 23.434 for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedure for location tracing configuration request/response - Add the procedure for location history request/response - Add the procedure for location tracing configuration cancel request/response - Add information flow for above procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | History location querying will not be realized for the vertical applications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.x.1(new),9.3.x.2(new), 9.3.x.3(new), 9.3.2.x(new), 9.3.2.y(new), 9.3.2.z(new), 9.3.2.o(new), 9.3.2.p(new), 9.3.2.q(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.x	Location history querying procedure

9.3.x.1	Location tracing configuration create procedure

Figure 9.3.x.1-1 illustrates the high level procedure of creating location tracing configuration from VAL server to the LM server.

Figure 9.3.x.1-1: Location tracing configuration procedure

- The VAL server sends a location tracing configuration request to the LM server to configure the location tracing service in advance. The location tracing configuration request may contain the geographical area for history location tracing, events/conditions for triggering the location history report, location QoS, the identities for the UE and service, the time till when the SEAL LM needs to store the location history data and the required positioning method(s) , etc.

NOTE:	Step 1 can be also performed when the VAL server requires updating the location tracing configuration corresponding to location management server.

- The LM server checks whether the VAL server is authorized to request the location tracing configuration service. If the request is approved, the LM server stores the configuration received in step 1. If the request includes multiple UEs, the LM server creates and stores the contexts for each UE.

- The LM server sends the location tracing configuration responses to the VAL server.

9.3.x.2	Location history request procedure

Figure 9.3.x.2-1 illustrates the high level procedure of location history request which the VAL server triggers to query the history location data for some specific UEs in some specific conditions.

Pre-conditions:

1.	The location management server has obtained the UE location data from multiple sources as defined in clause 9.3.3 and clause 9.3.5 periodically based on the event triggers or conditions received in the request of step 1 in clause 9.3.x.1.

Figure 9.3.x.2-1: Location history request procedure

- The VAL server sends a location history request to the LM server with the specific conditions (e.g. the given location area, time/period, etc.) to query the history location data for a single UE or multiple UEs.

- The LM server checks the authorization for the request. If the request is authorized, the LM server matches the obtained UE location data with the location history request. Before the match, the LM server may transform the obtained UE location data into the same format of the location area information received in step 1 if needed.

- The LM server sends a location history response to the VAL server with the matched UE location information. If there is no matched UE location data, the failure reason can be included.

9.3.x.3	Location tracing configuration cancel procedure

Figure 9.3.x.3-1 illustrates the high level procedure of location tracing configuration cancel request which the VAL server triggers to cancel the location tracing configuration for a single UE or multiple UEs.

Figure 9.3.x.3-1: Location tracing configuration cancel procedure

- If the VAL server decides to stop the location tracing services for the UE(s), it sends a location tracing configuration cancel request to the LM server with the UE identities.

- The LM server stops getting the UE’s location and deletes the stored UE location data and the location tracing configuration for the service.

- The LM server sends location tracing configuration cancel response to the VAL server.

* * * Next Change * * * *

9.3.2.x	Location tracing configuration request

Table 9.3.2.x-1 describes the information flow from the VAL server to the location management server for creating the location tracing configuration.

Table 9.3.2.x-1: Location tracing configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location tracing configuration is requested. |
| Location area | O | Indicates the Geographic area location information where the VAL server wishes to query the UE history location data. |
| Location QoS | O | Definition of the location Quality of Service for which the location history information is requested. |
| Trigger condition | O | Indicates the event that trigger the sending of the location history report. |
| Time duration | O | Indicates the time till when the history location data needs to be stored. |
| Required positioning method | O | Indicates the requested positoning method to be used for the history location report. |

9.3.2.y	Location tracing configuration response

Table 9.3.2.y-1 describes the information flow from the location management server to the VAL server for the location tracing configuration response.

Table 9.3.2.y-1: Location tracing configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the location tracing configuration request result. |

9.3.2.z	Location history request

Table 9.3.2.z-1 describes the information flow from the VAL server to the location management server for querying the location history information.

Table 9.3.2.z-1: Location history request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location history service is requested. |
| Requested location history information | O | The information for which the history location data is requested. |
| > Location area | O | Indicates the Geographic area where the VAL server wishes to query the UE history location data. |
| > Time | O | Indicates the time when the VAL server wishes to query the UE history location data. |
| > Time duration | O | Indicates the period during that the VAL server wishes to query the UE history location data. |

9.3.2.o	Location history response

Table 9.3.2.o-1 describes the information flow from the location management server to the VAL server for the location history response.

Table 9.3.2.o-1: Location history response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identities of requested VAL users or VAL UEs. |
| Matched UE location history information | M | Identifies of UE location history data that matched the requested location history request from the VAL server. |
| Timestamp | O | Timestamp of the UE location report |

9.3.2.p	Location tracing configuration cancel request

Table 9.3.2.p-1 describes the information flow from the VAL server to the location management server for canceling the location tracing configuration.

Table 9.3.2.p-1: Location tracing configuration cancel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location tracing configuration is requested. |

9.3.2.q	Location tracing configuration cancel response

Table 9.3.2.q-1 describes the information flow from the location management server to the VAL server for the location tracing configuration cancel response.

Table 9.3.2.q-1: Location tracing configuration cancel response

| Information element | Status | Description |
| --- | --- | --- |
| Cancel status | M | It indicates the location tracing configuration cancel request result. |

* * * End of Changes * * * *