3GPP TSG-SA WG6 Meeting #62	S6-243753

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243640, S6-243232)

Source:	Convida Wireless LLC

Title:	HFL training and inferencing

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

The conclusion for key issue #3 in TR 23.700-82 stated that necessary updates to solution #22 can be considered for normative work. The contribution introduces a procedure for AIMLE HFL training and inferencing with necessary updates for clarification.

2. Reason for Change

AI/ML training is a highly iterative process and are performed over many training rounds. In the case of federated and distributed learning, the training is performed with many AIMLE clients. Due to the repetitive nature of AI/ML training, the AIMLE server can be configured to manage the training process over many training rounds for AIMLE service consumers (e.g. VAL servers). AI/ML inferencing can also be provided within the same procedure but for a single round.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	HFL training

### 8.x.1	General

AI/ML training is a highly iterative process and is performed over many training rounds. In the case of federated and distributed learning, the training is performed with many AIMLE clients. Due to the repetitive nature of AI/ML training, the AIMLE server can be configured to manage the training process over multiple training rounds for VAL servers. Note that the HFL training procedure supports horizontal federated learning, distributed learning, transfer learning, and split AI/ML.

The following clauses specify procedures, information flows, and APIs to support HFL training.

### 8.x.2	Procedure

Pre-conditions:

- AIMLE client discovery and selection have been performed.

- Datasets are available and prepared for AI/ML training at the AIMLE clients.

Figure 8.x.2-1: HFL training

  - An AIMLE server receives a ML model training request from a VAL server as described in clause 8.a. If the AIMLE server determines to use HFL training, the procedure continues to step 2.

  - The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.b.

  - At the beginning of a training round, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria is provided in step 1, the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training.

  - The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.x.3.1-1.

  - Each AIMLE client sends a HFL training subscription response with information as described in Table 8.x.3.2-2. If the AIMLE client is not able to grant the subscription (e.g. is not able to perform the training), the AIMLE client sends a response with a fail status and the procedure skips to step 7.

  - Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule.

  - Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.x.3.1-3. The AIMLE client provides an AIMLE service ID for the HFL training operation.

  - If errors were encountered, the procedure skips to step 8.

If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients.

If the training schedule is not complete (e.g. there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 7 are repeated for the next training round.

When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.d to store the AI/ML model in the ML repository.

  - The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.a.

Editor’s Note: The description of the procedure will be further discussed and possibly reformatted.

### 8.x.3	Information flows

#### 8.x.3.1	HFL training subscription request

Table 8.x.3.1-1 shows the request sent by the AIMLE server to AIMLE clients for the HFL training subscription procedure.

Table 8.x.3.1-1: HFL training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AI/ML model and model parameters | M | Information about the AI/ML model and model parameters for use in HFL training. |
| Dataset identifier | M | The dataset identifier associated with the dataset used for HFL training. |
| Number of data samples | M | The number of data samples required for a round of HFL training. |
| Operational schedule | O | A schedule for when training is to occur. |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event-based (e.g. based on percentage completion), upon completion of each training round. |

Editors Note: The presence of Dataset identifier in the HFL training subscription request is FFS.

Table 8.x.3.1-2 shows the response sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.x.3.1-2: HFL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request: success, fail. |
| Subscription ID | O | An identifier for the subscription only if the status is success. |
| AIMLE service ID | M | The service identifier for the AIMLE HFL training operation. |

Table 8.x.3.1-3 shows the notification sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.x.3.1-3: HFL training subscription notification

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| AIMLE service ID | M | The service identifier for the AIMLE HFL training operation. |
| HFL training output | M | ML model parameters from HFL training. |
| Errors list | O | A list of errors encountered during a HFL training round |
| Timestamp | O | A timestamp for the notification |

* * * End of Changes * * * *