3GPP TSG-CT Meeting #105	CP-242213

Melbourne, Australia; 9th – 10th September 2024	was C4-243426

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1037 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorize ML model service consumer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3, eNA_Ph3_SEC |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Annex X.10 of 3GPP TS 33.501, it is stated that: In case of NF service consumer request ML models on behalf of another ML model consumer (e.g., NWDAF containing AnLF), the token request message shall also contain the NF instance ID and Vendor ID of the ML model consumer. The NF Service Consumer also includes the CCA of the ML model consumer that it has received in the service request from the ML model consumer. The AccessTokenRequest data type shall include the ML model consumer NF instance and Vendor ID when the NWDAF requests ML models on behalf of another ML model consumer. And the AccessTokenClaims shall include the ML model consumer NF instance ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the description of sourceNfInstanceId in the AccessTokenReq to include the authorization of an MTLF to request ML models on behalf of an AnLF from another MTLF as defined in SA WG3. Add sourceVendorID to the AccessTokenRequest data type to indicate the vendor ID of the ML model consumer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-compliant with stage-2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5.2.2, 6.3.5.2.4, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI definition of the Nnrf_AccessToken API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Add WIC eNA_Ph3_SEC and Huawei as a cosource. Rev 2: The CR has been agreed in the CT4#124 in Maastricht. Editorial changes in the description of sourceNfInstanceId in table Table 6.3.5.2.4-1 by adding a colon symbol ":" after the if and adding comma sign "," before or. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.3.5.2.2	Type: AccessTokenReq

Table 6.3.5.2.2-1: Definition of type AccessTokenReq

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| grant_type | string | M | 1 | This IE shall contain the grant type as "client_credentials". Enum: "client_credentials" |
| nfInstanceId | NfInstanceId | M | 1 | This IE shall contain the NF instance id of the NF service consumer. |
| nfType | NFType | C | 0..1 | This IE shall be included when the access token request is for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the NF type of the NF service consumer. (NOTE 3) |
| targetNfType | NFType | C | 0..1 | This IE shall be included when the access token request is for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the NF type of the NF service producer. |
| scope | string | M | 1 | This IE shall contain the scopes requested by the NF service consumer. The scopes shall consist of a list of NF service name(s) of the NF service producer(s) or resource/operation-level scopes defined by each service API, separated by whitespaces, as described in IETF RFC 6749 [16], clause 3.3. The service name(s) included in this attribute shall be any of the services defined in the ServiceName enumerated type (see clause 6.1.6.3.11). The resource/operation-level scopes shall be any of those defined in the "securitySchemes" clause of each service API. pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$' See NOTE 2. |
| targetNfInstanceId | NfInstanceId | C | 0..1 | This IE shall be included, if available and if it is an access token request for a specific NF Service Producer. When present this IE shall contain the NF Instance ID of the specific NF Service Producer for which the access token is requested. |
| requesterPlmn | PlmnId | C | 0..1 | This IE shall be included when the NF service consumer in one PLMN requests a service access authorization for an NF service producer from a different PLMN. It may be present when a service access authorization in the same PLMN need to be requested. When present, this IE shall contain the PLMN ID of the requester NF service consumer. (NOTE 3) (NOTE 4) |
| requesterPlmnList | array(PlmnId) | C | 2..N | This IE shall be included when the NF service consumer serving a PLMN, with more than one PLMN ID, requests a service access authorization for an NF service producer from a different PLMN. It may be present when a service access authorization in the same PLMN, with more than one PLMN ID, need to be requested. When present, this IE shall contain the PLMN IDs of the requester NF service consumer. (NOTE 4) |
| requesterSnssaiList | array(Snssai) | O | 1..N | When present, this IE shall contain the list of S-NSSAIs of the requester NF service consumer. If this IE is included in an Access Token Request sent towards a different PLMN, the requester NF shall provide S-NSSAI values of the target PLMN, that correspond to the S-NSSAI values of the requester NF. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF Service Producer. (NOTE 3) |
| requesterFqdn | Fqdn | O | 0..1 | When present, this IE shall contain the FQDN of the requester NF Service Consumer. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF Service Producer. (NOTE 3) |
| requesterSnpnList | array(PlmnIdNid) | O | 1..N | When present, this IE shall contain the list of SNPNs the requester NF service consumer belongs to. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF Service Producer. (NOTE 3) |
| targetPlmn | PlmnId | C | 0..1 | This IE shall be included when the NF service consumer in one PLMN or SNPN requests a service access authorization for an NF service producer from a different PLMN. When present, this IE shall contain the PLMN ID of the target PLMN (i.e., PLMN ID of the NF service producer). |
| targetSnpn | PlmnIdNid | C | 0..1 | This IE shall be included when the NF service consumer in one PLMN or SNPN requests a service access authorization for an NF service producer from a different SNPN. When present, this IE shall contain the SNPN ID of the target SNPN (i.e., SNPN ID of the NF service producer). |
| targetSnssaiList | array(Snssai) | O | 1..N | This IE may be included during an access token request for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the list of S-NSSAIs of the NF Service Producer. |
| targetNsiList | array(string) | O | 1..N | This IE may be included during an access token request for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the list of NSIs of the NF Service Producer. |
| targetNfSetId | NfSetId | O | 0..1 | This IE may be included during an access token request for an NF type and not for a specific NF / NF service instance. When present, this IE shall contain the NF Set ID of the NF Service Producer. |
| targetNfServiceSetId | NfServiceSetId | O | 0..1 | This IE may be included during an access token request for a specific NF / NF service instance. When present, this IE shall contain the NF Service Set ID of the NF Service Producer. This may be used by the NRF to validate that the requester NF service consumer is allowed to access the target NF service instance. (NOTE 3) |
| hnrfAccessTokenUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the Access Token Service (see clause 6.3.2) of the NRF in home PLMN. It shall be included during an access token request for an hSMF in the home routed roaming scenario, if it is returned from the NSSF in the home PLMN (see clause 6.1.6.2.11 of 3GPP TS 29.531 [42]). |
| sourceNfInstanceId | NfInstanceId | C | 0..1 | This IE shall be included, if available and if it is an access token request from the DCCF as NF Service Consumer request data from NF Service Producers on behalf of the source NF. When present this IE shall contain the NF Instance ID of the source NF which intend to collect data from NF Service Producer. This IE shall be included, if available and if the requester is an NWDAF containing MTLF service that requests ML model(s) on behalf of another ML model consumer. When present, this IE shall contain the NF Instance ID of the ML model consumer (e.g. NWDAF containing AnLF). See Annex X.10 of 3GPP TS 33.501 [15]. |
| vendorId | VendorId | C | 0..1 | Vendor Id of requester, shall be included if the requester is an NWDAF containing MTLF and/or AnLF requesting access token to be used for MTLF and/or AnLF service(s) (see Annex X.10 of 3GPP TS 33.501 [15]). |
| analyticsIds | array(NwdafEvent) | C | 1..N | Analytics Id(s) of requester, shall be included if the requester is an NWDAF containing MTLF and/or AnLF requesting access token to be used for MTLF and/or AnLF service(s) (see Annex X.10 of 3GPP TS 33.501 [15]). |
| requesterInterIndList | array(MlModelInterInd) | C | 1..N | This IE shall be included if the NF Service consumer is NWDAF containing MTLF requesting access token to be used for MTLF service(s) and the requester supports interoperability, i.e. the IE is available. The IE contains the Interoperability indicator of the requester per Analytics ID (see Annex X.10 of 3GPP TS 33.501 [15]). |
| sourceVendorId | VendorId | C | 0..1 | This IE shall be included, if available and if the requester is an NWDAF containing MTLF service that requests ML model(s) on behalf of another ML model consumer. When present, this IE shall contain the Vendor ID of the ML model consumer (e.g. NWDAF containing AnLF). See Annex X.10 of 3GPP TS 33.501 [15]. |
| NOTE 1: This data structure shall not be treated as a JSON object. It shall be treated as a key, value pair data structure to be encoded using x-www-form-urlencoded format as specified in clause 17.13.4 of W3C HTML 4.01 Specification [26]. NOTE 2: Though scope attribute is optional as per IETF RFC 6749 [16], it is mandatory for 3GPP as per 3GPP TS 33.501 [15]. NOTE 3: An access token request should be rejected if the requester NF is not allowed to access the target NF based on the authorization parameters in the NF profile of the target NF. The authorization parameters in NF Profile are those used by NRF to determine whether a given NF Instance / NF Service Instance can be discovered by an NF Service Consumer in order to consume its offered services (e.g. "allowedNfTypes", "allowedNfDomains", etc.). Based on operator's policies, an access token request not including the requester's information necessary to validate the authorization parameters in the target NF Profile may be rejected. NOTE 4: When the NF service consumer is serving a PLMN consisting of one PLMN ID, the attribute "requesterPlmn" shall be used; otherwise, if the NF service consumer is serving a PLMN consisting of more than one PLMN ID, the attribute "requesterPlmnList" shall be used. |  |  |  |  |

EXAMPLE:

The following is an example of an Access Token Request message, with a request body encoded as x-www-form-urlencoded, with following input parameters:

-	NF Instance Id of the NF Service Consumer: 4e0b2760-0356-42c4-b739-8d6aaa491b63

-	NF Type of the NF Service Consumer: AMF

-	NF Type of the NF Service Producer: UDM

-	Requested scopes: "nudm-sdm", "nudm-uecm" and "nudm-ueau"

-	PLMN ID of the NF Service Consumer: MCC=123, MNC=456

-	PLMN ID of the NF Service Producer: MCC=321, MNC=654

-	S-NSSAIs of the NF Service Producer: (SST=1, SD=A08923) and (SST=2)

-	NSIs of the NF Service Producer: "Slice A, instance 1" and "Slice B, instance 2"

Note that the URL-encoding of the request body requires to percent-encode the reserved characters ([ ] { } " : ,) that appear in JSON-encoded structured input parameters (such as "requesterPlmn"), and in string input parameters (such as "scope", or "targetNsiList" array elements). Spaces are percent-encoded as '+'.

The request body, before URL-encoding, and displayed in multiples lines only for illustration purposes, would be:

grant_type=client_credentials

&nfInstanceId=4e0b2760-0356-42c4-b739-8d6aaa491b63

&nfType=AMF

&targetNfType=UDM

&scope=nudm-sdm nudm-uecm nudm-ueau

&requesterPlmn={"mcc":"123","mnc":"456"}

&targetPlmn={"mcc":"321","mnc":"654"}

&targetSnssaiList=[{"sst":1,"sd":"A08923"},{"sst":2}]

&targetNsiList=Slice A, instance 1

&targetNsiList=Slice B, instance 2

The actual request message, after URL-encoding, and where all input parameters are contained into one single line in the request body, would be:

POST /oauth2/token

Content-Type: application/x-www-form-urlencoded

Accept: application/json

grant_type=client_credentials&nfInstanceId=4e0b2760-0356-42c4-b739-8d6aaa491b63&nfType=AMF&targetNfType=UDM&scope=nudm-sdm+nudm-uecm+nudm-ueau&requesterPlmn=%7B%22mcc%22%3A%22123%22%2C%22mnc%22%3A%22456%22%7D&targetPlmn=%7B%22mcc%22%3A%22321%22%2C%22mnc%22%3A%22654%22%7D&targetSnssaiList=%5B%7B%22sst%22%3A1%2C%22sd%22%3A%22A08923%22%7D%2C%7B%22sst%22%3A2%7D%5D&targetNsiList=Slice+A%2C+instance+1&targetNsiList=Slice+B%2C+instance+2

* * * Next Change * * * *

##### 6.3.5.2.4	Type: AccessTokenClaims

Table 6.3.5.2.4-1: Definition of type AccessTokenClaims

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| iss | NfInstanceId | M | 1 | This IE shall contain NF instance id of the NRF. , corresponding to the standard "Issuer" claim described in IETF RFC 7519 [25], clause 4.1.1 |
| sub | NfInstanceId | M | 1 | This IE shall contain the NF instance ID of the NF service consumer, corresponding to the standard "Subject" claim described in IETF RFC 7519 [25], clause 4.1.2. |
| aud | Audience | M | 1 | This IE shall contain the NF service producer's NF instance ID(s) (if the exact NF instance(s) of the NF service producer is known) or the NF type of NF service producers for which the claim is applicable, corresponding to the standard "Audience" claim described in IETF RFC 7519 [25], clause 4.1.3. |
| scope | string | M | 1 | This IE shall contain the name of the NF services and the resource/operation-level scopes for which the access_token is authorized for use; this claim corresponds to a private claim, as described in IETF RFC 7519 [25], clause 4.3. pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$' |
| exp | integer | M | 1 | This IE shall contain the expiration time after which the access_token is considered to be expired, corresponding to the standard "Expiration Time" claim described in IETF RFC 7519 [25], clause 4.1.4. |
| consumerPlmnId | PlmnId | C | 0..1 | This IE shall be included if the NRF supports providing PLMN ID of the NF service consumer in the access token claims, to be interpreted for subject (sub IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501 [15]. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| consumerSnpnId | PlmnIdNid | C | 0..1 | This IE shall be included if the NRF supports providing SNPN ID of the NF service consumer in the access token claims, to be interpreted for subject (sub IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501[15]. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerPlmnId | PlmnId | C | 0..1 | This IE shall be included if the access token is granted for an NF service producer belonging to an PLMN and the NRF supports providing PLMN ID of the NF service producer in the access token claims, to be interpretted for audience (aud IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501 [15]. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerSnpnId | PlmnIdNid | C | 0..1 | This IE shall be included if the access token is granted for an NF service producer belonging to an SNPN and the NRF supports providing SNPN ID of the NF service producer in the access token claims, to be interpreted for audience (aud IE), as specified in clause 13.4.1.2 of 3GPP TS 33.501 [15]. When present, it shall contain the SNPN ID of the SNPN the target NF service producer belongs to. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerSnssaiList | array(Snssai) | O | 1..N | This IE may be included if the NRF supports providing list of S-NSSAIs of the NF service producer in the access token claims. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerNsiList | array(string) | O | 1..N | This IE may be included if the NRF supports providing list of NSIs of the NF service producer in the access token claims. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerNfSetId | NfSetId | O | 0..1 | This IE may be included if the NRF supports providing NF Set ID of the NF service producer in the access token claims and if the audience contains an NF type. When present, it shall indicate the NF Set ID of the NF service producer instances for which the claim is applicable. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| producerNfServiceSetId | NfServiceSetId | O | 0..1 | This IE may be included during an access token request for a specific NF / NF service instance, if the targetNfServiceSetId IE is present in the Access Token Request. When present, this IE shall contain the NF Service Set ID of the NF Service Producer for which the access token is applicable. If an NF service producer that receives this IE in the token included in the authorization header does not understand this IE, it shall be ignored. |
| sourceNfInstanceId | NfInstanceId | C | 0..1 | This IE shall be included if the NRF supports providing NF Instance ID of the source NF in the access token claims, if: - it is an access token request from the DCCF as NF Service Consumer request data from NF Service Producers on behalf of the source NF, to be interpreted as subject (sub IE). See Annex X of 3GPP TS 33.501 [15]. or, - it is an access token request from an NWDAF containing MTLF service when the NWDAF requests ML models on behalf of another ML model consumer, in this case, this IE shall contain the NF Instance ID of the ML model consumer (e.g. NWDAF containing AnLF). See Annex X.10 of 3GPP TS 33.501 [15]. |
| analyticsIdList | array(NwdafEvent) | C | 1..N | This IE shall be provided when the producer is an NWDAF containg MTLF and/or AnLF and requested access token to be used for MTLF and/or AnLF service(s). The IE contains Analytics Id(s) that the consumer is authorized to access as specified in Annex X.10 of 3GPP TS 33.501 [15]. |

* * * Next Change * * * *

# A.4	Nnrf_AccessToken API (NRF OAuth2 Authorization)

openapi: 3.0.0

info:

version: '1.3.0'

title: 'NRF OAuth2'

description: |

NRF OAuth2 Authorization.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

schemas:

AccessTokenReq:

description: Contains information related to the access token request

type: object

required:

- grant_type

- nfInstanceId

- scope

properties:

grant_type:

type: string

enum:

- client_credentials

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

targetNfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

scope:

type: string

pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$'

targetNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

requesterPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

requesterPlmnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 2

requesterSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

requesterFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

requesterSnpnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

targetPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

targetSnpn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

targetSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

targetNsiList:

type: array

items:

type: string

minItems: 1

targetNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

targetNfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

hnrfAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

sourceNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

vendorId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/VendorId'

analyticsIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

requesterInterIndList:

type: array

items:

$ref: '#/components/schemas/MlModelInterInd'

minItems: 1

sourceVendorId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/VendorId'

AccessTokenRsp:

description: Contains information related to the access token response

type: object

required:

- access_token

- token_type

properties:

access_token:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

type: integer

scope:

type: string

pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$'

AccessTokenClaims:

description: The claims data structure for the access token

type: object

required:

- iss

- sub

- aud

- scope

- exp

properties:

iss:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

sub:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

aud:

anyOf:

- $ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

- type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

scope:

type: string

pattern: '^([a-zA-Z0-9_:-]+)( [a-zA-Z0-9_:-]+)*$'

exp:

type: integer

consumerPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

consumerSnpnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

producerPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

producerSnpnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

producerSnssaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

producerNsiList:

type: array

items:

type: string

minItems: 1

producerNfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

producerNfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

sourceNfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

analyticsIdList:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

[…]

* * * End of Changes * * * *