3GPP TSG-CT Meeting #105	CP-242245

# Melbourne, Australia; 9th – 10th September 2024                                revision of CP-242021

# Source:	CT4

# Title:	New WID on CT aspects of UPF enhancement for Exposure and SBA Phase 2

# Document for:	Approval

# Agenda Item:	19.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT aspects of UPF enhancement for Exposure And SBA Phase 2

Acronym:	UPEAS_Ph2

Unique identifier:	1050001

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_UPEAS_Ph2 | SA2 | 1020003 | Study on UPF enhancement for Exposure And SBA Phase 2 |
| UPEAS_Ph2 | SA2 | 1040035 | UPF enhancement for Exposure And SBA Phase 2 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 990112 | UPF enhancement for Exposure and SBA | UPEAS Phase 1 in Rel-18 |

Dependency on non-3GPP (draft) specification:

3	Justification

SA#104 has approved a new Rel-19 WID on UPF enhancement for Exposure and SBA Phase 2(UPEAS_Ph2, UI: 1040035, SP-240995). The new WID aims to specify the enhancements to 5GS for 3 KIs as per conclusions reached within TR 23.700-63.

Based on objectives listed in SP-240995, the impacts on protocols and interfaces under CT WGs' responsibilities are foreseen and the related work in CT WGs should be carried out within Rel-19.

4	Objective

The objective of this work item is to specify the stage 3 aspects to support the UPF enhancement for Exposure and SBA Phase 2 according to stage 2 requirements agreed under the stage 2 work item UPEAS_Ph2.

The following impacts on 3GPP CT working groups are identified:

CT3:

-	Possible impacts on the SMF service (e.g. update of the Nsmf_EventExposure service or introduce a new SMF service), to support the retrieval of the NATed UE public IP address and Port.

-	Possible enhancement to the Nsmf_EventExposure service to indicate subscription termination due to PDU session release.

-	Potential impacts on the Nnef_TrafficInfluence service to support the AF interaction with 5GS for the provision of header handling information.

-	Potential impacts on the Npcf_PolicyAuthorization and Npcf_SMPolicyControl services to support the delivery of header handling information.

-	Potential extension of UDR application data to support header handling related information.

CT4:

NRF API enhancement:

-	Possible impacts on the Nnrf_NFManagement service to register new UPF capabilities (e.g. NAT information exposure, Packet inspection capabilities, operator configurable UPF capabilities, handling of Headers/Tags) in UpfInfo.

-	Impacts on the Nnrf_NFDiscovery service to discover UPF with new capabilities.

PFCP enhancement:

-	Possible impacts on N4 interface to report new UP Function Features (e.g. NAT information exposure, Packet inspection capabilities, operator configurable UPF capabilities).

-	Possible impacts on PFCP, to require the UPF to detect and/or handle specific headers/tags and report to the AF.

UDM API enhancement:

-	Impacts on Nudm_SubscriberDataManagement service to indicate UPF functionalities that are required and/or preferred for the PDU session, e.g. packet inspection required.

UPF API enhancement:

-	Impacts on the Nupf_EventExposure service or introduce a new UPF service, to support the retrieval of the NATed UE public IP address and Port base on the private UE IP address.

-	Impacts on Nupf_EventExposure to support Header/Tag detection/handling report.

-	Add the NEF as the consumer of Nupf_EventExposure service to obtain UPF data collection for a specified PDU session or any UE and subscribe Header/Tag detection report.

-	Add a new condition of periodic report in which the subscription is reported as being terminated in the UPF because of the UPF change.

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.244 | Possible impacts on N4 interface to report new UP Function Features (e.g. NAT information exposure, Packet inspection capability, operator configurable parameters for non-standard or partially supported UPF capabilities, and etc.). Possible impacts on N4, to require the UPF to detect and/or handle specific headers/tags and report to the AF based on the detection and/or handling. | TSG CT#109(September 2025) | CT4 |
| 29.503 | Impacts on Nudm_SubscriberDataManagement service to add new attributes in SessionManagementSubscriptionData to indicate UPF functionalities that are required and/or preferred for the PDU session, e.g. packet inspection required. | TSG CT#109(September 2025) | CT4 |
| 29.510 | Possible impacts on the Nnrf_NFManagement service, to register new UPF capabilities (e.g. NAT information exposure, Packet inspection capabilities, operator configurable UPF capabilities, handling of Headers/Tags, ) in UpfInfo. Impacts on the Nnrf_NFDiscovery service to discover UPF with new capabilities. | TSG CT#109(September 2025) | CT4 |
| 29.564 | Impacts on the Nupf_EventExposure service or introduce a new service, to support the retrieval of the NATed UE public IP address and Port base on the private UE IP address. Impacts on Nupf_EventExposure to support Header/Tag detection report. Add the NEF as the consumer of Nupf_EventExposure service to obtain UPF data collection for a specified PDU session or any UE and subscribe Header/Tag detection report. Add a new condition of periodic report in which the subscription is reported as being terminated at the UPF because of the UPF change. | TSG CT#109(September 2025) | CT4 |
| 29.508 | Possible impacts on the Nsmf_EventExposure service or introduce a new service, to support the retrieval of the NATed UE public IP address and Port. Possible enhancement to the Nsmf_EventExposure service to indicate subscription termination due to PDU session release. | TSG CT#109(September 2025) | CT3 |
| 29.512 | Potential impacts on Npcf_SMPolicyControl service to provide the handling information based on the AF request information. | TSG CT#109(September 2025) | CT3 |
| 29.513 | Potential impacts on related policy control signalling flows. | TSG CT#109(September 2025) | CT3 |
| 29.514 | Potential impacts on Npcf_PolicyAuthorization service to provide the handling information for an existing PDU session based on the AF request information. | TSG CT#109(September 2025) | CT3 |
| 29.519 | Potential extension of UDR application data to support header handling related information. | TSG CT#109(September 2025) | CT3 |
| 29.522 | Potential impacts on the Nnef_TrafficInfluence service to support transfer the AF request for handling of Headers/Tags. | TSG CT#109(September 2025) | CT3 |

6	Work item Rapporteur(s)

Rapporteur: Shuang Liang, ZTE, liang.shuang3@zte.com.cn

7	Work item leadership

CT4

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

ZTE

Vodafone

China Mobile

China Telecom

Samsung

CATT

Deutsche Telekom

Nokia

Ericsson

AT&T