3GPP TSG-CT3 Meeting #137	C3-245333

# Hefei, China, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.508 | CR | 0299 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF event exposure architecture corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN, 5G_CIoT |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The architecture of the Nsmf_EventExposure service is wrong and misaligned with SA2 and CT4 specs in that it does not include the SMF as a possible NF service consumer (for the case that the event detection and reporting shall be performed by an intermediate SMF aka I-SMF) and is unclear about the method with which the SMF subscribes to the I-SMF, potentially leading to different implementations and interoperability issues, as well as missing logic in SMF implementations, which is required for the end to end functionality of the 5GC architecture that includes an I-SMF. Further, the SMF can also subscribe to the V-SMF in Home-Routed roaming scenarios (for the DDD event) as specified during the 5G_CIoT Work Item. Corrected also further mistakes in the service architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the SMF as NF service consumer of the SMF Event Exposure service and explain how subscriptions are served in the I-SMF case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potentially different implementations and interoperability issues, as well as missing required logic in SMF implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2, 4.1.3.1, 4.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

### 4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture is also described in 3GPP TS 29.513 [7].

The Session Management Event Exposure Service (Nsmf_EventExposure) is part of the Nsmf service-based interface exhibited by the Session Management Function (SMF). The SMF exhibiting this interface may also be an I-SMF or a V-SMF.

The known NF service consumers of the Nsmf_EventExposure service are:

-	Network Exposure Function (NEF),

-	Access and Mobility Management Function (AMF),

-	Application Function (AF),

-	Unified Data Management (UDM),

-	Network Data Analytics Function (NWDAF),

-	Data Collection Coordination Function (DCCF), and

-	Session Management Function (SMF).

NOTE 1:	The SMF can be an NF service consumer only when the NF service producer is an I-SMF or a V-SMF.

NOTE 2:	The PCF can implicitly subscribe on behalf of the AF or NEF to the UP_PATH_CH, TRAFFIC_CORRELATION event and/or the QOS_MON event by including the information on AF or NEF subscription within the PCC rule.

The AMF accesses the Session Management Event Exposure Service at the SMF via the N11 Reference point.

Figure 4.1.2-1: Reference Architecture for the Nsmf_EventExposure Service; SBI representation

Figure 4.1.2-2: Reference Architecture for the Nsmf_EventExposure Service: reference point representation

# *** Next Change ***

#### 4.1.3.1	Session Management Function (SMF)

The Session Management function (SMF) provides:

-	Session Management e.g. Session establishment, modification and release;

-	UE IP address allocation & management;

-	Selection and control of UP function;

-	Termination of interfaces towards Policy control functions; and

-	Control part of policy enforcement and QoS.

When the NF service producer is an I-SMF, the SMF subscribes / unsubscribes onto the I-SMF using the Nsmf_EventExposure API on behalf of the event consumer (e.g. at I-SMF /I-UPF insertion or when a consumer subscribes or unsubscribes while an I-SMF serves the PDU Session) and the notifications are sent directly to the event consumer.

# *** Next Change ***

#### 4.1.3.2	NF Service Consumers

The Network Exposure Function (NEF);

-	provides means to securely expose the services and capabilities provided by 3GPP network functions to e.g. 3rd parties or internal exposure consumer NF.

The Access and Mobility Management function (AMF) provides:

-	Registration management;

-	Connection management;

-	Reachability management; and

-	Mobility Management.

The Application Function (AF)

-	interacts with the 3GPP Core Network to provide services.

The Unified Data Management (UDM).

-	has access to subscriber information, can determine the SMF serving a user based on that data, and can then subscribe to event notifications for a user (e.g. when triggered by the NEF).

The Network Data Analytics Function (NWDAF)

-	collects data based on event subscription provided by AMF, SMF, UPF, PCF, UDM, AF (directly or via NEF) and OAM;

-	retrieves information about NFs;

-	performs on demand provision of analytics to NF service consumers, as indicated in clause 6, 3GPP TS 23.288 [21].

The Data Collection Coordination Function (DCCF)

-	coordinates the collection and distribution of data and analytics.

The Session Management function (SMF) provides:

-	Session Management e.g. Session establishment, modification and release;

-	UE IP address allocation & management;

-	Selection and control of UP function;

-	Termination of interfaces towards Policy control functions; and

-	Control part of policy enforcement and QoS.

# *** End of Changes ***