3GPP TSG RAN Meeting #105		                                                                      RP-242223

Melbourne, Australia, September 9-12, 2024

Agenda item:    	9.3.2.3

Source:              	Meta

Title:                   	Discussion on Rel-19 XR for NR Phase 3

Document for:  	Discussion and Decision

___________________________________________________________________________________

# Introduction

In this contribution, we discuss our views on revising the WID of the Rel-19 XR for NR Phase 3 [1].

# 2.     	Discussion

# 2.1    	Multi-modality Support

In the revised WID [1], the support of multi-modality has a checkpoint at RAN#105 as:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed).

RAN2 has made the following agreements regarding the support of multi-modality:

- Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. - Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. - FFS whether this can be applied to DL. - RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation) - For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. - For PDU set discard enhancements: - RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4. - RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

Based on the discussions and agreements made in RAN2, multi-modality awareness is supported in RAN in Rel-19 for UL and DL. The multi-modality awareness information (e.g., MMSID, synchronization information, association information, etc.) can be used in admission control, QoS flow to DRB mapping, scheduling, and PDU set discard enhancements.

The multi-modality awareness information from the UE can be available from enhancing the UAI framework. The multi-modality awareness information from the CN has dependency from other Working Groups (e.g., SA2, SA4). For example, as stated in [2], forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Besides MMSID, the availability and form of other information from CN to RAN has not been concluded in SA2.

Considering the progress made in RAN2 on the support of multi-modality, the objective can be updated as follows:

Proposal 1: Update the objective of multi-modality support as:

- Specify aspects related to multi-modality (intra-UE) to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. [RAN2].

  - Specify UAI enhancements to support multi-modality awareness from the UE in RAN.

  - Specify multi-modality awareness from the CN in RAN.
NOTE: Coordination with SA2/SA4 is needed.

  - Specify PDU Set discard enhancements across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows.
NOTE: Coordination with SA2/SA4 is needed.

# 2.2    	Scheduling Enhancements

In the revised WID [1], the scheduling enhancements has a checkpoint at RAN#105 as:

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105

RAN2 has made the following agreements regarding LCP enhancements and Delay Status Report (DSR) enhancements:

For LCP enhancements: - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR; - The solution should consider impact on UE complexity (as already indicated in SI objective description); - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data. For DSR enhancements: - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG; - There will be a single triggering threshold, as in Rel-18.

Accordingly, the objective in the WID can be updated as follows:

Proposal 2: Update the objective of scheduling enhancements as:

- Specify scheduling enhancements to enable high XR capacity while meeting delay requirements/avoiding too late PDUs [RAN2]

  - Specify LCP enhancements using delay/deadline information of packets;

  - Specify DSR enhancements with multiple pairs of remaining time and buffer size for a LCG.

# 2.2    	RAN Awareness and UL Rate Control

In the revised WID [1], a note is captured as:

NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

In RAN #104, a way-forward was made as an outcome of offline discussions in [3]. In [4], we discuss and propose updating Rel-19 XR for NR Phase 3 WID objectives to support RAN awareness and UL rate control.

Proposal 3: Replace the note with an objective to support RAN awareness and UL rate control:

- Specify the UL congestion signaling for XR rate control per QoS flow/per DRB, as a means to enable RAN awareness and XR cross-layer optimization framework. [RAN2]

# 2.2    	Other WI Objectives

Other objectives in the Rel-19 XR for NR Phase 3 WID are progressing as expected. Therefore, there is no need to revise them in the WID.

# 3. Conclusion

In this contribution, we discuss our views on revising the WID of the Rel-19 XR for NR Phase 3 and have the following proposals:

Proposal 1: Update the objective of multi-modality support as:

- Specify aspects related to multi-modality (intra-UE) to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. [RAN2].

  - Specify UAI enhancements to support multi-modality awareness from the UE in RAN.

  - Specify multi-modality awareness from the CN in RAN.
NOTE: Coordination with SA2/SA4 is needed.

  - Specify PDU Set discard enhancements across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows.
NOTE: Coordination with SA2/SA4 is needed.

Proposal 2: Update the objective of scheduling enhancements as:

- Specify scheduling enhancements to enable high XR capacity while meeting delay requirements/avoiding too late PDUs [RAN2]

  - Specify LCP enhancements using delay/deadline information of packets;

  - Specify DSR enhancements with multiple pairs of remaining time and buffer size for a LCG.

Proposal 3: Replace the note with an objective to support RAN awareness and UL rate control:

- Specify the UL congestion signaling for XR rate control per QoS flow/per DRB, as a means to enable RAN awareness and XR cross-layer optimization framework. [RAN2]

References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

- S2-2409444, LS reply on multi-modality awareness at RAN

- RP-241651, Outcome from the XR discussion for NR Phase 3 WI scope, Nokia

- RP-242224, On RAN Awareness and UL Rate Control for XR, Meta, CATT, CMCC, Google, vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Spreadtrum, Samsung, Apple, MediaTek Inc., Futurewei