3GPP TSG RAN Meeting #105	RP-242373

Melbourne, Australia, 9-12 September 2024

Source:	SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI

Title:	New WID Proposal: Introduction of HAPS additional operating bands

Document for:	Approval

Agenda Item:	9.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

- Title: Introduction of HAPS additional operating bands

Acronym: xxxxxx

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-19.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X |  |  |
| No | X | X |  | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_NTN_solutions | RAN2 | 860046 | Solutions for NR to support non-terrestrial networks (NTN) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

3	Justification

In Rel-17 NTN WI (Solutions for NR to support non-terrestrial networks (NTN)), RAN4 agreed to study identifying an operation band in FR1 for High Altitude Platform Station (HAPS) and conducting coexistence study in order to drive RF requirements used as baseline for HAPS [R4-2103877]. Bases on the results of coexistence study in 2 GHz band, required values of HAPS BS ACLR as 45dB and ACS as 46dB were agreed and those studies were reflected in TR 38.863 [R4-2205557]. In terms of reflection in the BS specification, the following agreements were made in RAN4[R4-2203080] [R4-2207346]:

- HAPS is referred as Wide Area BS class in clause 4.4 in TS 38.104.

- NR band n1 can be applied for HAPS operation considering that ITU Radio Regulations identified HAPS frequency band only in 2 GHz at that time.

- NR UEs as defined by current TS 38.101-1 can support HAPS deployments with no additional changes needed in TS 38.101-1.

From November to December 2023, the ITU’s WRC-23 was held in Dubai, United Arab Emirates. At the conference, an official decision was reached to add and modified frequency bands shown in Table 1 which could be used for HAPS base stations. Previously, only the 2 GHz band was available for use by HAPS as IMT base stations (HIBS), but the usage of other global mobile spectrum bands such as 694-960 MHz, 1.7 GHz, and 2.6 GHz were prohibited under ITU regulations.

The frequency bands approved at WRC-23 are already being used for terrestrial (ground-based) services in many countries around the world and can be used with existing user equipment (e.g., smartphones). By making it possible for operators to select frequency bands with flexibility, it is expected that the introduction and development of HAPS-based mobile broadband communications will spread across various countries and regions.

NOTE:	In the current ITU terminology, the use of HAPS to implement IMT (i.e., offer mobile wireless services) is referred to as “HAPS as IMT Base Stations” or HIBS.

Table 1: Frequencies for HAPS as IMT Base Stations (HIBS) after WRC-23

| Frequency Range | Region | F,low – F,high | Remarks |
| --- | --- | --- | --- |
| 694-960 MHz | Region 1 and 2 | 694/698 - 960 MHz (NOTE 1) |  |
|  | Region 3 (6 countries) | 698 – 960 MHz (NOTE 2) | Australia, Maldives, Micronesia, Papus New Guinea, Tonga and Vanuatu |
|  | Region 3 (8 countries) | 703 – 733 MHz (NOTE 3) 758 – 788 MHz 890 – 915 MHz 935 – 960 MHz | China, India, Indonesia, Japan, Korea(Rep. of), Malaysia, the Philippines and Thailand |
| 1710-2170 MHz | Region 1 and 3 | 1710 – 1980 MHz (NOTE 4) 2010 – 2025 MHz 2010 – 2170 MHz |  |
|  | Region 2 | 1710 – 1980 MHz (NOTE 5) 2110 – 2160 MHz |  |
| 2500-2690 MHz | Region 1 and 2 | 2500 – 2690 MHz (NOTE 6) |  |
|  | Region 3 | 2500 – 2655 MHz (NOTE 7) |  |
| NOTE 1: 694/698 – 728 MHz, 805.3 – 806.9 MHz and 830 – 835 MHz to be used for uplink only. NOTE 2: 698 – 728 MHz and 830 – 835 MHz to be used for uplink only. NOTE 3: 703 – 728 MHz to be used for uplink only. NOTE 4: 1710 – 1785 MHz to be used for uplink only in Region 1. 1710 – 1815 MHz to be used for uplink only in Region 3. NOTE 5: 1710 – 1785 MHz to be used for uplink only. NOTE 6: 2500 – 2510 MHz to be used for uplink only. NOTE 7: 2500 – 2535 MHz to be used for uplink only. |  |  |  |

Following the decision of WRC-23, this WI is proposed to add the HAPS operating bands n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85 and n90 in TS 38.104 based on the frequency arrangements described in "HIBS-CHARACTERISTICS" [1], a liaison document in ITU-R that provides the technical and operational characteristics of HIBS.

Note that frequencies added in RR include regulatory conditions (e.g., PFD limits), but these conditions depend on the operational measures in each country, and are not necessary to reflect in base station specifications.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The core part of this work item includes:

- Assess whether existing ACLR and ACS requirements can be reused for the newly proposed HAPS bands.

  - The frequency ranges other than the 694-960 MHz should be prioritized.

- Specify the following NR bands for HAPS operation in TS 38.104.

  - n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85, n90

Note 1: HAPS can be supported using TN functionality available in Rel-17.

Note 2: The frequency range of the downlink in bands n2 and n25 is limited to 1930-1980 MHz. The frequency in band n41 is limited to 2500-2690 MHz.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Update the BS conformance specifications in case any different ACLR or ACS is specified.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.104 | Add HAPS operating bands to clause 5.2 | TSG#106 | Core part |
| 38.141-1 | Update the BS conformance specifications | TSG#106 | Performance part |
| 38.141-2 | Update the BS conformance specifications | TSG#106 | Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Kyohei Sakamoto, SoftBank, kyohei.sakamoto@g.softbank.co.jp

7	Work item leadership

RAN4

8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

SoftBank Corp.

Nokia

Deutsche Telekom

NTT DOCOMO

KDDI

CHTTL

Samsung

Ericsson

- References

[1] ITU-R WP5D#43 [HIBS-CHARACTERISTICS] (Annex 4.10 in Doc. )