3GPP TSG RAN WG1 Meeting #117                                                        	                          R1-2405619

Fukuoka, Japan, May 20th – 24th, 2024

Title:	Reply to RAN2 LS on type 3 PH value for the serving cell configured with mTRP

Response to:	          R1-2403828 (R2-2403968)

Release:	Release 18

Work Item:	          NR_feMIMO-Core, NR_MIMO_evo_DL_UL-Core

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Darcy Tsai

E-mail Address:	Darcy.Tsai@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN2’s LS R1-2403828 (R2-2403968) on type 3 PH value for the serving cell configured with mTRP. With regard to the RAN2’s questions from Question a to d, RAN1 provides the corresponding answers as below:

Response to RAN2 about Enhanced Multiple Entry PHR for multiple TRP MAC CE for Rel-17 mTRP PUSCH repetition:

Question a: Whether UE can provide one type 3 PH value instead of two type 1 PH values for a serving cell that is configured with mTRP PUSCH repetition?

Answer on Question a: According to current RAN1 specification, for a serving cell configured with mTRP PUSCH repetition and configured with a single UL carrier, the UE cannot provide one type 3 PH value instead of two type 1 PH values for the serving cell. For a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers, RAN1 has no consensus on whether/how current RAN1 specification can support the UE to report one type 3 PH value instead of two type 1 PH values.

Question b: If answer to Question a is yes, in which case will the UE report the type 3 PH value for this serving cell, and in which case will the UE report two Type 1 PH values for this serving cell.

Response to RAN2 about Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE for Rel-18 STx2P:

Question c: Whether UE can provide one type 3 PH value with one Pcmax instead of two type 1 PH values with two Pcmax for a serving cell that is configured with multipanelSchemeSDM or multipanelSchemeSFN?

Answer on Question c: No

Question d: If answer to Question c is yes, in which case will the UE provide type 3 PH value with one Pcmax for this serving cell, in which case will the UE provides two type 1 PH values with two Pcmax for this serving cell.

2. Actions:

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above replied information into consideration for their future work.

3. Date of Next TSG-WG1 Meetings:

TSG RAN WG1 Meeting #118    		19th August – 23th August, 2024		Maastricht, Netherlands

TSG RAN WG1 Meeting #118-bis    	14th October – 18th October, 2024	TBC, China