3GPP TSG RAN WG1 #117		R1-240xxxx

Fukuoka, Japan, 20th-24th May 2024

Agenda Item 	:	9.11.4

Source 	:	Moderator (Sony)

Title 	:	DRAFT FL Summary #1 for IoT-NTN

Document for	: Discussion and Decision

1	Introduction	2

2	WID objectives	3

3	Previous agreements	3

4	NPUSCH	7

4.1	Overall summary of issues raised in Tdocs	7

4.2	Evaluation results on OCC schemes	8

4.3	Choice of OCC scheme	11

4.4	Evaluation results on DMRS	13

4.5	DMRS	14

4.6	Signalling and configuration	16

4.7	UL segments	17

4.8	Compatibility with other features	17

5	NPRACH	17

6	Conclusions	17

7	References	17

# Introduction

Main Introduction

This document is the Feature Lead Summary document for the Rel-19 IoT-NTN work item [1].

At this initial stage of the work, the goal is to have an “Study the benefit and identify the options for usage of OCC on NPUSCH and NPRACH”, as stated in the workplan [2].

This FLS contains a set of proposals, which can hopefully be addressed in online meeting time at some stage. The document also contains a set of questions. These questions are intended for the purpose of sharing company views. If there is enough agreement, it might be possible to generate proposals.

NPUSCH

The following issues are discussed for NPUSCH:

- OCC evaluations. Simulation results of the previously agreed OCC schemes.

- Choice of OCC schemes. Can we choose a single scheme to further consider based on evaluations and workload?

- DMRS evaluations. Comparison of CDM and TDM schemes.

- DMRS. Can we choose between CDM and TDM DMRS? Where should the DMRS symbols be located?

- Updated Evaluation assumptions. Some minor corrections to the evaluation assumptions agreed in RAN1#116 need to be considered.

NPRACH

NPRACH issues will be discussed after Monday 20 May.

Follow the naming convention in this example:

- IoTNTNFLS1-v000.docx

- IoTNTNFLS1-v001-CompanyA.docx

- IoTNTNFLS1-v002-CompanyA-CompanyB.docx

- IoTNTNFLS1-v003-CompanyB-CompanyC.docx

If needed, you may “lock” a discussion document for 30 minutes by creating a checkout file, as in this example:

- Assume CompanyC wants to update IoTNTNFLS1-v002-CompanyA-CompanyB.docx.

- CompanyC uploads an empty file named IoTNTNFLS1-v003-CompanyB-CompanyC.checkout.

- CompanyC checks that no one else has created a checkout file simultaneously, and if there is a collision, CompanyC tries to coordinate with the company who made the other checkout (see, e.g., contact list below).

- CompanyC then has 30 minutes to upload IoTNTNFLS1-v003-CompanyB-CompanyC.docx.

- If no update is uploaded in 30 minutes, other companies can ignore the checkout file.

In file names, please use the hyphen character (not the underline character) and include ‘v’ in front of the version number, as in the examples above and in line with the general recommendation, otherwise the sorting of the files will be messed up (which can only be fixed by the RAN1 secretary).

To avoid excessive email load on the RAN1 email reflector, please note that there is NO need to send an info email to the reflector just to inform that you have uploaded a new version of this document. Companies are invited to enter the contact info in the table below.

Issues for which comments are invited in this FLS are labelled with [FL1].

The table below provides a list of points of contact within companies for this WI. Contact details from RAN1#116bis Changsha use a blue font. Please feel free to update your contact details and convert into a black font.

[FL1] Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| SONY | Martin Beale | martin.beale@sony.com |
| Ericsson | Gerardo Agni Medina Acosta | gerardo.agni.medina.acosta@ericsson.com |
| Lenovo | Zhi Yan | yanzhi1@lenovo.com |
| LGE | Daesung Hwang Seungmin Lee Hanjun Park | daesung.hwang@lge.com edison.lee@lge.com hanjun0128.park@lge.com |
| Gatehouse Satcom | René Brandborg Sørensen | rbs@gatehouse.com |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| InterDigital, Inc | Umer Salim | umer.salim@interdigital.com |
| ETRI | Pansoo Kim | pskim@etri.re.kr |
| vivo | Zichao Ji Siqi Liu | jizichao@vivo.com liusiqi@vivo.com |
| Sharp | Zhanping Yin | zyin@sharplabs.com |
| Xiaomi | Xuemei Qiao | qiaoxuemei@xiaomi.com |
| Spreadtrum | Lei Gu | Lei.gu@unisoc.com |

# WID objectives

The IoT-NTN WID [1] has the following objectives:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

# Previous agreements

The following agreements were made in RAN1#116 Athens:

Agreement#116-IoT-NTN #1 For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level - Intra-symbol pre-DFT spreading OCC Agreement#116-IoT-2 The following evaluation assumptions are used for the study of OCC for NPUSCH format 1: Parameter; value. scenario; orbit; GEO; LEO600. Elevation angle; 12.5 degree; 30degree. Channel and impairments; carrier frequency; 2GHz. Channel model; NTN-TDL-C The channels from different UE are independent.. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible.. Timing error; Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO.. Power imbalance; Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value. transmitter; SCS; 3.75KHz and 15KHz; 15kHz. Number of tones; Single tone; Single tone and multi tone up to 12 tones. Waveform; DFT-s-OFDM. Frequency hopping; w/o frequency hopping. MIMO scheme; SISO. DMRS configuration; For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent; For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent. Number of resource unit (); Up to proponent; Up to proponent. Modulation order; Up to proponent; Up to proponent. TBS (); Up to proponent; Up to proponent. Number of repetitions (); Up to proponent. OCC length; Up to 4. OCC sequence; Up to proponent. Number of UE; Up to 4. Velocity of UE; 3km/h. receiver; Receiver algorithm; MMSE. Channel estimation; Real channel estimation. KPI; SNR at 10% BLER; Report for baseline and OCC schemes. Aggregated throughput; Total throughput of up to 4 UEs multiplexed.

The following agreements were made in RAN1#116bis Changsha:

Agreement For the NPUSCH evaluation assumptions, update the DMRS configuration, as follows: DMRS configuration; For baseline evaluations: OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz For OCC evaluations: Up to proponent; For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Up to proponent. Agreement At least the following NPRACH OCC schemes are considered by RAN1 for study: - Intra-symbol group OCC - Inter-symbol group(s) OCC - Inter-repetition OCC Agreement The study of OCC for NPRACH does not consider NPRACH format 2. Agreement The following evaluation assumptions are used for the study of OCC for NPRACH: Parameter; value. Scenario; Orbit and elevation angle; GEO at 12.5 degrees; LEO600 at 30 degrees. Channel and impairments; carrier frequency; 2GHz. Channel model; NTN-TDL-C The channels from different UE are independent.. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible.. Timing error; Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO.. Power imbalance; Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value. Transmitter; NPRACH format; 1 or 0. MIMO scheme; SISO. Number of repetitions (); Up to proponent. OCC length; Up to proponent. OCC sequence; Up to proponent. Number of UE; Up to proponent. Velocity of UE; 3km/h. Total NPRACH time / frequency resource utilisation; To be reported by proponent.. KPI; Target detection probability; 99%. Target false alarm probability; 0.1%. SNR operating point; Report SNR where target detection probability and false alarm probability are reached for baseline and OCC schemes. Agreement OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. Agreement For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study: - TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. - FFS: Detailed mapping - CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. - FFS: Detailed mapping - Other schemes are not precluded, including combinations of the above Agreement For the NPUSCH evaluation assumptions, update the frequency error assumption, as follows. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. For GEO, the same frequency error is applied to each subframe of a transport block. For LEO, the same frequency error is applied to each subframe of a segment (if applied in the evaluation). Companies to report their assumption on frequency error across segments..

# NPUSCH

## Overall summary of issues raised in Tdocs

The following is an overall summary of issues raised by companies in input contributions.

- NPUSCH evaluation results

  - Single-tone OCC evaluation results:

    - 15kHz [Huawei, vivo, ZTE, Xiaomi, Oppo]

    - 3.75kHz [Huawei, QC]

  - Multi-tone evaluation results: [Huawei, vivo, ZTE, CATT, Xiaomi, Oppo]

- DMRS multiplexing schemes:

  - Views on CDM vs TDM [Spreadtrum, HW, vivo, ZTE, LGE, CATT, Lenovo, Ericsson, NEC, ETRI, Oppo, Qualcomm]

  - Cyclic-shift based for multi-tone [HW, Nokia]

  - Location of DMRS symbols [Ericsson, Qualcomm, NEC, LGE]

  - Evaluation results for CDM and TDM DMRS [HW, ZTE, Oppo]

- Configuration of OCC scheme

  - OCC scheme can be configured by network [Lenovo, NEC, Sharp, ETRI]

- Signalling of OCC sequence

  - DCI, semi-static (RRC) or based on C-RNTI

- Types of OCC sequence

  - DFT codes [ZTE, Apple]

    - From PUCCH format 1 in TS38.211 [ZTE]

  - Walsh codes [CATT, Apple]

- OCC size

  - Maximum 4 [Apple]

    - NPDCCH signalling would be the bottleneck if we supported more than OCC4 [Apple]

      - NPDCCH needs to schedule NPDSCH as well as NPUSCH [Apple]

  - Maximum 2 [Ericsson]

    - Phase shift is too large for OCC4 [Ericsson]

  - 2,4 [Apple]

  - Support a limited number of OCC lengths [Nokia]

    - Supporting too many OCC lengths would increase complexity [Nokia]

- Views on OCC schemes for NPUSCH

  - Most companies expressed preferences. These have been discussed in previous meetings (see [3]). We now have some evaluation results that we can use to decide on a scheme.

- Segments

  - Consider the effects of UL segments on OCC operation [spreadtrum ,LGE, Nokia, Ericsson]

    - There will be loss of phase continuity between segments [spreadtrum, LGE]

    - If OCC spans a dropped symbol within a segment gap, the whole segment is dropped, otherwise there will be a loss of orthogonality from a partial OCC unit [Nokia]

- Other features (other than connected mode dynamic grant) with which NPUSCH OCC should work

  - IDLE / INACTIVE (including EDT) [Xiaomi]

    - Xiaomi are unsure whether OCC applies to these cases in the current WID

  - EDT [QC, TCL]

  - PUR [QC, TCL]

  - RACH-less EDT (Rel-19) [QC]

- Compatibility and coexistence with legacy UEs [Nokia]

- NPRACH periodicity

  - NPUSCH with OCC should fit within NPRACH periodicity [Ericsson]

- Odd NPUSCH scheduling delays [Ericsson]

  - NPDCCH should be able to signal odd subframe offset to increase the number of scheduling opportunities for NPUSCH [Ericsson]

## Evaluation results on OCC schemes

The specific details of the simulations are contained within the Tdocs. The evaluations vary depending on the assumptions, including the choice of LEO/GEO, format used (number of RU, number of repetitions, TBS), choice of DMRS scheme, details of how each scheme is implemented etc. Hence, the tables below shows the SNR degradation of each scheme from the baseline scheme of no OCC at a 10% BLER operating point.

15kHz single-tone

Evaluation results were provided on the following OCC schemes for 15kHz single-tone:

- Time domain OCC where OCC spreads across:

  - Symbol-level [Huawei, vivo, ZTE, Oppo]

  - Slot-level [Huawei, vivo, ZTE, Oppo]

  - Repetition-level / RV-level [Huawei, vivo, ZTE, Xiaomi]

Table 1 – 15kHz single tone: SNR degradation for OCC schemes

|  | OCC2 |  |  | OCC4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company | Symbol | Slot | Rep/RV | Symbol | Slot | Rep/RV |
| Huawei | 0.30 | 0.36 | 3.18 | 1.56 | 1.86 | 5.19 |
| Vivo | 3.2 | 3.2 | 3.5 |  |  |  |
| ZTE | 0.3 | 0.3 | 0.3 | 1.6 | 1.8 | 1.9 |
| Xiaomi |  |  | 0.9 |  |  | 4.1 |
| Oppo | 0.3 | 0.5 |  |  |  |  |

Notes: Huawei results are shown for their best-performing DMRS scheme (TDD). Vivo report in their text that the code rate is increased for their symbol-level and slot-level schemes.

The following observation is drawn: there is significant performance degradation as the time-span of the OCC scheme increases. This observation is based on the results from Huawei, ZTE and Xiaomi (comparing OCC2 and OCC4) and the results from Huawei (comparing the symbol-level and slot-level schemes to the rep-/RV-level schemes). The discussion text from vivo indicates that their symbol-level and slot-level schemes lead to an increase in code rate as OCC is applied, leading to a drop in symbol-level / slot-level performance. FL understands that other companies assume that changes to physical channel mapping would mean that this loss of code rate would not happen.

The table below allows companies who provided simulation results to comment on the FL’s summary table in Table 1.

| Company | Comment |
| --- | --- |

3.75kHz single-tone

Evaluation results were provided on the following OCC schemes for 3.75Hz single-tone:

- Time domain OCC where OCC spreads across:

  - Symbol-level [Huawei, Qualcomm]

  - Slot-level [Huawei]

  - Repetition-level / RV-level [Huawei]

Table 2 – 3.75kHz single tone: SNR degradation for OCC schemes

|  | OCC2 |  |  | OCC4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company | Symbol | Slot | Rep/RV | Symbol | Slot | Rep/RV |
| Huawei | 0.83 | 1.23 | 8.03 | 5.49 | 6.01 | large |
| Qualcomm | 0.0 |  |  | 0.43 |  |  |

Notes: Huawei results are shown for their best-performing DMRS scheme (TDD).

The following observation is drawn: there is significant performance degradation as the time-span of the OCC scheme increases. This is evident from the degradation between OCC2 and OCC4 for the results from both companies. The Huawei results also show that as the time-span of the scheme increases (symbol-level -> slot-level –> rep/RV-level), the performance degradation increases.

The table below allows companies who provided simulation results to comment on the FL’s summary table in Table 2.

| Company | Comment |
| --- | --- |

15kHz multi-tone

Evaluation results were provided on the following OCC schemes for 15kHz multi-tone:

- Time domain OCC where OCC spreads across:

  - Symbol-level [Huawei, vivo, ZTE, CATT, Oppo]

  - Slot-level [Huawei, vivo, ZTE, CATT, Oppo]

  - Nslot-level [Huawei, vivo, ZTE, Xiaomi]

  - RV-level [Huawei]

- Intra-symbol pre-DFT spreading OCC [Huawei, vivo]

Table 3 – 15kHz multi-tone: SNR degradation for OCC schemes

|  | OCC2 |  |  |  |  | OCC4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company | Intra-symbol | Symbol | Slot | Nslot | RV | Intra-symbol | Symbol | Slot | Nslot | RV |
| Huawei | 0.79 | 0.72 | 0.94 | 1.17 | large | 0.77 | 0.47 | 2.57 | 8.43 | large |
| Vivo | 2.7 | 3.5 | 1.3 | 1.3 | 1.3 |  |  |  |  |  |
| ZTE |  | 0 | 0 | 0 |  |  | 0.3 | 0.5 | 0.7 |  |
| CATT |  | 3.5 | 3.8 |  |  |  |  | 3.6 |  |  |
| Xiaomi |  |  |  | 1.8 |  |  |  |  | 8.0 |  |
| Oppo |  | 0.5 | 0.5 |  |  |  |  |  |  |  |

Notes: Huawei results are shown for their best-performing DMRS scheme (TDD). Vivo report in their text that the code rate is increased for their symbol-level and slot-level schemes.

The following observation is drawn: there is significant performance degradation as the time-span of the OCC scheme increases. This is evident from the degradation between OCC2 and OCC4 for the results from Huawei, ZTE and Xiaomi. It is also shown in the results from Huawei based on the performance degradation as the timespan of the OCC scheme increases (from intra-symbol level through to RV-level). An outlier is the result from vivo. The discussion text from vivo indicates that their symbol-level schemes lead to an increase in code rate as OCC is applied, leading to a drop in symbol-level / slot-level performance. FL understands that this increase in code rate could be avoided based on changes to physical channel mapping.

The table below allows companies who provided simulation results to comment on the FL’s summary table in Table 3.

| Company | Comment |
| --- | --- |

Overall observations

[FL1] Observation 4.2-1: The performance of the OCC schemes degrades as the time span of the schemes increases (symbol-level through to RV-level).

Companies are invited to comment on observation 4.2-1. Note that there is no intention to reach an agreement on this observation in the meeting.

| Company | Comment |
| --- | --- |
| Lenovo | Is the observation for NPUSCH format 1 OCC scheme? We are OK with the observation in general, but when we do down-selection for the OCC scheme, we should not only consider the performance (e.g., uplink system overall capacity increase? Per-UE SNR performance? What kind of performance) but also consider the standard impact, compatibility with legacy system, etc. |
| Spreadtrum | Fine with the observation. |

[FL1] Observation 4.2-2: The SNR performance degradation of OCC4 is greater than the SNR performance degradation of OCC2. Note that there may still be an overall aggregate throughput gain from OCC4.

Companies are invited to comment on observation 4.2-2. Note that there is no intention to reach an agreement on this observation in the meeting.

| Company | Comment |
| --- | --- |
| Lenovo | It is better to give an explicit observation for OCC 2 and OCC4 on overall aggregate throughput gain. |
| Spreadtrum | Fine with the observation. |

## Choice of OCC scheme

The specification impacts of the schemes have been discussed by various companies. It is generally acknowledged that the schemes with shorter time span (e.g. symbol-level and slot-level) will lead to changes in the physical channel processing chain / resource mapping (for example, repeating symbols or slots). An example summary of the changes required is provided by Huawei (see yellow highlighted cells in the table):

Table 4 Comparison of different OCC schemes (from R1-2403941)

| Spec impact |  |  | RV cycling | Resource mapping | Scheduling restriction | Resource allocation | DMRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RV/Repetition-level | ST |  | Same RV across time span of an OCC sequence | No change | time span of an OCC sequence within a segmentation | No change | TDM/CDM |
|  | MT |  |  |  |  |  | Additional CS |
| Nslot/ Slot level | ST |  | No change | repeat slot by occ-length before mapping new slot | time span of an OCC sequence within a segmentation | scale NRU or TBS | TDM/CDM |
|  | MT | Slot-level |  |  |  |  | Additional CS |
|  |  | Nslot level | No change | No change | OCC length may be restricted to Time span of an OCC sequence within a segmentation | No change |  |
| Symbol level | ST |  | No change | repeat DFT-s-OFDM symbol by occ-length before mapping new symbol | Time span of an OCC sequence within a segmentation OCC length should divide number of data symbols per slot | scale NRU or TBS | TDM/CDM |
|  | MT |  |  |  |  |  | Additional CS |
| Intra-symbol | ST |  | N/A | N/A | N/A | N/A | N/A |
|  | MT |  | No change | pre DFT spreading | OCC length should divide allocated number of subcarriers | scale NRU or TBS | Additional CS |

While there are more minor changes from Nslot-level / repetition-level / RV-level schemes, the performance of these schemes is degraded (see section 4.2) compared to the shorter timespan schemes. Furthermore, it is likely that changes will be required to the DMRS structure to support OCC for any of the schemes and these changes will lead to changes in physical channel mapping / resource mapping anyway.

Some companies [Lenovo, NEC, Sharp, ETRI] have raised the issue of whether it should be possible to configure the OCC scheme that is applied (one network might configure symbol-level OCC while another network configures RV-level). This configurability would lead to an increase in specification impact (multiple schemes would need to be specified), UE capability issues (different UEs would report different capabilities in terms of their supported OCC schemes) and further difficulty in pairing UEs for OCC (not only would the network needs to pair UEs with compatible traffic characteristics, numbers of repetitions, modulation schemes, location, power, performance [Ericsson]), but the network would also need to pair UEs based on supported OCC scheme.

For 3.75kHz single tone, the performance degradation with increasing timespan is more critical, hence it is proposed that the shortest timespan OCC scheme is applied, leading to the following proposal.

[FL1] Proposal 4.3-1: For 3.75kHz single-tone OCC, RAN1 supports symbol-level OCC. Other OCC schemes are not pursued for 3.75kHz single-tone OCC.

Companies are invited to comment on proposal 4.3-1. Companies could comment on:

- whether they support the proposal

- if there is a different scheme that they would propose as a single scheme

- whether they want to support multiple schemes and have the eNB configure one of the schemes

- whether they are OK to perform some down-selection at this time, but are not yet ready to down-select to a single scheme

| Company | Comment |
| --- | --- |
| Ericsson | If NPUSCH Format 1 single-tone with 3.75 kHz is intended to be supported, then an OCC symbol-level granularity seems to be a suitable choice. |
| Lenovo | In general, we are OK to support unified solution for single tone OCC and multi-tone OCC if possible, otherwise we can have separate discussion on each case. Regarding the OCC for 3.75KHz and 15KHz for single tone case, there is no obvious benefit to have different schemes, so it is better to discuss together. when we do down-selection for the OCC scheme (for both single tone and multiple tone cases), we should not only consider the performance (e.g., uplink system overall capacity increase? Per-UE SNR performance?) but also consider the standard impact, compatibility with legacy system, etc. Based on that, we should list some trade-off schemes (e.g., symbol level, slot level, etc) with more details compared with previous meeting for further down-selection in next meetings, and it seems the symbol-level based solution is the one extreme among all potential solutions. |
| LGE | In our understanding, the performance difference between symbol-level OCC and slot-level OCC for single-tone NPUSCH with 3.75kHz SCS is less than 1dB. Meanwhile, the slot-level OCC would keep the existing DMRS pattern. On the other hand, we may need to newly define DMRS pattern to support symbol-level OCC. To minimize workload, our 1st preference is to support only slot-level OCC. To bypass 200MHz frequency offset that will affect to OCC orthogonality, it can be considered that eNB estimates the CFO for the reived NPUSCH without OCC, then the eNB indicates applying OCC to NPUSCH when the estimated CFO is less than a certain threshold. |
| Spreadtrum | We think 3.75kHz and 15kHz should be a unified design, so the Proposal 4.3-1 should include “15kHz” as follow. Proposal 4.3-X: For 3.75kHz and 15kHz single-tone OCC, RAN1 supports symbol-level OCC. Other OCC schemes are not pursued for 15kHz single-tone OCC.. |

Some companies [Huawei, Samsung, vivo, CATT, Xiaomi, Sharp, Nokia, Oppo] propose that there should be a unified design between the single-tone and multi-tone schemes and / or between 15kHz and 3.75kHz single-tone schemes. [Ericsson] propose that if both 3.75kHz and 15kHz tone spacings are supported then symbol-level is supported, but if only 15kHz tone spacing is supported then slot-level is supported

[FL1] Proposal 4.3-2: For 15kHz single-tone OCC, RAN1 supports symbol-level OCC. Other OCC schemes are not pursued for 15kHz single-tone OCC.

Companies are invited to comment on proposal 4.3-2. Companies could comment on:

- whether they support the proposal

- if there is a different scheme that they would propose as a single scheme

- whether they want to support multiple schemes and have the eNB configure one of the schemes

- whether they are OK to perform some down-selection at this time, but are not yet ready to down-select to a single scheme

- whether a unified scheme (with 3.75kHz and / or multi-tone) is supported

| Company | Comment |
| --- | --- |
| Ericsson | If NPUSCH Format 1 single-tone is intended to be supported for both 15 kHz and 3.75 kHz SCS, then Proposal 4.3-2 is ok aiming for pursuing a common design. |
| Lenovo | See comment above |
| LGE | In our understanding, the performance difference between symbol-level OCC and slot-level OCC for single-tone NPUSCH with 3.75kHz SCS is less than 0.5dB. To minimize workload, our 1st preference is to support only slot-level OCC. To bypass 200MHz frequency offset that will affect to OCC orthogonality, it can be considered that eNB estimates the CFO for the reived NPUSCH without OCC, then the eNB indicates applying OCC to NPUSCH when the estimated CFO is less than a certain threshold. |
| Spreadtrum | We think Proposal 4.3-2 should be combined with Proposal 4.3-1, whether 3.75 kHz or 15kHz of single tone should be support. Proposal 4.3-X: For 3.75kHz and 15kHz single-tone OCC, RAN1 supports symbol-level OCC. Other OCC schemes are not pursued for 15kHz single-tone OCC.. |

While the performance evaluations in section 4.2 have shown that OCC schemes can work for multi-tone with little performance degradation compared to the baseline no-OCC case, some companies [vivo, Interdigital, Ericsson, Mediatek] suggest that single-tone schemes are prioritised over multi-tone schemes. Some of the arguments include:

- single-tone operation is supported in NB-IoT for the sake of UL capacity anyway. A network would initially choose single-tone non-OCC operation in order to increases capacity and would only then use an OCC scheme if capacity needed to be further improved

- single-tone operation is appropriate for the link budgets observed in NTN deployments

Other companies [Huawei, Samsung, vivo, CATT, CMCC, Xiaomi, Sharp, Nokia, Oppo] propose that there should be a unified scheme for single-tone and multi-tone. It is not clear that this would reduce the workload as both single-tone and multi-tone schemes would still have to be specified. Furthermore, the choice of scheme for single-tone might be impacted by considerations on multi-tone, where the multi-tone scheme is less suitable for deployment anyway. It would hence seem preferable to priortise support of single-tone. Decisions can be taken on the basis of single-tone considerations and these can then be applied to multi-tone at a future data if there is time and will.

Hence the following proposal is made.

[FL1] Proposal 4.3-3: RAN1 prioritises support of single-tone OCC over multi-tone OCC.

Companies are invited to comment on proposal 4.3-3. Companies could comment on:

- whether they support the proposal

- whether there are cases where a network would choose to use multi-tone OCC in preference to single-tone

- if there were a unified scheme, would design decisions be primarily motivated by single-tone considerations?

- any other suggestions to manage the workload

| Company | Comment |
| --- | --- |
| Ericsson | Ok with Proposal 4.3-3, because in addition to decide whether to support NPUSCH Format 1 single-tone for 15 kHz SCS, or 3.75 kHz SCS or both, RAN1 still needs to discuss the DMRS design. |
| Lenovo | Hope to have discussion in the same priority for the two cases since multi-tone OCC have the benefit on uplink capacity enhancement. |

## Evaluation results on DMRS

A performance comparison of TDM and CDM schemes for DMRS was considered by Huawei, ZTE and Oppo for single-tone OCC. Note that for multi-tone OCC, DMRS for different UEs in an OCC pairing could be separated on the basis of cyclic shift rather than on the basis of TDM / CDM.

The results for 15kHz single-tone operation are shown in Table 5 below. This table shows the degradation of CDM over TDM. Note that the results from Huawei show the opposite – that CDM performs worse than TDM.

Table 5 – DMRS: CDM performance gain over TDM

|  | OCC2 |  | OCC4 |  |
| --- | --- | --- | --- | --- |
| Company | Symbol | Slot | Symbol | Slot |
| Huawei | -3.32 | -3.28 | -4.70 | -6.01 |
| ZTE | 1.2 | 1.2 | 1.6 | 1.6 |
| Oppo | 1.5 | 1.8 |  |  |

The relative performance of CDM and TDM for DMRS may depend on:

- Assumed DMRS structure (although the structures used in all of the simulations above seem to be similar)

- Decoding algorithm (this is not explicitly laid out in contributions)

The TDM and CDM structures used by Huawei, ZTE and Oppo in the simulations are summarised in the following figure (from Oppo R1-2404864):

Figure 1 - CDM and TDM DMRS structures (from Oppo R1-2404864)

Given the divergence of results, it is difficult to draw conclusions on whether CDM or TDM should be supported for DMRS based on evlauation results. Companies are encouraged to further analyse the perormance of TDM and CDM for DMRS are bring further results to RAN1#118.

The table below allows companies who provided simulation results to comment on the FL’s summary table in Table 5.

| Company | Comment |
| --- | --- |

## DMRS

The following views on DMRS were submitted in input documents to RAN1#117:

- Comparison of CDM and TDM DMRS

  - TDM DMRS performance better than CDM performance [Huawei]

  - TDM DMRS performance worse than CDM performance [ZTE, Oppo]

    - TDM DMRS performance is worse due to lower DMRS density [ZTE]

    - TDM DMRS causes non-contiguous UL transmissions and may lead to phase discontinuity [LGE]

    - TDM DMRS creates large gap for measuring CFO (a bad thing) [LGE]

  - CDM allows OCC operation to apply to both DMRS and data [CATT]

  - Preferred DMRS scheme:

    - CDM: Spreadtrum (?), ZTE, LGE, CATT, NEC (?), ETRI, Oppo, Qualcomm (?)

    - TDM: Huawei, Lenovo,

- Location of DMRS symbols

  - TDM of DMRS (in current locations) can only support the OCC multiplexing of 2UEs for 3.75kHz single-tone [Spreadtrum]

  - DMRS locations modified to support pull-in range and to maintain DMRS overhead, i.e. adopt a cluster structure [NEC, Qualcomm]

  - Increase number of DMRS symbols [LGE]

  - Guard period location in 3.75kHz slot format needs to keep the same location as legacy, irrespective of spreading [Ericsson]

  - Remapping of DMRS symbols to new locations may incur interference to DMRS from legacy UEs [Lenovo]

- Multi-tone DMRS

  - Multi-tone DMRS can be differentiated based on cyclic-shifts [Huawei, CMCC, Nokia]

- Others

  - Additional seed for DMRS random number generation [LGE]

CDM vs TDM

It is difficult to decide between CDM and TDM of DMRS based on evaluation results alone as the evaluation results differ. Maybe we could make progress on the basis that:

- Some evaluation results show CDM performance is better than TDM performance.

- There is a higher DMRS density when CDM is applied. When DMRS is applied in two symbols and OCC-ed, isn’t there twice as much DMRS power than for TDM case where some DMRS symbols are dropped?

- TDM causes non-contiguous transmissions, which might affect the overall performance of OCC (if there is a loss of phase coherency either side of the DMRS).

- A majority of companies support CDM. However, we should make decisions based on technical merits rather than votes.

[FL1] Proposal 4.5-1: For single-tone transmissions, a CDM DMRS scheme is supported.

Companies are invited to comment on proposal 4.5-1. Companies could comment on:

- Whether you support the proposal

- Is there better wording?

- Is there a fatal flaw in the CDM scheme that TDM doesn’t have

| Company | Comment |
| --- | --- |
| Ericsson | We should clarify that the proposal is for NPUSCH Format 1 single-tone with 3.75 kHz SCS. We also need clarify what is going to happen with the “Guard Period” which is part of the slot format for 3.75 kHz SCS, for which we should keep its location as per legacy before and after the OCC spreading. We can be ok upon adding those clarifications. |
| Lenovo | If symbol-level based OCC scheme is adopted for single tone, DMRS scheme should be enhanced. We are not ready to accept the CDM DMRS scheme (although we are open with either TDM or CDM), since we don’t have in mind that what kind of CDM DMRS scheme is on the table (how about DMRS density in each slot? New DMRS symbol position for slot? ). Can we list some details on the solution for better understanding? If slot-level based/repetition scheme-based OCC scheme is adopted for single tone, we are OK that the DMRS should be CDM OCC accordingly. |

Location of DMRS symbols

As described in the company views above, there are potentially good reasons for supporting DMRS in different symbol locations compared to those used in Rel-18. These reasons include:

- Support pull-in range for operations with 200Hz CFO while maintaining DMRS overhead

- DMRS locations would change as part of the spreading process anyway

These issue about spreading changing the DMRS locations is considered by Ericsson (R1-2404534):

Figure 5a: Example of legacy DMRS distributions using the legacy slot format for NPUSCH Format 1 with 15 kHz SCS before the OCC spreading.

Figure 5b: Example of legacy DMRS distributions using the legacy slot format for NPUSCH Format 1 with 15 kHz SCS after the OCC spreading.

A DMRS structure to support a bigger pull-in range while maintaining overhead is illustrated by Qualcomm in R1-2405175.

The default assumption would be that OCC DMRS locations are the same as Rel-18. The FL view is that RAN1 should at least consider new OCC DMRS locations. Having the visibility that this is a possibility might help to converge evaluation results.

[FL1] Proposal 4.5-2: For single-tone OCC multiplexing, RAN1 studies:

- Rel-18 mapping of DMRS to symbols in the slot structure

- Alternative mappings of DMRS to symbols in the slot structure

Companies are invited to comment on proposal 4.5-2. Companies could comment on:

- Whether you support the proposal

- Is there better wording?

- Is there a better proposal that would allow us to investigate how to achieve the goal of a DMRS structure that performs acceptably based on a 200Hz CFO?

| Company | Comment |
| --- | --- |
| Ericsson | We think the proposal should differentiate between 15 kHz SCS and 3.75 kHz SCS. For 15 kHz we believe the legacy DRMS mapping can be re-used, whereas for 3.75 kHz SCS an alternative DMRS mapping needs to be discussed. |
| LGE | Depending on whether symbol-level OCC is supported or not, alternative mapping of DMRS may not need to be discussed. We can add “at least for symbol-level OCC” to the 2nd bullet. |

## Signalling and configuration

The following issues related to configuration and signalling of OCC were considered:

- Configuration of basic unit length of OCC

  - Can be configured by a value X [Len]

- Signalling of OCC sequence

  - DCI, semi-static (RRC) or based on C-RNTI

As per RAN1#116 and RAN1#116bis, it is considered that these signalling issues can be considered at a future meeting.

## UL segments

The following issues on UL segments were considered by companies, as for previous meetings:

- Segments

  - Consider the effects of UL segments on OCC operation

    - There will be loss of phase continuity between segments

    - If OCC spans a dropped symbol within a segment gap, the whole segment is dropped, otherwise there will be a loss of orthogonality from a partial OCC unit

At previous meetings, companies preferred to decide on the basic scheme before considering UL segments. FL considers that this is still the situation. Hence, FL suggests that we consider issues related to UL segments in a future meeting.

## Compatibility with other features

The following issues on compatibility with other features were considered in company contributions:

- Other features (other than connected mode dynamic grant) with which NPUSCH OCC should work

  - IDLE / INACTIVE (including EDT) [Xiaomi]

    - Xiaomi are unsure whether OCC applies to these cases in the current WID

  - EDT [QC, TCL]

  - PUR [QC, TCL]

  - RACH-less EDT (Rel-19) [QC]

- Compatibility and coexistence with legacy UEs [Nokia]

It seems like OCC multiplexing for NPUSCH format 1 should operate with the following schemes:

  - Connected mode dynamic grant

  - EDT

  - PUR

  - RACH-less EDT (Rel-19)

OCC multiplexing for NPUSCH format 1 should coexist with legacy UEs.

We should probably decide on the basic OCC multiplexing scheme before considering the features that OCC is going to need to operate with. RAN1 is going to have to consider the compatibility and coexistence with legacy UEs in any case. FL hence proposes that we consider the basic OCC schemes before considering how to apply OCC to the feature list above.

# NPRACH

[To be discussed after Monday 20 May].

# Conclusions

This document is the feature lead summary for IoT-NTN in RAN1#116bis. It contains the FLS discussion and lists the proposals that were considered in offline and online sessions.

# References

[1] RP-234070 	New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

[2] R1-2401298	Work Plan for Rel-19 IoT NTN. Mediatek (rapporteur)

[3] R1-2403719	FL Summary #2 for IoT-NTN. Moderator (Sony). RAN1#116bis, Changsha. April 2024.

R1-2403941	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2404044	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum Communications

R1-2404135	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2404197	Discussion on IoT-NTN uplink capacity enhancement	vivo

R1-2404217	Discussion on UL capacity enhancement for IoT NTN	ZTE

R1-2404264	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2404310	Discussion on IoT-NTN Uplink Capacity Enhancement	Apple

R1-2404326	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2404393	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2404442	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2404474	Discussion on the IoT -NTN uplink capacity/throughput enhancements	CMCC

R1-2404534	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2404610	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2404672	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2404695	IoT NTN OCC multiplexing methods for NPUSCH and NPRACH	Sharp

R1-2404742	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2404787	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2404864	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2405093	Discussion on IoT-NTN uplink capacity and throughput	MediaTek Inc.

R1-2405175	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2405178	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL