3GPP TSG RAN WG3 Meeting #124	R3-243847

Fukuoka, Japan, 20 - 24 May, 2024

Agenda Item:	10.2

Source:	Huawei (moderator)

Title:	SoD of MRO for others

Document for:	Approval

# Introduction

This is a summary of offline discussions with a set of proposed agreements and open issues

# For the Chairman’s Notes

MRO for CHO with Candidate SCG(s)

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases)

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS

MRO for S-CPAC

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

# Background

Previous agreements in RAN3

MRO for CHO with candidate SCG failure and near failure cases

Work on the scenarios of failure in S-CPAC. The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

R19 SON/MDT solution discussion is based on R18 work.

# Discussion

## MRO for CHO with Candidate SCG(s)

### Failure use cases

- Starting with the use cases collected so far:

Discussion:

- Are fast MCG recovery cases included?

- Can we agree to agree use cases above?

Conclusion:

- Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases)

- Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified

### Multiple CHO configurations

Which scenarios do we support:

- Case 1: CHO with candidate SCGs is performed, when only CHO with candidate SCGs is configured, or CHO with candidate SCGs and at least one of legacy R16 CHO and legacy R17 CHO with SCG are configured;

- Case 2: legacy R17 CHO with SCG is performed, when CHO with candidate SCGs and legacy R17 CHO with SCG are configured, or, when CHO with candidate SCGs, legacy R16 CHO and legacy R17 CHO with SCG are configured,

- Case 3: legacy R16 CHO is performed, when CHO with candidate SCGs and legacy R16 CHO are configured, or, when CHO with candidate SCGs, legacy R16 CHO and legacy R17 CHO with SCG are configured.

Discussion:

- The idea is to inform source cell which configuration was used. This could e.g. be included in SHR

Conclusion

- RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs

- Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS

### Near failure use cases

Discussion:

- Do we agree to pursue this now? Or do we do it after?

- What are the impacts foreseen? Can we reuse SHR/SPR?

Conclusion:

## MRO for S-CPAC

### Failure use cases

Discussion

- CPAC used as baseline?

- (maybe too early) For st2 definition: can we use definition for CPAC as baseline and add on top?

- Do we optimize both initial and subsequent failures

Conclusion

- CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

### Successful use cases

Discussion

- Enhance UE history to reduce ping pong?

- Collect info for initiating node? Any need for UE reporting

Conclusion:

-

### Near failure use cases

Discussion

- Do we pursue this now?

- What are the impacts foreseen? Can we reuse SPR?

Conclusion:

-