3GPP TSG-RAN WG4 Meeting #112bis	                                                                                       R4-2415097

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	CATT

Title: 	Discuss on AIoT R2D LLS in RAN4

Agenda item:	6.21.1

Document for:	Discussion

- Introduction

In the co-existence simulation, the SINR threshold is needed for the outage UE. Usually SINR threshold is derived from LLS. D2R LLS is conducted in RAN1, RAN4 may be able to use RAN1 conclusion. But R2D LLS for passive devices is not in the RAN1 formal agreement and link budget is used for the analysis, so RAN4 may need to do the simulation internally. This contribution provides our thinking on some issues for R2D LLS.

- Discussion

  - General model for passive device R2D link

For the passive device R2D link model, our understanding is shown in Figure 1.

Figure 1: General model for passive device R2D link

In Figure 1, noise is added to the input signal of ED. After ED, some noise is residual then to be detected by the OOK detection module. After the decision in the OOK detection, Manchester decoder outputs the bit stream. In last RAN4 meeting, ED input SNR and ED output SNR concept were brought out [2]. It’s benefit to align the understanding and which SNR is used to derive the SNR threshold. So we have the following proposal.

Proposal 1: Companies to align the following concept for passive device R2D LLS.

- What processing modules are included in the LLS link.

- ED input SNR and ED output SNR concept and which SNR is used to derive the SNR threshold.

2.2 RF ED model and comparator model

For the ED and comparator model, some alignment is also needed. Figure 2 is the typical ED+comparator architecture and Figure 3 shows the ED model usually used in the simulation.

Figure 2: Typical ED + comparator architecture

Figure 3: Squiring + LPF model for ED

This digital ED model is to use squaring and low pass filtering. It may be sufficient for the case of the ASK signal like (1+sin(x))*sin(y) if the frequency sin(y) is much higher than sin(x). However, if the AIoT waveform is different than the ASK, this model may not work well. Hilbert Transform is another digital ED model, but it seems it’s also not perfect for some waveform. Figure 4 is copied from [3]. The waveform in Figure 4 was simulated in Cadence, it’s more realistic compared with the RF component performance.

Figure 4: A simulated waveform for ED and comparator using Cadence, copied from [3]

As shown in Figure 4, if the waveform is OOK with single tone, ED can’t detect the envelope perfectly especially for for the OOK ‘0’ when negative clipping happens. ED has some limitations such as ripples and negative clipping. The time constant τ = RC determines the discharging time of the capacitor. The comparator output is also impacted by the ED limitations. This performance is not modelled in the ED model in Figure 3, especially for the capacitor discharging behavior which impacts the comparator’s output. When the waveform is OFDM based OOK waveform, the envelope detector behavior may be different compared with the single tone OOK due to the OFDM random power.

For the detector/comparator part, how to do the 0/1 decision also need discussion. Some identified issues are as following,

- How to generate the data for the detection: averaging the power or averaging the decision?

  - Sampling rate for the ADC

  - Any down sampling compared with the output from comparator?

  - How to generate the data for hard decision

- Detection threshold：Fixed or adjustable?

RAN4 should discuss if the ED and comparator model need to be aligned between the companies.

Proposal 2: RAN4 should discuss if the ED and comparator model need to be aligned between the companies.

- Conclusion

This contribution provides our thinking on AIoT R2D LLS in RAN4. We have the following proposals,

Proposal 1: Companies to align the following concept for passive device R2D LLS.

- What processing modules are included in the LLS link.

- ED input SNR and ED output SNR concept and which SNR is used to derive the SNR threshold.

Proposal 2: RAN4 should discuss if the ED and comparator model need to be aligned between the companies.

- References

[1] R4-2414304, “WF on Co-existence study for ambient IoT”, CMCC, RAN4#112

[2] R4-2413030, “On the RF Requirements for Ambient IoT Devices, Huawei, RAN4#112

[3] A. Trigui, M. Ali, A. C. Ammari, Y. Savaria and M. Sawan, "Energy Efficient Generic Demodulator for High Data Transmission Rate Over an Inductive Link for Implantable Devices," in IEEE Access, vol. 7, pp. 159379-159389, 2019, doi: 10.1109/ACCESS.2019.2950622.