3GPP TSG-RAN WG4 Meeting #112bis	R4-2415564

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	On FR2 OTA testing of STxMP

Source:	Samsung

Agenda item:	6.13.2

Document for:	Discussion

1.	Introduction

In this contribution, discussions on AoA separation and UE orientation, measurement grid, test skipping rules, etc. are presented based on the progress of last meeting, and a new proposal regarding polarization configuration is presented as well.

2. 	Discussion

AoA separation and UE orientation have been extensively discussed for several meeting cycles and companies’ views are more and more converged. The agreements of last meeting are as following [1, R4-2413534]:

Issue 1-2-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreement - Take below proposal as the starting point: - For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enabled (i.e., from Beam Peak Search for MOP in 6.2 of TS 38.101-2, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and AoA pair selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg}. - FFS whether a test direction other than the beam peak direction of single CC operation without STxMP enabled can be used for first AoA - Due to lack of absolute probe location definition of the multi-AoA UE RF test system, defined in Clause 5.2.2 of TR38.871, FFS on whether or how such test system can guarantee the same relative angular orientations between specific UE test directions and probes.

Though there is no absolute probe location in multi-AoA UE RF test system, there is clear relative location among probes. As long as relative relationship between AoA1 and AoA2 for testing is guaranteed, the test system would be doable for STxMP testing.

The agreed starting point is to use the legacy beam peak direction obtained from single CC testing as AoA1, and AoA2 will be decided by AoA1 plus UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg}. Note that the legacy beam peak direction can be obtained under any UE orientation, but STxMP testing is required to be tested under UE declared UE orientation only. It is necessary to transform the legacy beam peak direction with reference to the UE declared UE orientation, if the legacy beam peak direction is not obtained under declared UE orientation.

Proposal 1:	By default, AoA1 direction for STxMP testing is to use the legacy beam peak direction with reference to the declared UE orientation. Transform of beam peak direction from legacy UE orientation to declared UE orientation may be needed if the legacy beam direction was not obtained under declared UE orientation.

Proposal 2:	AoA2 direction is to use AoA1 plus UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

Note that AoA1 direction is not fixed to P0 probe, but just make sure AoA1 and AoA2 pair can be found the same as the relative relationship proposal 1 and proposal 2 defined.

Based on proposal 1 and 2, it is obvious that there is bias between AoA1 and AoA2 since AoA1 is probably showing best EIRP which aligns with peak direction of panel 1, but AoA2 might be biased from the peak direction of panel 2. If the test results based on default AoA pair according to proposal 1 and 2 fail the spec, we should allow tuning of the test directions in xz plane. This kind of tuning can be regarded as a local scan with certain measurement grid step. As long as test results do not pass the spec, the local scan does not stop until AoA1 EIRP fail the spec in both sides of legacy beam peak. The measurement grid for the local scan can be further discussed but the step size should not be greater than the legacy grid for beam peak search.

Proposal 3:	If the test results based on default AoA pair according to proposal 1 and 2 pass the spec, then EIRP PUMAX,f,c,k testing finishes, and measurement grid for scan is not necessary.

Proposal 4:	If the test results based on default AoA pair according to proposal 1 and 2 fail the spec, then a local scan in xz plane is needed. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the spec in both sides of legacy beam peak.

Proposal 5:	The measurement grid for the local scan can be further discussed but the step size should not be greater than the legacy grid for beam peak search.

Regarding the measurement for TRP testing for STxMP, legacy grid can be resued.

Proposal 6:	Ruse the legacy TRP grid for TRP testing of STxMP

About the test skipping rule for EIRPmax and TRPmax, in last meeting a range for additional margin was agreed as [0.5, 2] dB.

Issue 1-3-2: Additional margin for skipping rule for EIRPmax testing - Agreement - Further discuss additional margin considering the range of [0.5dB, 2dB]. - Encourage companies to provide technical Analysis and/or simulation results.

In our understanding, the skipping rule before taking additional margin into account is already conservative. The skipping rule assumes the 2nd beam peak of panel 2 shows the same power level as the real beam peak and exactly overlaps between the two, actually it is already a very extreme case and not likely to occur in real product. So we think it is enough to consider 0.5dB as addition margin.

Proposal 7:	consider 0.5dB as additional margin for EIRPmax and TRPmax test skipping rule, i.e., “If the peak EIRP and TRP of single carrier is no greater than EIRPmax – 3.5dB or TRPmax – 3.5dB, the corresponding EIRPmax or TRPmax verification can be skipped”

The polarization issue has not been discussed in previous meetings. For 2AoA testing, there will be four kinds of polarization combinations. For multi-Rx, it was agreed to only consider two of them to reduce the test complicity, refer to TR 38.751:

In the end, the UE RF requirement is defined as the average (arithmetic mean) of the metric values for two DL polarization test conditions: the first condition is when the DL polarizations at both TRPs are ‘’ and the second condition is when they are ‘’. N%= N1%(AoA1θ, AoA2θ)+N2%(AoA1φ, AoA2φ)2 Where ‘N%(.,.)’ is the metric value for the requirement for the DL pols specified in the subscripts.

EIRP is more complicated than RX testing because for single AoA testing there are four EIRP component testing (2 link pol x 2 measurement pol). When EIRP comes to 2AoA case, the whole EIRP measurements would be 4 link pol combinations x 4 EIRP components = 16 EIRP testing. It is suggested to limit the polarization combinations to simplify the test. Different from Multi-Rx where ‘’ and ‘’ are adopted , crossed link polarization (‘’ and ‘’) for the two AoAs may have the benefits of smaller interference between the probes.

Proposal 8:	It is proposed to limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference.

3. 	Conclusion

Proposal 1:	By default, AoA1 direction for STxMP testing is to use the legacy beam peak direction with reference to the declared UE orientation. Transform of beam peak direction from legacy UE orientation to declared UE orientation may be needed if the legacy beam direction was not obtained under declared UE orientation.

Proposal 2:	AoA2 direction is to use AoA1 plus UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system.

Proposal 3:	If the test results based on default AoA pair according to proposal 1 and 2 pass the spec, then EIRP PUMAX,f,c,k testing finishes, and measurement grid for scan is not necessary.

Proposal 4:	If the test results based on default AoA pair according to proposal 1 and 2 fail the spec, then a local scan in xz plane is needed. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the spec in both sides of legacy beam peak.

Proposal 5:	The measurement grid for the local scan can be further discussed but the step size should not be greater than the legacy grid for beam peak search.

Proposal 6:	Ruse the legacy TRP grid for TRP testing of STxMP

Proposal 7:	consider 0.5dB as additional margin for EIRPmax and TRPmax test skipping rule, i.e., “If the peak EIRP and TRP of single carrier is no greater than EIRPmax – 3.5dB or TRPmax – 3.5dB, the corresponding EIRPmax or TRPmax verification can be skipped”

Proposal 8:	It is proposed to limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference.

4. 	References

- R4-2413534    “WF on Rel-19 NR_FR2_OTA”, Qualcomm Incorporated

- TR 38.751