3GPP TSG-RAN WG4 Meeting#112bis		R4-2415590

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.21.3.4

Source: 	Spreadtrum Communications

Title: 	Discussion on methods and feasibility values of CW interference cancellation

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

Agreement in RAN4#112: - FFS on methods and feasible values for CW cancellation

Discussion

According to the RAN1’s discussion of evaluation scenarios and RAN4’s conclusion on co-existence case as follow.

Scenario; CW Inside/outside topology; Diagram of the scenario; Description of the scenario; Device 1/2a/2b; CW spectrum; D2R spectrum; R2D spectrum. D1T1-A1; CW inside topology; - CW node inside topology 1 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different; Device 1, 2a; Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL); Same as CW. D1T1-A2; - CW node inside topology 1 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D; Same as D1T1-A1; Same as CW. D1T1-B; CW outside topology; - CW node outside topology 1 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same; Case 1-4 (outside topology, UL); Same as CW. D1T1-C; No CW; - No CW Node.; Device 2b; N/A; UL. D2T2-A1; CW inside topology; - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2; Device 1, 2a; Case 2-2 (inside topology, UL); Same as CW. D2T2-A2; - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R; Same as D2T2-A1; Same as CW. D2T2-B; CW outside topology; - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R; Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL); Same as CW. D2T2-C; No CW; - No CW Node. - BS communicates with R; Device 2b; N/A; FFS. Note: this table is for the case where D2R is in the same spectrum as CW2D.. No.; deployment scenario and topology; spectrum; (Aggressor -> Victim); note. 1-1; D1T1-A2- legacy UE only outdoor; R2D: DL CW2D and D2R: DL; Device->NR DL; D2R. NR DL->reader; D2R. Reader->NR DL; R2D. NR DL->device; R2D. 1-2; D1T1-A2-legacy UE indoor; R2D: DL CW2D and D2R: DL; Device->NR DL; D2R. NR DL->reader; D2R. Reader->NR DL; R2D. NR DL->device; R2D. 2-1; D1T1-B-legacy UE only outdoor; R2D: DL CW2D and D2R: UL; Device->NR UL; D2R. NR UL->reader; D2R. Reader->NR DL; R2D. NR DL->device; R2D. 2-2; D1T1-B-legacy UE indoor; R2D: DL CW2D and D2R: UL; Device->NR UL; D2R. NR UL->reader; D2R. Reader->NR DL; R2D. NR DL->device; R2D. 3 (O); D1T1-A2-legacy UE indoor; R2D: UL CW2D and D2R: DL; Device->NR DL; D2R. NR DL->reader; D2R. Reader->NR UL; R2D. NR UL->device; R2D. 4 (O); D1T1-B-legacy UE indoor; R2D: UL CW2D and D2R: UL; Device->NR UL; D2R. NR UL->reader; D2R. Reader->NR UL; R2D. NR UL->device; R2D. 5-1; D2T2-A2-legacy UE only outdoor; R2D: UL CW2D and D2R: UL; Device->NR UL; D2R. NR UL->reader; D2R. Reader->NR UL; R2D. NR UL->device; R2D. 5-2; D2T2-A2-legacy UE indoor; R2D: UL CW2D and D2R: UL; Device->NR UL; D2R. NR UL->reader; D2R. Reader->NR UL; R2D. NR UL->device; R2D. 6-1 (O); D2T2-B-legacy UE only outdoor; R2D: UL CW2D and D2R: DL; Device->NR DL; D2R. NR DL->reader; D2R. Reader->NR UL; R2D. NR UL->device; R2D. 6-2 (O); D2T2-B-legacy UE indoor; R2D: UL CW2D and D2R: DL; Device->NR DL; D2R. NR DL->reader; D2R. Reader->NR UL; R2D. NR UL->device; R2D.

Cases on CW interfere intra A-IoT system

Table1: Cases on CW interfere intra A-IoT system

| No. | deployment scenario and topology | spectrum | Aggressor | Victim |
| --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2 | R2D: DL CW2D and D2R: DL | CW2D | R2D |
|  |  |  | CW2D | D2R |
| 2-1 | D1T1-B | R2D: DL CW2D and D2R: UL | CW2D | D2R |
| 5-1 | D2T2-A2 | R2D: UL CW2D and D2R: UL | CW2D | R2D |
|  |  |  | CW2D | D2R |

Cases on CW interfere NR system

Table2: Cases on CW interfere NR system

| No. | deployment scenario and topology | spectrum | Aggressor | Victim |
| --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | CW2D | NR DL |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | CW2D | NR DL |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | CW2D | NR UL |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | CW2D | NR UL |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | CW2D | NR UL |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | CW2D | NR UL |

Proposal 1: Whether R2D and CW exist at the same time needs to be clarified. If they exist at the same time, cases on CW interfere intra A-IoT system in Table 1 can be considered. Otherwise, only CW interfere at D2R receiver needs to be considered.

Proposal 2: Cases on CW interfere NR system in Table 2 can be considered.

CW cancellation ability at intra A-IoT system

It is observed that CW interferes at D2R receiver and R2D receiver during intra-A-IoT system. Firstly, we analyse the CW interference at the D2R receiver. According to RAN1’s conclusion about CW waveform characteristics, CW includes two different waveforms, respectively single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tone waveforms with a gap. For different waveforms, the complexity of interference suppression is quite different. For single-tone, measuring the CW characteristics would be relatively easy. However, for two tone, requiring additional narrow band-pass filtering. It may be necessary to consider the intermodulation components of two tones. Obviously, measuring the CW characteristic would be relatively hard.

Observation 1: Compared to single-tone waveform, two tones waveform will cause more interference components and increase the complexity of D2R receiver.

For interference suppression capability, at least the following three parts can be considered [2]:

- Part 1: Spatial domain, e.g., antenna isolation, additional circulator or directional coupler, etc.

- Part 2: Analog domain, e.g., RF-IC suppression, etc.

- Part 3: Digital domain, e.g., digital BB interference cancellation, etc.

- Table 3: CW interference suppression capability for A2 and A1/B

|  | A2 | B |
| --- | --- | --- |
| Part 1: Spatial domain | Antenna separation, 30dB Cross-polarisation, 50dB Circulator, a directional coupler, or a quadrature hybrid, 45dB | 58dB@20m (free-space propagation) |
| Part 2: Analog domain | Balun circuit 45dB ; 50dB ; 42-60dB | Less(10-20dB)/no RF-IC |
| Part 3: Digital domain | 60dB; 43dB | 60dB; 43dB |

According to the table3, we evaluate the CW cancellation capability for difference cases as follow in table4.

Table 4: CW cancellation capability at D2R receiver

| No. | deployment scenario and topology | spectrum | Aggressor | Victim | CW cancellation（dB） |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2 | R2D: DL CW2D and D2R: DL | CW2D | D2R | 138 |
| 2-1 | D1T1-B | R2D: DL CW2D and D2R: UL | CW2D | D2R | 155 |
| 5-1 | D2T2-A2 | R2D: UL CW2D and D2R: UL | CW2D | D2R | 120 |

Proposal 3: CW cancellation capability at D2R receiver is proposed in Table4.

Secondly, we need to analyse the CW interference at R2D receiver. As we all known that, the suppression capability of R2D receiver will be relatively poor compared to D2R receiver. It may not be possible to obtain interference cancellation ability through routine research. Hence, we need to model CW interference. How to model CW interference, we need to consider CW power, CW ACLR/IBE and so on, especially for two tones.

Proposal 4: If R2D and CW exist at the same time, for CW interference at R2D receiver, how to model CW interference needs to be considered.

CW cancellation ability at NR system

With regard to CW interference at outdoor NR base station and NR UE, according to the co-existence results, from my point of view, it can be ignored since it will have relatively large propagation loss and wall penetrating loss. We just need to consider CW interference at indoor NR UE. In our understanding, CW cancellation ability at NR UE is similar to D2R receiver. In addition, we could schedule RB/RE resources for NR UE by NR base station to avoid CW interference, including single-tone, two single tones and 3rd order intermodulation products of two tones.

Observation 2: The CW cancellation ability at NR UE and NR BS is similar to that at D2R receiver.

Proposal 5: Schedule RB/RE resource for NR UE by NW to avoid CW interference can be considered.

Conclusions

In this paper, we provide our views on methods and feasibility of CW interference cancellation, we have made the following observations and proposals.

Observation 1: Compared to single-tone waveform, two tones waveform will cause more interference components and increase the complexity of D2R receiver.

Proposal 1: Whether R2D and CW exist at the same time needs to be clarified. If they exist at the same time, cases on CW interfere intra A-IoT system in Table 1 can be considered. Otherwise, only CW interfere at D2R receiver needs to be considered.

Proposal 2: Cases on CW interfere NR system in Table 2 can be considered.

Proposal 3: CW cancellation capability at D2R receiver is proposed in Table4.

Proposal 4: If R2D and CW exist at the same time, for CW interfere at R2D receiver, how to model CW interference needs to be considered.

Observation 2: The CW cancellation ability at NR UE and NR BS is similar to that at D2R receiver.

Proposal 5: Schedule RB/RE resource for NR UE by NW to avoid CW interference can be considered.

Reference

- R4-2414305, WF on impacts of A-IoT  on RF requirements, Huawei, TSG RAN WG4 meeting #112

- R1-2405915, Discussion on waveform characteristics of external carrier wave for Ambient IoT, Spreadtrum, TSG RAN WG1 meeting#118