3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415767

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for intermediate UE

Agenda Item:	6.21.3.3

Document for:	Approval

- Introduction

In the last meeting, the following requirement is FFS:

- FFS whether following requirements are needed or not:

- TX

  - Output power dynamics

  - Transmit ON/OFF power

  - Transmit ON/OFF time mask

  - IBE

- RX

  - ACS

  - In-band blocking

  - Out-of-band blocking

  - Narrow band blocking

  - Receiver spurious response

  - Rx spurious emission

In this contribution we provide our views on these requirements.

- Discussion

- Output power dynamic/ Transmit ON/OFF time mask/Transmit ON/OFF power

As we mentioned in the last meeting, unlike AIoT BS, the intermediate UE need to support both NR and AIoT, and all these requirements are already verified in NR requirements, and not need to be defined for AIoT side.

- IBE

In [2], we provide the co-existence study for D2T2 and we observe that reusing NR IBE assumption for intermediate reader may lead to large throughput loss in NR, which should be updated based on co-existence study.

Observation 1: The NR IBE assumption will lead to co-existence issue which need to be updated for intermediated UE.

Proposal 1: The following Tx requirement is not needed for intermediate UE

  - Output power dynamics

  - Transmit ON/OFF power

  - Transmit ON/OFF time mask

Proposal 2: The ACLR should be defined for intermediated UE based on co-existence study instead of reuse NR IBE.

- In-band blocking/Out-of-band blocking/Narrow band blocking/Receiver spurious response/Rx spurious emission

Similar to Tx requirements, these requirements are fully verified form NR side when the D2R is in the DL spectrum. If the intermediate UE need to support receive the D2R in UL spectrum, all these requirements need to be defined since different receiver is used.

Proposal 3: The following Rx requirement is only defined for intermediate UE when it need to support receive D2R in UL spectrum

  - In-band blocking

  - Out-of-band blocking

  - Narrow band blocking

  - Receiver spurious response

  - Rx spurious emission

- Conclusion

In this contribution, we provide our view on AIoT BS RF requirement.

Observation 1: The NR IBE assumption will lead to co-existence issue which need to be updated for intermediated UE.

Proposal 1: The following Tx requirement is not needed for intermediate UE

  - Output power dynamics

  - Transmit ON/OFF power

  - Transmit ON/OFF time mask

Proposal 2: The ACLR should be defined for intermediated UE based on co-existence study instead of reuse NR IBE.

Proposal 3: The following Rx requirement is only defined for intermediate UE when it need to support receive D2R in UL spectrum

  - In-band blocking

  - Out-of-band blocking

  - Narrow band blocking

  - Receiver spurious response

  - Rx spurious emission

- References

- R4-2414305 WF on impacts of A-IoT on RF requirements, RAN4#112, Huawei

- R4-2415764, Discussion on the co-existence for D2T2, RAN4#112bis, vivo