3GPP TSG-RAN WG4 Meeting #112bis                                 R4-2415961

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	6.1.1.2.1

Document for:	Approval

- Introduction

In RAN#112, the WF was agreed upon in [1], and there are still some FFS issues. In this paper, we will propose our views and discuss these issues.

- Extended UE channel bandwidth further study

2.1 Approaches of converting outer RB allocation to inner RB allocation

#### At RAN4#112, the following was agreed:

#### Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

- Proposals

  - Approach 1: UE CBW based

    - Option 1: The extended UE CBW based method as captured in the WF in last meeting.

  - Approach 2: BS CBW based

    - Option 2: BS CBW based inner/outer method. (Skyworks)

      - BS NRB instead of UE NRB.

      - Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart.

      - The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR.

      - For margin or stricter emissions, the BS can signal a reduced NRB and RBshift.

    - Option 2a: the feature ‘narrower UE channel BW within wider BS bandwidth’ is specified for a UE configured with a UE-specific CHBW within a wider BS bandwidth, the said UE compliant with selected OOBE requirements applicable for the wider BS bandwidth (the cell bandwidth). (Ericsson, Nokia)

- WF

  - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach

One way of viewing Option 1 and Option 2 is that it is like a UE extended CBW where the bandwidth extension on one or both sides can be far greater than ½ of the UE channel bandwidth, but also where the extension may be smaller than the ½ channel bandwidth. From our analysis, we have not seen any value in enabling a larger extension than ½ of the channel bandwidth in achieving MPR reduction for outer RB allocations. With regards to allowing more granularity of bandwidth extension where the extended available bandwidth on each side would be less than ½ of the UE channel bandwidth, there appear to be diminishing returns in terms of configuration flexibility vs gains.

Our aim at the start of this work item was to keep the level of complexity and RAN5 testing as well as any interoperability testing relatively simple to minimize ambiguity of what is required of the UE, and also avoid large specification effort. The more granularity we have of the bandwidth extension on each side, the more scenarios will need to be analyzed to ensure that there are no issues, the more complex the specification and implementation of this feature will become (risking ambiguity in the eventual specifications), the more RAN5 work will be needed and more testing that will need to be performed for each UE, and market adoption will no doubt be impacted. Therefore, we would request to keep this functionality relatively simple for the UE, and not over-engineered. Also, the main scenario where this would be useful “within the BS CBW” would be a for a Redcap UE, and the RedCap UE will not support wider channel bandwidths than 20MHz. More testing time = more cost for such a UE, so we would prefer to focus on simple cases for bandwidth extension with justifiable gains rather than full flexibility.

Proposal 1: Limit symmetrical and asymmetrical UE channel bandwidth extension to preferably a single bandwidth extension of ½ bandwidth extension on one or both sides of the original UE channel bandwidth.

2.2 Applicability of IBE in region between UE CBW and ext-CBW

#### At RAN4#112, the following was agreed:

#### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW

- WF

  - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW

As highlighted in MediaTek’s analysis [2] and [3] in previous meetings, the UE CBW = BS CBW for LTE, and therefore IBE was always applicable for UE Tx emissions within a BS CBW, and ACLR/SEM was applicable for emissions outside of the BS CBW. For NR, we defined the same approach for an eMBB UE that operates UE CBW = BS CBW. However, for UE CBW < BS CBW that does not apply today and we see no reason from a specification perspective why it could not apply – especially for a RedCap UE where in high FR1 bands UE CBW < BS CBW may frequently apply.

From a device implementation perspective, if the ACLR/SEM restrictions leading to Outer RB allocation MPR are removed, for IBE not to become a new bottleneck, it would need to be clarified that the IBE formulas in TS38.101-1 6.4.2.3 apply according to the Extended UE channel bandwidth (i.e. assume that non-allocated RBs extend to NRB of extended channel bandwidth rather than NRB of original UE channel bandwidth.

It is acknowledged that the inter-RB interference within the BS CBW may be slightly higher from applying “extended” IBE rather than applying ACLR/SEM, but this is a trade-off that operators may make in their networks. Therefore, we believe it would be reasonable for operators to have full control over whether the “extended CBW” is permitted to be used, via network signalling to the UE, whereby e.g. a network may permit the usage of extended CBW if it is interested to enable the MPR reduction feature within the cell.

Proposal 2: Agree that an “extended” IBE shall apply in the region between UE CBW and extended CBW, i.e. where the NRB of extended channel bandwidth in the formula is assumed to apply as opposed to that of the original channel bandwidth. Also agree to allow operators to be given control via network signalling to the UE of whether usage of extended UE CBW is permitted within the cell.

2.3 Starting point of ΔfOOB for UE ACLR/SEM

#### At RAN4#112, the following was agreed:

#### Issue 1-2-5: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

  - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth.

- WF

  - FFS in next meeting

We propose to apply ACLR and SEM from the same point. We would not like to apply ACLR from edge of ext-CBW and SEM from edge of UE CBW, as this would complicate further the definitions of Inner and Outer RB allocations, with increased impact from SEM likely leading to more restrictions on narrow allocations near the edge of the UE CBW, which may put more restrictions on the Inner RB allocation applicability for this.

Proposal 3: Agree that requirements for ACLR and SEM apply from edge of extended CBW.

  - Spurious Emission FOOB boundary

#### At RAN4#112, the following was agreed:

#### Issue 1-2-6: Boundary to apply SE

- WF

  - [SE applicability depends on the extended CBW]

The Spurious Emission boundary is currently specified to be at the end of the OOBE boundary, and 5MHz away from the ACLR region. As stated in MediaTek documents [2] and [3] in the previous meetings, ITU allows the Spurious Emission to apply 250% of the channel BW from the centre of the channel. 3GPP requirements are more stringent but driven by legacy UMTS rather than regulation. Therefore, shifting the boundary compared to current 3GPP FOOB is feasible.

We do believe that from specification perspective it would be more reasonable to shift the FOOB if we apply an extended CBW. Also from a minimum requirements perspective, if we do not shift the SE boundary but we shift the OOBE, then there would be a point where the two requirements overlap and the Spurious Emission requirement would be more stringent than ACLR/SEM.

Proposal 4: Agree to shift the FOOB boundary to the edge of the OOBE for extended CBW.

2.5 Integral region of OOBE – based on UE CBW or ext-CBW

At RAN4#112, the following was agreed:

#### Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE

- Proposals

  - Option 1: The integral region and the boundary of OOBE should be based on UE CBW.

  - Option 2: The integral region and boundary of OOBE is based on extended UE CBW.

- WF

  - FFS in next meeting

We provide our analysis of the pros and cons in the table below. We have also performed simulations to identify whether there would be any issue to apply OOBE based on the UE CBW, starting from edge of extended CBW.

Table 1: Analysis of OOBE integration options

|  | OOBE over UE CBW | OOBE over ext-CBW |
| --- | --- | --- |
| Specification impact | More complex: Would need to define the “shifted edge” in OOBE section. Mapping needed to higher channel BW size for MPR rules. | Simpler: Can re-apply MPR formulas and refer to defined channel BW sizes and corresponding OOBE requirements. |
| Impact to other requirements | If SE shifted with OOBE region, then it can still fulfil ITU requirements for 250% of UE CBW. Similar IBE requirements impact for both options, i.e. extending IBE applicability outside of UE CBW. | SE for 250% of UE CBW would not be achievable if applied from edge of ext-CBW OOBE region. No issue if the 250% applied to ext-CBW instead. Similar IBE requirements impact for both options, i.e. extending IBE applicability outside of UE CBW. |
| Impact on MPR reduction | Our simulations suggest that Inner RB Allocation MPR can apply for all RB allocations inside 20MHz channel | From MPR formulas it can be derived that Inner RB Allocation MPR can apply for all RB allocations inside 20MHz channel |

2.6 Ratio of UE channel bandwidth extension required

At RAN4#112, the following was agreed:

#### Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW

- Proposals

  - Proposal 1: (Apple)

    - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel.

    - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third.

  - Proposal 2: (Qualcomm)

    - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW.

    - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW.

  - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo)

  - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO)

  - Proposal 5: (Huawei)

    - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication.

    - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency.

      - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK

From our analysis provided in previous meetings, extending the channel bandwidth by 1/2 of the UE CBW symmetrically would enable the NRB to be equivalent to an Inner RB allocation from an MPR perspective, which aligns with the definition in TS38.101-1. Therefore, we think that extending the channel bandwidth by 1/2 of the UE CBW symmetrically can be used for symmetrical UE channel bandwidth extension. If the BS CBW is such that there is insufficient bandwidth available to extend by that amount on each side, then either the original emissions requirements and MPR would apply, OR asymmetrical UE channel bandwidth extension can be considered (see later proposal)

Proposal 5: RAN4 agree to consider the extension of the channel bandwidth by 1/2 of the UE CBW symmetrically can be used for “symmetrical” UE channel bandwidth extension. If the BS CBW is such that there is insufficient bandwidth available to extend by that amount on each side, then either the original emissions requirements and MPR would apply, OR “asymmetrical” UE channel bandwidth extension can be considered (see later proposal).

2.7 Extended UE channel bandwidth exceeding BS CBW edge

At RAN4#112, the following was agreed:

#### Issue 1-2-9: Whether and how to consider the Asymmetrical extended CBW approach

- WF

  - Asymmetrical extended CBW approach can be considered, FFS the details

In the last RAN4 meeting, some companies showed realistic deployment scenarios for asymmetrical extended CBW applications. We do believe that an asymmetrical extended CBW scenario can be considered to support this case. This scenario is depicted in the figure below. This was defined as “one-sided emissions relaxation” in MediaTek documents [2] and [3] submitted to RAN4#110bis and RAN4#111.

Figure 1: Asymmetrical extended CBW

Two aspects to consider for asymmetric channel bandwidths are:

- Based on existing Inner RB allocation definitions, it would not be feasible for a full RB allocation of the CBW to be equivalent to an Inner RB allocation of extended CBW.

- The centre location of the CBW would not be at the centre of the extended CBW, and this may lead to some differences in the leakage profile of the UE transmission and achievable MPR reduction for a given RB allocation.

Simulation analysis on this scenario is provided in section 3 for QPSK only at this stage, and demonstrates that, for an RB allocation of no more than 2/3 of the NRB of the CBW, with 1/3 of NRB unallocated as shown in figure 1, MPR reduction can still be applied.

Proposal 6: For the case where a UE CBW is allocated at the edge of the BS CBW, agree to consider further an “Asymmetrical extended CBW” to allow outer RB allocation MPR reduction applicability for a UE CBW allocated at the edge of the BS CBW, considering an RB allocation of no more than 2/3 of NRB for QPSK and with 1/3 of NRB unallocated as shown in the figure.

- 3. Simulation analysis

The following table provides the outcome of our simulation analysis and also evaluated the CP-OFDM and DFT-s-OFDM waveform with 15kHz SCS.

The results are provided assuming SEM/ACLR according to 20MHz CBW, with PC3 requirements assumed. We evaluated the following cases:

- 20MHz with “Symmetrical extended CBW” of 40MHz (see figure 2a)

- 20MHz with “Asymmetrical extended CBW” of 30MHz (see figure 2b)

Figure 2a: Symmetrical extended CBW evaluation scenario

Figure 2b: Asymmetrical extended CBW evaluation scenario

For Asymmetrical extended CBW, the following specific aspects are highlighted:

- The LO was allocated at the centre of the 20MHz CBW, i.e. 10MHz from left edge of 30MHz ext-CBW.

- The maximum evaluated size of LCRB and corresponding RBstart was selected to ensure that the number of non-allocated RBs (LCRB/2) inside the channel provided sufficient guardband to adhere to existing requirements at the left edge of the channel – values for DFTS and CP-OFDM are different due to restricted RB allocation sizes for DFTS-OFDM.

- We only modelled QPSK so far, but plan to evaluate 16QAM for the next meeting.

Table 2a: Evaluation results for Symmetric extended UE CBW

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 2b: Evaluation results for Asymmetrical extended UE CBW

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {25, 50}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 71}, {105, 1} | ALL PASS | ALL PASS |

Observation 1: All modelled RB allocations pass the test. These test points covered the most restrictive cases. We believe that this is sufficient to demonstrate that the requirements would be passed for all RB allocations within NRB of the actual UE CBW.

While not all aspects are modelled, we believed it can also be considered that:

- If the UE fulfils the requirements for OOBE integrated over actual UE CBW, then for OOBE integrated over Extended CBW (40MHz and 30MHz respectively) the requirements would also be fulfilled.

- For 30kHz SCS, the same outcome should also apply scaled for the corresponding NRB size adjustment within the UE CBW.

Observation 2: An extended CBW is shown to be feasible for reducing MPR for Outer RB allocation MPR for PC3, for Symmetrical Extended UE CBW (for QPSK and 16QAM) and for Asymmetrical extended UE CBW (for QPSK, with 16QAM FFS).

- 4. Conclusion

In this contribution, we have presented our perspectives on power domain enhancement for NR single carrier, and have provided the following observations and proposals:

Proposal 1: Limit symmetrical and asymmetrical UE channel bandwidth extension to preferably a single bandwidth extension of ½ bandwidth extension on one or both sides of the original UE channel bandwidth.

Proposal 2: Agree that an “extended” IBE shall apply in the region between UE CBW and extended CBW, i.e. where the NRB of extended channel bandwidth in the formula is assumed to apply as opposed to that of the original channel bandwidth. Also agree to allow operators to be given control via network signalling to the UE of whether usage of extended UE CBW is permitted within the cell.

Proposal 3: Agree that requirements for ACLR and SEM apply from edge of extended CBW.

Proposal 4: Agree to shift the FOOB boundary to the edge of the OOBE for extended CBW.

Proposal 5: RAN4 agree to consider the extension of the channel bandwidth by 1/2 of the UE CBW symmetrically can be used for “symmetrical” UE channel bandwidth extension. If the BS CBW is such that there is insufficient bandwidth available to extend by that amount on each side, then either the original emissions requirements and MPR would apply, OR “asymmetrical” UE channel bandwidth extension can be considered (see later proposal).

Proposal 6: For the case where a UE CBW is allocated at the edge of the BS CBW, agree to consider further an “Asymmetrical extended CBW” to allow outer RB allocation MPR reduction applicability for a UE CBW allocated at the edge of the BS CBW, considering an RB allocation of no more than 2/3 of NRB for QPSK and with 1/3 of NRB unallocated as shown in the figure.

Observation 1: All modelled RB allocations pass the test. These test points covered the most restrictive cases. We believe that this is sufficient to demonstrate that the requirements would be passed for all RB allocations within NRB of the actual UE CBW.

Observation 2: An extended CBW is shown to be feasible for reducing MPR for Outer RB allocation MPR for PC3, for Symmetrical Extended UE CBW (for QPSK and 16QAM) and for Asymmetrical extended UE CBW (for QPSK, with 16QAM FFS).

- 5. References

- R4-2414441, “WF on power domain enhancements”, Huawei

- R4-2405619, “Discussion on power domain enhancement for NR single carrier”, MediaTek

- R4-2407910, “Discussion on power domain enhancement for NR single carrier”, MediaTek