# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416056

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirements for NR NTN HPUE

Agenda Item:	6.8.2.2.1

Document for:	Approval

Introduction

In the RAN#103 meeting, the WI [1] for Enhanced requirements and test methodology for NR and IoT NTN was established to introduce for HPUE feature. The objectives for HPUE Tx requirements are listed below.

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In the RAN4#112 meeting, the WF [2] on UE RF requirements for NTN HPUE was approved with the following agreements.

Issue 3-1-1: General considerations - Agreement - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements. Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - For PC2 IoT NTN, at least 1Tx shall be considered - For other PCs (both IoT NTN and NR NTN), FFS Issue 3-2-1: NR-NTN HPUE feasibility for different power classes Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes - Agreement of Issue 3-2-1 to Issue 3-2-3: - For PC2, it is feasible to support both handheld and non-handheld devices. - FFS other power classes for applicable device type. Issue 3-3-1: Maximum output power - Agreement - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes.

In this paper, we’d like to share our views on parts of Tx RF requirements, feasibility and RF architecture.

Feasibility and RF architecture for different power classes

In last meeting, RAN4 reached the consensus that both handheld and non-handheld devices can support PC2. However, there is no consensus on whether the other power classes can be applicable for handheld device. We’d like to share the following aspects to discuss the feasibility for different UE power classes.

Demands: Based on the rough evaluation, there are around 10~15dB gaps for handheld UE coupling loss between LEO scenario and GEO scenario. Due to large pathloss, it’s very hard to apply wideband communication technology for GEO scenario. Thus, GEO scenario, is not friendly to wideband communication technology, i.e. NR NTN. It’s not technically reasonable to force handheld UE to support other higher power classes for NR NTN as PC2 can meet the demands of LEO scenario.

UE implementation/cost: currently, PC3 is widely supported for handheld UE in TN FDD bands. Although PC2 for TN FDD bands was introduced in Rel-17 with both 1Tx and 2Tx RF architecture, this PC2 feature in FDD bands is not widely supported or implemented in handheld UE due to the difficulties of some RF components, e.g. FDD band duplexer and so on. If we assume handheld UE to support PC1.5/PC1 by using 1 Tx chain, all of RF components have to be redesign in order to tolerate the higher output power, which are not realistic. And this RF architecture is not assumed for TN PC1.5 TDD bands. If we want to reuse the current industry ecosystem as much as possible, the only way to implement PC1.5 is to use 2Tx architecture with two PC2 PAs or 4Tx architecture with four PC3 PAs. However, since the handheld UE has the limited form factor and can’t tolerate the large power consumption, currently it’s infeasible to implement PC1.5/PC1 for FDD bands in handheld UE.

Standard perspective: In TN UE specification, only TDD bands can support PC1.5 with two Tx RF architecture assumption as it is very mature that current industry can support 26dBm with one Tx chain in TDD bands. However, in FDD bands, the situation seems different that current industry can’t widely support 26dBm with one Tx chain in FDD bands. Thus, current TN UE specification doesn’t support PC1.5 in FDD bands. Generally, NTN UE need to reuse the ecosystem of TN RF components. From standard perspective, it is premature to specify PC1.5/PC1 for FDD bands in handheld UE.

Proposal 1: For NR NTN, only non-handheld UE can support PC1 in Rel-19.

Proposal 2: For NR NTN, only non-handheld UE can support PC1.5 in Rel-19.

Tx requirements

1) SEM requirements

Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

Currently, the SEM requirements for PC3 NR NTN UE are specified below.

Table 6.5.2.2-1: General NR spectrum emission mask ΔfOOB (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 5; 10, 15, 20. ± 0-1; -13; -13; 1 % of channel BW. ± 1-5; -10; -10; 1 MHz. ± 5-6; -13. ± 6-10; -25. ± 5-BWChannel; -13. ± BWChannel-(BWChannel+5); -25.

For 5, 10, 15 and 20MHz, the absolute emission limit that UE can’t exceed in the first adjacent channel are 1.47dBm/5MHz, 0.61dBm/10MHz, 0.92dBm/15MHz, and 1.47dBm/20MHz respectively. If we assume the same SEM requirements for PC2 UE, that means UE has to meet 24.5~25.5dB ACLR requirements. Similarly, for PC1.5 and PC1, UE has to meet 27.5~28.5dB ACLR and 29.5~30.5dB ACLR requirements respectively.

Observation 1: When ACLR requirements are less than 24.5dB for PC2, 27.5dB for PC1.5 and 29.5dB for PC1, current SEM requirements specified in TS 38.101-5 can be relaxed correspondingly.

2) SAR

Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

In last meeting, RAN4 has agreed to use P-MPR as starting point, but other solutions similar to TN duty cycle are not precluded. However, as we discussed in last meeting, UL coverage is the bottleneck of satellite communication scenario and the system is very sensitive to the UE transmitter power reduction. Thus, the P-MPR solution is not very friendly to NTN UE. Apart from that, for FDD bands, there is no fixed UL/DL pattern, so there is no accurate duty cycle concept in FDD deployment. It’s necessary to study the additional method to guarantee the SAR compliance without additional power reduction.

From NTN UE perspective, as long as the UL transmission percentage does not exceed a threshold within several minutes based on SAR restriction, the SAR requirement can be satisfied without additional UE efforts. In addition, if network scheduling for UL-DL configuration in FDD bands is predicted by UE, it’s beneficial for UE to fully utilize their resources and control the power consumption.

From NTN SAN perspective, network needs to decide the starting time of UL scheduling, the period of duty cycle and UL transmission percentage considering the higher output power, SAR requirements, transmission repetition, targeted throughput and MCS.

Observation 2: For NTN HPUE scenario, it’s essential to guarantee the SAR compliance from network scheduling without NTN UE output power reduction.

Proposal 3: Send RAN1 LS to check whether the network scheduling can be enhanced to guarantee NTN UE comply with SAR requirements without NTN UE output power reduction.

Summary

Proposal 1: For NR NTN, only non-handheld UE can support PC1 in Rel-19.

Proposal 2: For NR NTN, only non-handheld UE can support PC1.5 in Rel-19.

Observation 1: When ACLR requirements are less than 24.5dB for PC2, 27.5dB for PC1.5 and 29.5dB for PC1, current SEM requirements specified in TS 38.101-5 can be relaxed correspondingly.

Observation 2: For NTN HPUE scenario, it’s essential to guarantee the SAR compliance from network scheduling without NTN UE output power reduction.

Proposal 3: Send RAN1 LS to check whether the network scheduling can be enhanced to guarantee NTN UE comply with SAR requirements without NTN UE output power reduction.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)

[2] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung