# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416126

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on (e)RedCap RRM requirements of R19 NR NTN

Source: 	OPPO

Agenda item:	6.26.4.1

Document for:	Discussion

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1], including downlink coverage enhancements, support of regenerative payload, and support of (e)RedCap UEs with NR FR1-NTN.

In this paper, we will provide our views on support of (e)RedCap UEs with NR FR1-NTN.

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Discussion on support of (e)RedCap UE with NTN

- Sub-topic 6-1 Applicability clarification - Issue 6-1-1: The operating band of (e)Redcap UE with FR1-NTN <Agreements> - For RRM requirements, the operating band of (e)Redcap UE with FR1-NTN bands include all of the following NR-NTN FR1-NTN bands: - n256, n255 and n254 defined in Table 5.2-1 in TS 38.108. - For RRM requirements, RAN4 shall only study the FDD operation and no TDD discussion in NR NTN scenario. - Issue 6-1-2: The bandwidth of (e)Redcap UE with FR1-NTN <Way Forward> - Both bandwidth with and without reduction of R18 eRedCap UE should be supported for (e)RedCap UE with FR1-NTN bands. - Note: There is no RRM impact on the two different types of eRedCap UEs (bandwidth with and without reduction). - Issue 6-1-3: The satellite types considered for (e)Redcap UE with FR1-NTN <Agreements> - For RRM requirements, the satellite types considered for (e)Redcap UE with FR1-NTN bands include both GSO and Non-GSO. - Issue 6-1-4: The capability considered for (e)Redcap UE with FR1-NTN <Agreements> - GNSS capabilities and simultaneous GNSS and NR-NTN operation is supported for (e)Redcap UE with FR1-NTN. - Issue 6-1-5: The network scenario considered for (e)Redcap UE with FR1-NTN <Way Forward> - For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier. - Issue 6-1-6: The SCS considered for (e)Redcap UE with FR1-NTN <Way Forward> - For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

Considering the limitation of (e)Redcap UE, we support to consider NR SA operation mode with single carrier for (e)Redcap UE with FR1-NTN bands.

Proposal 1: RAN4 to only consider NR SA operation mode with single carrier for (e)Redcap UE with FR1-NTN bands.

For RRM requirements, we can consider to support 15kHz and 30kHz with higher priority, while 60KHz seems not very popular.

Proposal 2: For SCS, RAN4 to consider 15KHz and 30KHz wish high priority for (e)Redcap UE with FR1-NTN.

- Sub-topic 6-2 General consideration on RRM requirements - Issue 6-2-1: The general principle for defining the RRM requirements for (e)RedCap UE with FR1-NTN <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, MTK, Xiaomi, Samsung, HW, vivo, ZTE): - A general principle is that to define the RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN. - Proposal 2 (Ericsson): - RRM requirements for RedCap UE in NTN can be started by taking RedCap UE in TN as baseline. - The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced for RedCap UE in NTN. - Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN Online agreement <Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps <Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT) - Issue 6-2-3: Features to be considered for (e)RedCap UE with FR1-NTN <Way Forward> - RAN4 to analyze the RRM requirement impacts from following features for (e)RedCap UE with FR1-NTN case by case: - 1Rx - FFS: NCD-SSB - HD-FDD - FFS: R17 relaxed measurement - FFS: RRC_IDLE/RRC_INACTIVE mode eDRX enhancement - FFS: Determine if CSSF and concurrent gap is supported in Redcap NTN - FFS: FH in PRS measurement - Single cell search and measurement engine

In last meeting, RAN4 agreed on a list of requirements defined for both NTN and (e)RedCap UE requirements, only defined for NTN, and defined for (e)Redcap and not been defined for FR1-NTN, which are the most fundamental ones for the basic RRM related procedures. RAN4 also needs to go through all the applicable requirements for RedCap and NTN.

The agreed features specific for RedCap, can also apply to normal UEs. But for other RedCap features, e.g., eDRX, NCD-SSB and R17 relaxed measurement, we think RAN4 has not defined requirements for those features in NTN for normal UEs. Thus it may be a bit strange to apply only for RedCap UE with FR1-NTN but not for other NTN UEs.

- Sub-topic 6-5 eDRX enhancement #### Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN? <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

For the FFS part, we suggest to not consider NCD-SSB, R17 relaxed measurement, and eDRX enhancement for (e)RedCap UE with FR1-NTN.

In addition, RAN4 also agreed to support 1 Rx and HD-FDD for RedCap + NTN requirements. As some company comments, 1RX is an important means for RedCap UE to reduce the cost and complexity, and HD-FDD is explicitly mentioned in the objective description. FH in PRS measurement aims at improving measurement accuracy where the measurement is not restricted by the UE RF BW (20MHz) but can reach cell BW (100MHz). However, the cell BW in FR1-NTN bands are limited (~30MHz), so the gain from FH may be limited. We prefer not to consider FH in PRS measurement.

Proposal 3: Suggest to not consider NCD-SSB, R17 relaxed measurement, FH in PRS measurement and eDRX enhancement for (e)RedCap UE with FR1-NTN.

If most companies support eDRX enhancement, we can compromise to study which eDRX cycle are applicable, e.g., only introduce the eDRX enhancement for Rel-17 RedCap UEs.

Proposal 4: Compromise to consider part of eDRX cycles introduced for R17.

For Rel-17/18 Conditional Handover, and RACH-less Handover which are specific for NTN features, we think they can also be optionally supported by (e)Redcap UE with FR1-NTN bands. It is also ok to check with RAN1/2 on such procedures.

Proposal 5: Open to support CHO and RACH-less HO for (e)Redcap UE with FR1-NTN bands.

- Sub-topic 6-3 Reduction in the number of UE Rx branches #### Issue 6-3-1: The principle for defining the requirements for 2Rx/1Rx (e)Redcap UE with FR1-NTN Online agreement <Agreement> - For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx. - RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE, if needed. - As baseline, follow the same principle for (e)RedCap UE in TN network. - For 2Rx (e)RedCap UEs with FR1-NTN: Reuse the existing requirements for NTN as a baseline. - Meanwhile, consider the following difference between 2Rx (e)RedCap and non-RedCap UE: - The number of searchers - Others if any #### Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ? <Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

RAN4 has agreed to define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE with FR1-NTN bands. For 2Rx (e)RedCap UEs with FR1-NTN, RAN4 reuses the existing requirements for NTN as a baseline. For 1RX (e)RedCap UE, we can also relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

Proposal 6: For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

The most of  NTN features should be supported in RedCap + NTN requirements. But as companies pointed out, one exception is the support of multiple SMTCs. So far, NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap there is no scaling in measurement period meaning UE has to support parallel measurement for the two SMTCs. This may be a bit challenging for (e)RedCap UE with reduced complexity. We also share the similar concern on support of multiple SMTCs.

Proposal 7: RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands.

Conclusions

In this paper we provided our views on RRM requirements for support of (e)RedCap UEs with NR FR1-NTN.

Proposal 1: RAN4 to only consider NR SA operation mode with single carrier for (e)Redcap UE with FR1-NTN bands.

Proposal 2： For SCS, RAN4 to consider 15KHz and 30KHz wish high priority for (e)Redcap UE with FR1-NTN.

Proposal 3: Suggest to not consider NCD-SSB, R17 relaxed measurement, FH in PRS measurement and eDRX enhancement for (e)RedCap UE with FR1-NTN.

Proposal 4: Compromise to consider part of eDRX cycles introduced for R17.

Proposal 5: Open to support CHO and RACH-less HO for (e)Redcap UE with FR1-NTN bands.

Proposal 6: For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

Proposal 7: RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands.

Reference

- R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT