3GPP TSG RAN WG4 Meeting #112-bis		R4-2416212

Hefei, China, Oct.14th – 18th, 2024

Source: 	Nokia

Title: 	Reply to LS R4-2414918

Agenda item:       6.12.1

Document for:     Discussion

# 1	Introduction

There is a LS R4-2414918 for Rel-18 from ETSI TC MSG/TFES has been received regarding to larger than 72mm UE OTA test [1]. In this paper, we’d like to share our views to reply to the LS:

The detailed LS are as follows: 1. Overall description: MSG TFES would like to inform 3GPP TSG RAN4 and RAN5 that MSG TFES has been developing LTE UE harmonised standard EN 301 908-13 to cover Over The Air (OTA) antenna performance requirements in terms of Total Radiated Sensitivity (TRS) and Total Radiated Power (TRP) for devices over 72mm wide. However, the corresponding phantom definition and UE positioning for LTE device above 72mm in width is missing in 3GPP LTE specifications. To ensure the compliance of the requirements in EN 301 908-13, there are two possible solutions: - TRP/TRS phantom definition and positioning guideline for UE above 72mm in width is supplemented by 3GPP RAN4 and RAN5 in LTE specifications. - TRP/TRS phantom definition and positioning guideline for NR UE wider than 72mm specified in TS 38.161 is reused in ETSI LTE UE specifications (EN 301 908-13). MSG TFES would highly appreciate 3GPP specification updates and/or feedbacks before March 2025 to facilitate timely completion of LTE UE harmonized standard. 2. Actions: MSG TFES would respectfully ask 3GPP RAN4 and RAN5 to include TRP/TRS phantom definition and positioning guideline for UE above 72mm to LTE specifications by March 2025. In case this is not feasible, MSG TFES would like to ask for confirmation whether there are technical concerns on directly applying the NR specifications on the phantom definition and positioning guidelines also for LTE.

# 2	Discussion

2.1	Phantom definition and positioning guideline

The phantom of head and hand are the reflections of the real human hand and head. The size of them and the materials of them used during OTA test were defined based on thorough studies and strict verification in both academia and industry.

The size of UE may change the holding comfortability of the UE and the experience of large screen, but this will not change the fact that how human will hold UE and how to put UE next the ear to make the phone calls. The size and materials of human head and hand will not change. Therefore, phantom definition and positioning guidelines should not be changed.

The only issue observed is the size of the holder of phantom for size 2 UEs. Since the current phantom designs are used for OTA tests of size 1 UEs, then, it is possible and make sense that the holder cannot hold size 2 UEs.

For the holder design of size 2 UEs, it may need some attention that the alignment of size 2 UE with the head phantom should be the same as that of size 1 UEs. For example, the ear speaker center of size 2 UEs should be still next to the center human/phantom’s ear and such. As well as that the shape of the phantom around the holder needs to be carefully changed if any.

Observation 1: How humans will hold UE and how to put UE next the ear to make the phone calls will not change for both size 1 UEs and size 2 UEs. The only issue observed is the size of the holder of phantom for size 2 UEs and the alignment of the holder with the phantom.

Proposal 1: There is no change for TRP/TRS phantom definition and positioning guideline for UE above 72mm in width.

Proposal 2: The holder for size 2 UEs needs to be carefully aligned with phantom so that the center of speak of size 2 UEs is still next to the center of the phantom ear and such.

2.2	Solutions suggested in LS

Based on the LS, there are two solutions suggested as below:

To ensure the compliance of the requirements in EN 301 908-13, there are two possible solutions:

- TRP/TRS phantom definition and positioning guideline for UE above 72mm in width is supplemented by 3GPP RAN4 and RAN5 in LTE specifications.

- TRP/TRS phantom definition and positioning guideline for NR UE wider than 72mm specified in TS 38.161 is reused in ETSI LTE UE specifications (EN 301 908-13).

In order to address the solutions in the LS, it is possible to consider using solution 2 first and further discuss solution 1. For example, the alignment of the holders for size 1 UEs with the head phantom and the alignment of holder of size 2 UEs with head phantom should be based on the same rule according to the observations and proposals in previous sessions.

Proposal 3: To address the solutions in the LS, it is possible to consider using the solution 2 first and further discuss the solution 1, for example, add supplementary description of the alignment of the holders for size 1 UEs with the head phantom and the alignment of holder of size 2 UEs with head phantom.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: How humans will hold UE and how to put UE next the ear to make the phone calls will not change for both size 1 UEs and size 2 UEs. The only issue observed is the size of the holder of phantom for size 2 UEs and the alignment of the holder with the phantom.

Proposal 1: There is no change for TRP/TRS phantom definition and positioning guideline for UE above 72mm in width.

Proposal 2: The holder for size 2 UEs needs to be carefully aligned with phantom so that the center of speak of size 2 UEs is still next to the center of the phantom ear and such.

Proposal 3: To address the solutions in the LS, it is possible to consider using the solution 2 first and further discuss the solution 1, for example, add supplementary description of the alignment of the holders for size 1 UEs with the head phantom and the alignment of holder of size 2 UEs with head phantom.

# References

- R4-2414918, LS to 3GPP on LTE TRP/TRS test method for UE over 72mm wide.

# Appendix A:

| # Liaison Statement |  |  |
| --- | --- | --- |
| Title: |  | LS to 3GPP on LTE TRP/TRS test method for UE over 72mm wide |
| Date: |  | 2024-09-30 |
| From (source): |  | ETSI TC MSG/TFES, Dominique Everaere |
| Contact(s): |  | (dominique.everaere@erisson.com); MSGTFESsupport@etsi.org; |
| To: |  | TSG RAN WG4,TSG RAN WG5 |
| Copy to: |  | TSG RAN(3GPPLiaison@etsi.org) |
| Response to: (if applicable) |  | N/A |
| Attachments: (if applicable) | N/A |  |

1. Overall description:

MSG TFES would like to inform 3GPP TSG RAN4 and RAN5 that MSG TFES has been developing LTE UE harmonised standard EN 301 908-13 to cover Over The Air (OTA) antenna performance requirements in terms of Total Radiated Sensitivity (TRS) and Total Radiated Power (TRP) for devices over 72mm wide. However, the corresponding phantom definition and UE positioning for LTE device above 72mm in width is missing in 3GPP LTE specifications.

To ensure the compliance of the requirements in EN 301 908-13, there are two possible solutions:

- TRP/TRS phantom definition and positioning guideline for UE above 72mm in width is supplemented by 3GPP RAN4 and RAN5 in LTE specifications.

- TRP/TRS phantom definition and positioning guideline for NR UE wider than 72mm specified in TS 38.161 is reused in ETSI LTE UE specifications (EN 301 908-13).

MSG TFES would highly appreciate 3GPP specification updates and/or feedbacks before March 2025 to facilitate timely completion of LTE UE harmonized standard.

2. Actions:

MSG TFES would respectfully ask 3GPP RAN4 and RAN5 to include TRP/TRS phantom definition and positioning guideline for UE above 72mm to LTE specifications by March 2025. In case this is not feasible, MSG TFES would like to ask for confirmation whether there are technical concerns on directly applying the NR specifications on the phantom definition and positioning guidelines also for LTE.

3. Date of next meetings of the originator:

MSG TFES#82		Sophia Antipolis, France	15-16.01.2025

MSG TFES#83		Paris, France			17-18.03.2025