# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2416227

# Maastricht, The Netherlands, 19 – 23 August 2024

# Title:	Requirements for 40MHz CBW in Band n28

# Agenda Item:	5.12.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on REFSENS requirements for the new request to support 40MHz channel bandwidth (CBW) in Band n28 for both Power Class 3 (PC3) and Power Class 2 (PC2) [1].

# Discussion

## Remaining open items from last meeting’s WF agreements [1]

The following items remain for further discussion at this meeting. Agreements were reached on:

- UL configuration for REFSENS / RSD

- PC2 RSD for

### Receiver requirements

Requirements for PC3 REFSENS and PC2 RSD remain open for Band n28 40MHz CBW:

- PC3: A REFSENS level of -66.4dBm for SCS 15kHz was proposed as the average of -65.9dBm [2], -67.1dBm [3], -66.3dBm [4].

- PC2 RSD for further study (FFS). Two companies provided RSD analyses [3,4] at RAN4 #112.

### Transmitter requirements

- Further check if ΔMPR= 0.5dB is sufficient.

- Further study NS_18 for 40MHz CBW PC3, PC2.

- Further check whether all currently specified UE-to-UE coex requirements are applicable for 40 MHz CBW.

## Transmitter requirements

### MPR

Maximum Power Reduction (MPR) for NR within FR1 is agnostic of the uplink channel bandwidth (CBW) except when the relative uplink CBW ratio exceeds 3% for FDD bands or 4% for TDD bands, in which case a relaxation to MPR is allowed and is specified with parameter “MPR”.  Table 1 summarizes the previously agreed MPR for Band n28 and Band n71 vs the calculated UL CBW ratio. The MPR for Band n28 remains to be specified for:

- 30MHz CBW for PC2,

- 40MHz CBW for PC3 and for PC2.

Table 1: Agreed MPR for UL relative CBW ratio exceeding 3% for FDD and 4% for TDD band vs Band n28 request to support 40MHz CBW.

| NR Band | Duplex | Power Class | UL CBW (MHz) | UL CBW Ratio | ΔMPR (dB) |
| --- | --- | --- | --- | --- | --- |
| n28 and n83 | FDD | PC3 | 30 | 4.1% | 0.5 PC2: [FFS] |
| n40 and n97 | TDD | PC3 and PC2 | 100 | 4.3% | 1.0 |
| n71 | FDD | PC3 and PC2 | 25 30 35 | 3.7% 4.4% 5.1% | 0.5 |
| n28 | FDD | PC3 and PC2 | 40 | 5.6% | FFS |

Figure 1 shows the measured ACLR vs output of a Band n28 PA with linearity calibrated for PC3 operation assuming 4dB post PA losses, to meet -30dBc ACLR at MPR0 using the DFT-s-OFDM SCS15kHz fully allocated DFT-s-OFDM waveform. The measured PA calibration point is plotted with a red circle and is within 0.2dB of the “ideal” calibration point.  Each line represents the measured ACLR vs. output power for 20, 25, 30, 35 and 40MHz CBW measured for fully allocated QPSK DFT-s-OFDM waveforms. A little more than 0.4dB MPR is needed at 40MHz CBW. For PC2, we measure a MPR of ~0.7dB. The Band n28 UL CBW ratio at 40MHz CBW is close to that of Band n71 at 35MHz CBW for which 0.5dB MPR has been agreed for PC3 and PC2. Our measurements indicate that 0.5dB MPR is acceptable for Band n28 40MHz CBW for PC3 and for PC2.

Figure 1: Measured PC3 Band n28 ACLR for 20,25,30,35 and 40MHz CBW fully allocated DFT-s-OFDM QPSK.

Proposal 1: For Band n28, adopt MPR = 0.5dB for PC2 30MHz and 40MHz CBW and for PC3 40MHz CBW.

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 Power class 2 | 30 MHz 40 MHz | 0.5 |

### Spurious emissions for UE co-existence

For spurious emission coexistence requirements, we verify the measured worst-case PA emission level that may affect the range 758-773MHz for which the requirement is -32dBm/MHz. We measure at ~ -12dBm the power level (antenna referred) of a 1UL RB allocation for PC2 MPR0. The minimum Tx filter rejection required to pass the UE co-existence requirement is therefore 20dB. This attenuation should be achievable for a PC2 capable full-band duplexer.

Observation 1: A minimum of 20dB Tx filter rejection is needed to meet the UE coexistence requirement in range 758-773MHz.

### NS_18 40MHz CBW evaluation

The NS_18 measurements are performed with a Band n28 PA that is calibrated according to test conditions captured in clause 2.3.1. The results presented in this document are captured:

- with only a single voltage supply level (aka “Vcc”), and that is the voltage obtained for PA linearity calibration of each power class.

- Two cross-coupled PAs emulating 10dB antenna-to-antenna isolation, but due to the lack of time only 1Tx PC3 and 1Tx PC2 results are published

- The results are expressed as raw PA output back-off “OBO” relative to the PC3 and PC2 respective MPR0 levels and 4dB post-PA losses,

- A limited set of QPSK DFT-s-OFDM and QPSK CP-OFDM waveforms are measured at SCS 15kHz.

The measurements results are captured in Table 2 for:

- 1Tx PC3 for 40MHz CBW only,

- 1Tx PC2 for 20, 25, 30, and 40MHz CBW,

The following naming convention is used:

- 2D/2C denotes QPSK DFT-s-OFDM/ QPSK CP-OFDM respectively

- M0 denotes SCS 15kHz

- “100R000” denotes the UL RB allocation Lcrb = 100 (RBstart=0).

In Table 2, we highlight in yellow waveforms which are problematic from a measurement perspective due to single VCC operation.

Table 2: NS_18 1Tx PC3 and 1Tx PC2 output-back-off measurement results.

| WFM Type | DUT2 UL Configuration | PC3 NS_18 OBO | PC2 NS_18 OBO | WF [5] A-MPR |
| --- | --- | --- | --- | --- |
| 2D | 20M0_100R000 |  | [5..9] | - |
| 2C | 20M0_106R000 |  | [>10] | - |
| 2D | 25M0_001R028 |  | 2.2 | - |
| 2C | 25M0_001R028 |  | 3.7 | 6 |
| 2D | 25M0_027R016 |  | 2.2 | 4.5 |
| 2C | 25M0_029R016 |  | 3.8 | 6 |
| 2D | 25M0_054R060 |  | 0 | - |
| 2C | 25M0_059R062 |  | 1.7 | - |
| 2D | 25M0_064R053 |  | 0.9 | 2 |
| 2C | 25M0_067R054 |  | 2.2 | 5 |
| 2D | 25M0_075R058 |  | 1.2 | 3.5 |
| 2C | 25M0_075R058 |  | 2.7 | 5 |
| 2D | 25M0_128R000 |  | [5..9] | 10 |
| 2C | 25M0_133R000 |  | [>10] | 11.5 |
| 2D | 30M0_001R037 |  | 2.7 | 4.5 |
| 2C | 30M0_001R037 |  | 4 | 6 |
| 2C | 30M0_029R022 |  | 3.9 | 6 |
| 2D | 30M0_081R070 |  | 0.6 | 3.5 |
| 2C | 30M0_081R070 |  | 2.1 | 5 |
| 2D | 30M0_081R079 |  | 0.7 | 3.5 |
| 2C | 30M0_087R073 |  | 2.4 | 5 |
| 2D | 30M0_160R000 |  | [4..8] | 10 |
| 2C | 30M0_160R000 |  | [>10] | 11.5 |
| 2D | 40M0_001R056 | 2 | 1.9 | [FFS] |
| 2C | 40M0_001R056 | 2.6 | 3.8 |  |
| 2D | 40M0_027R042 | 1.4 | 2.2 |  |
| 2C | 40M0_029R041 | 2.7 | 3.7 |  |
| 2D | 40M0_216R000 | 3.9 | [5..10] |  |
| 2C | 40M0_216R000 | 5.5 | [>10] |  |

The examples of fully allocated DFT-s-OFDM waveforms for 20MHz, 30MHz and 40MHz CBW are shown in Figure 2 (left/centre/right).

- For 20MHz CBW, we suspect that re-measuring the waveform with a 2nd, lower voltage calibration Vcc will clear any ambiguity on the required OBO,

- For 30MHz CBW, the measured OBO exhibits a plateau around the NS_18 emission requirement level (scaled to the PA output pin) across the range of OBO ~ 4 to ~9dB.

- For 40MHz CBW, the measurements indicate that more than 10dB OBO is needed, but the exact value is difficult to extract because at such high OBO, the single VCC operation may no longer be representative of a commercial UE performance.

In the past we have often recommended that NS values which require A-MPR greater than 6 to 7dB be re-evaluated with a lower, 2nd calibration point, aka “Vcc2”. The yellow highlighted waveforms are such cases

Figure 2: Measured 1Tx PC2 Band n28 OBO for 20MHz (left), 30MHz (centre) and 40MHz CBW (right) fully allocated DFT-s-OFDM QPSK.

Observation 2: Several Band n28 UL configurations which require more than ~6dB OBO need to be re-evaluated with a 2nd VCC due to PA emission plateau and PA performance for which the MPR1 calibration VCC may no longer reflect the commercial UE performance, especially when OBO >10dB.

Proposal 2: Come back at next meeting to improve the accuracy of required OBO for waveforms where large (>10dB) A-MPR is needed, like WF [5] Region A4 configurations.

## PC3 and PC2 REFSENS

High self-desense “MSD” is expected for Band n28 40MHz CBW operation due to the overlap of the first adjacent channel of the uplink (UL) carrier with its own downlink (DL) carrier. We present power amplifier (PA) measurements to measure the level of interference that may affect the DL 40MHz Band n28 carrier for both PC3 and PC2.

### Power Amplifier Calibration

- The PA linearity is calibrated using a 20 MHz, SCS=15 kHz, QPSK, DFT-S-OFDM, LCRB=100 (RBstart=0) waveform to meet:

  - -30dBc ACLR at MPR1 26dBm (4dB post PA insertion loss) for PC2 operation;

  - -31dBc ACLR at MPR1 29dBm (4dB post PA insertion loss) for PC2 operation.

- Waveform impairments:

  - Local Oscillator (LO) leakage: -28 dBc;

  - IQ Image rejection: -28 dB;

  - C-IM3 -60dBc, C-IM5 -70dBc.

The measurements of the PA emissions are performed over the following range of UL RB allocations “LCRB” with UL RBs positioned closest to the DL band: 1,2,15,20,25,40,64,100,128,160,216.

### PC3, PC2 Measurement Results

Figure 1 below shows the measured noise in Rx band for PC3 (orange dots) vs. PC2 single Tx (red dots) vs. UL RB allocation LCRB. The Tx noise level is a minimum for close to 100 RBs for both power classes.

Figure 1: Measured PC3 and PC2 PA noise in Band n28 Rx band for 40MHz CBW.

The Band n28 REFSENS is verified using the UL RB configurations reproduced in Table 1 below. For 30MHz CBW LCRB = 25RB. For 40MHz CBW operation, we can consider adopting the same configuration as for 30MHz or a smaller RB allocation. The data collected in Figure 1 shows that Tx noise increases for LCRB < 25, we therefore propose to evaluate the MSD using LCRB = 25RB. Note however that the measured Tx noise reaches a minimum for LCRB~ 100RB.

For this UL configuration we measure the following Tx noise levels:

- PC3: -11.9dBm/MBW;

- PC2: -7.9 dBm/MBW.

Table 2 Existing UL configuration specifications for band n28 (Table 7.3.2-3) for CBW up to 30MHz.

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 3: The measured Tx noise affecting the Rx band is minimum at LCRB~ 100RB and increases for LCRB<100RB or LCRB>100RB.  MSD may be evaluated for LCRB = 25RB.

## PC3 REFSENS, PC2 RSD

For evaluation of Reference Sensitivity (REFSENS) and Reference Sensitivity Degradation (RSD), we assume:

  - An “ideal” RB scaled 40MHz REFSENS is -89.2dBm based on the 3MHz CBW REFSENS of -100.2 dBm;

  - 50dB and 45dB full-band duplexer rejection from Tx to Rx and Tx to antenna respectively;

  - 10dB antenna to antenna isolation; and,

  - 4dB front-end loss.

Based on the measured PA noise interference levels of -11.9dBm/MBW for PC3 and -7.9dBm/MBW for PC2, the Band n28 40MHz CBW REFSENS / MSD is evaluated at:

- For PC3: 40MHz REFSENS = -66.3dBm, due to 22.9dB self-desense,

- For PC2 1Tx: 40MHz RSD = 4dB, due to 26.9dB self-desense,

- For PC2 2Tx: 40MHz RSD = 8.5dB, due to 31.4dB self-desense.

Observation 4: The benefits of deploying 40MHz CBW in Band n28 is highly questionable considering the high self-desense that ranges from 22.9dB to 31.4dB for PC3 / PC2 operation. To our knowledge this is the highest single-band self-desense that RAN4 has ever studied for NR.

Proposal 3: For Band n28 40MHz CBW operation, consider adopting the following REFSENS requirements

PC3 REFSENS:

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  | -66.3 |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  | -66.4 |  |  |

PC2 1Tx RSD: 4dB,

PC2 2Tx RSD: 8.5dB.

RSD for CBW < 40MHz needs to be further studied.

# Conclusion

This contribution captures our proposals for the Band n28 40MHz and PC2 RF requirements based on PA measurement results.

Proposal 1: For Band n28, adopt MPR = 0.5dB for PC2 30MHz and 40MHz CBW and for PC3 40MHz CBW.

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 Power class 2 | 30 MHz 40 MHz | 0.5 |

Observation 1: A minimum of 20dB Tx filter rejection is needed to meet the UE coexistence requirement in range 758-773MHz.

Observation 2: Several Band n28 UL configurations which require more than ~6dB OBO need to be re-evaluated with a 2nd VCC due to PA emission plateau and PA performance for which the MPR1 calibration VCC may no longer reflect the commercial UE performance, especially when OBO >10dB.

Proposal 2: Come back at next meeting to improve the accuracy of required OBO for waveforms where large (>10dB) A-MPR is needed, like WF [5] Region A4 configurations.

The Band n28 at 40MHz CBW may be subject to severe self-desense due to ACLR 1 overlap.

Observation 3: The measured Tx noise affecting the Rx band is minimum at LCRB~ 100RB and increases for LCRB<100RB or LCRB>100RB.  MSD may be evaluated for LCRB = 25RB.

Observation 4: The benefits of deploying 40MHz CBW in Band n28 is highly questionable considering the high self-desense that ranges from 22.9dB to 31.4dB for PC3 / PC2 operation. To our knowledge this is the highest single-band self-desense that RAN4 has ever studied for NR.

Proposal 3: For Band n28 40MHz CBW operation, consider the following REFSENS requirement proposal for SCS 15kHz:

- PC3 REFSENS = -66.3dBm

- PC2 single Tx RSD = 4dB

- PC2 dual Tx RSD = 8.5dB

# Reference

[1] 	R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, CMCC, CBN, China Broadnet.

[2] 	R4-2413149 UE RF requirements for PC2 and 40 MHz, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Qualcomm Inc.

[3]	R4-2411476 REFSENS and RSD for n28 UL 40MHz, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Murata Corporation.

[4] 	R4-2413062 n28 40MHz PC2 PC3 REFSENS, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Solutions, Inc.

[5]	R4-2310245 WF on PC2 A-MPR for band n8 and n28, 3GPP TSG-RAN WG4 #107, Incheon, Korea, Huawei, HiSilicon, Apple, China Unicom.