3GPP TSG-RAN WG4 Meeting #112bis	R4-2416377

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Further discussion of NTN OTA performance metrics

Source:	OPPO

Agenda item:	6.12.2.2

Document for:	Approval

- Introduction

The performance metrics for NTN OTA were discussed during the past several RAN4 meetings. In the last RAN4 #112 meeting in Maastricht, there also were some proposals on NTN OTA performance metrics. And it did not reach any consensus.

This contribution further discusses the OTA performance metrics for NTN devices with smartphone formfactor based on the candidate metric options and the height of the satellite.

- Discussion

The latest agreement on the performance metrics of NTN OTA has been reached in the RAN4 #111 meeting in Fukuoka, where 5 categorized directions are summarized in the way forward, with Direction 1 excluded as below[1].

In the last RAN4 #112 meeting, the contribution [2] analyzed several types of performance metrics. And the proposal is that no longer focus on partial radiated powers and/or weighted TRPs, which greatly down-selected the potential directions, from the perspective of metrics not to be too dependent on device orientations. So it is helpful to check whether the UE orientation is arbitrary or directional during the process of NTN communication.

Proposal 1: Check whether the UE orientation is arbitrary or directional during the process of NTN communication.

Another issue keeping open discussion is that how to accommodate the NTN OTA performance metrics suitable for both GEO and NGSO satellites. The latest agreement has been reached in RAN4 #111 meeting in [1] as below.

From the analysis and findings in the previous contributions, the required antenna patterns for GEO is directional in a small spherical range, and that for NGSO is partial sphere or even the whole sphere (considering the situation that UE may be upside-down when a call is initiated). Generally, different antenna pattern requirement should be matched with corresponding performance metrics. On the other hand, it is beneficial to check whether the peek radiated direction of the NTN antenna without high-gain directivity can satisfy the GEO communication from the perspective of the link-level budget analysis.

Proposal 2: Check how many additional directivity antenna gain is required for GEO communication compared to GNSO communication from the perspective of the link-level budget analysis.

- Conclusions

This contribution further discusses the OTA performance metrics for NTN devices with smartphone formfactor based on the candidate metric options and the height of the satellite. And provide the following proposal.

Proposal 1: Check whether the UE orientation is arbitrary or directional during the process of NTN communication.

Proposal 2: Check how many additional directivity antenna gain is required for GEO communication compared to GNSO communication from the perspective of the link-level budget analysis.

- References

[1] R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo

[2] R4-2419771, NTN OTA Test Metrics, Keysight Technologies